Host Mode Mnemonic Indicators for AEA PK232/PK87

For example, whereas in verbose mode, the human operator types:

MFILTER 7, 19

In host mode, you would send the same command as:

MI7,19

Note the lack of a space between the I and 7.  No space is allowed!

Each command in the Host mode can be sent by issuing a unique mnemonic

or abbreviation in the form of a two-letter character group.  These

mnemonics are shown in the following list before each command.

8BITCONV       8B                  MAXFRAME       MX

AAB            AU                  MBX            MB

ABAUD          AB                  MCON           MC

ACHG           AG                  MDIGI          MD

ACRDISP        AA                  MEMORY         MM

ACRPACK        AK                  MFILTER        MI

ACRRTTY        AT                  MFROM          MF

ADDRESS        AE                  MHEARD         MH

ADELAY         AD                  MONITOR        MN

ALFDISP        AI                  MORSE          MO

ALFPACK        AP                  MPROTO         MQ

ALFRTTY        AR                  MRPT           MR

ALIST          AL                  MSPEED         MP

AMTOR          AM                  MSTAMP         MS

ARQ            AC                  MTO            MT

ARQTMO         AO                  MYALIAS        MA

ASCII          AS                  MYALTCAL       MK

ASPECT         AY                  MYCALL         ML

AWLEN          AW                  MYSELCAL       MG

AX25L2V2       AV                  NEWMODE        NE

AXDELAY        AX                  NOMODE         NO

AXHANG         AH                  NUCR           NR

BAUDOT         BA                  NULF           NF

BEACON         BE                  NULLS          NU

BITINV         BI                  OK             OK

BKONDEL        BK                  OPMODE         OP

BTEXT          BT                  PACKET         PA

CANLINE        CL                  PACLEN         PL

CANPAC         CP                  PACTIME        PT

CASEDISP       CX                  PARITY         PR

CBELL          CU                  PASS           PS

CCITT          CC                  PASSALL        PX

CFROM          CF                  PERSIST        PE

CHCALL         CB                  PPERSIST       PP

CHDOUBLE       CD                  PRCON          PC

CHECK          CK                  PRFAX          PF

CHSWITCH       CH                  PROUT          PO

CMDTIME        CQ                  PRTYPE         PY

CMSG           CM                  RAWHDLC        RW

CODE           C1                  RBAUD          RB

COMMAND        CN                  RCVE           RC

CONMODE        CE                  RECEIVE        RE

CONNECT        CO                  REDISPLA       RD

CONPERM        CY                  RELINK         RL

CONSTAMP       CG                  RESET          RS

CPACTIME       CI                  RESPTIME       RP

CRADD          CR                  RESTART        RT

CTEXT          CT                  RETRY          RY

CWID           CW                  RFEC           RF

DAYSTAMP       DS                  RXREV          RX

DAYTIME        DA                  SELFEC         SE

DCDCONN        DC                  SENDPAC        SP

DELETE         DL                  SIGNAL         SI

DFROM          DF                  SLOTTIME       SL

DISCONNE       DI                  SQUELCH        SQ

DWAIT          DW                  SRXALL         SR

EAS            EA                  START          ST

ECHO           EC                  STOP           SO

ESCAPE         ES                  TBAUD          TB

FAX            FA                  TCLEAR         TC

FAXNEG         FN                  TIME           TM

FEC            FE                  TRACE          TR

FLOW           FL                  TRFLOW         TW

FRACK          FR                  TRIES          TI

FSPEED         FS                  TXDELAY        TD

FULLDUP        FU                  TXFLOW         TF

GRAPHICS       GR                  TXREV          TX

HBAUD          HB                  UNPROTO        UN

HEADERLN       HD                  USERS          UR

HID            HI                  USOS           US

HOST           HO                  VHF            VH

HPOLL          HP                  WIDESHFT       WI

ID             ID                  WORDOUT        WO

ILFPACK        IL                  WRU            WR

IO             IO                  XFLOW          XW

JUSTIFY        JU                  XMIT           XM

KISS           KI                  XMITOK         XO

LEFTRITE       LR                  XOFF           XF

LOCK           LO                  XON            XN