W E L C O M E  T O  T H E  C B - C O M  S O F T W A R E  !

Tis documatation file says you the most important rules to use the

CB-COM user interface.

start the software by typing

cb-com

or

c

followed by the key enter.

You are now in the main menu:

It gives you the following selection:

1.      start TNC

2.      cofigurate parameters + start TNC

3.      edit configuration file: scc.ini

4.      remove L2 from momory

5.      show documentation

6.      end (return to DOS)

You can move th menu bar with the Cursor Up and Cursor Down keys and start

the selected program by typing Enter.

1. Starts the TNC program scc. Now you can use your CB-COM.

2. Starts a configuration window before doing scc. You can use the window

to manipulate up to 9 selected configuration parameters.

Default ar the parameters: mycall, txdelay, german, vadapter, rs232.

You can adapt this list to your wishes. (see later in this text)

If you truncate the configuration window with the key F1, scc.ini will

be manipulated and compiled automatically.

3. You can manipulate the scc.ini file with the text editor cbed. At first

time of using CB-COM and for all bigger manipulations you should do so.

4. Removes the memory resistend program L2.

5. Shows you this file.

6. Terminates the CB-COM user interface and returns to DOS.

The automatic video adapter detection denies with some Laptops and LCD

displays. They make it necessary to force the programs to work in

monochrom display mode.

To do so type:

cb-com /m

or

c /m

============================================================================

Documentation to the program Input

The program Input gives you the posibility to manipulate a selection

of up to 9 configuration parameters in a configuration file like scc.ini.

start input by typing

input [/m]

The optional parameter /m forces the program to work in monochrom display

mode. Normally the program gets the display information from the file

scc.ini.

The definitions of the input fields in the configuration window are in the

control file control.dat. You can manipulate this file with cbed.

Structure of the control file control.dat:

scc.ini

mycall;24;up to 4 mycalls

txdelay;3;delay after activating the radio transmitter. (10ms steps)

german;3;german character set with DIN mutated vowels

vadapter;1;video-typ; 0 = automatic, 1 = mono, 2 = color, 3 = LCD

rs232;1;number of the serial port

In the first line you find the name of the configuration file to mani-

pulate. The following lines are the instructions for creating the input

fields in the window. (up to 9)

Each instruction line must have the following structure:

keyword;length;comment

keyword is the identifier of the parameter in the configuration file.

(It must be free from blanks and semicolons.)

length is the length of the input field (= max. length of the string) at

the screen. (max. value of length = 59 - length of keyword)

comment is a line with up to 58 characters explanation

The program does not make a full syntax check of steuer.dat. You have to

check it yourself.

The upper instuction file steuer.dat generates the following input window

on the screen: (With default values from scc.ini)

+------------------------------------------------------------+

| check this parameters                                      |

+------------------------------------------------------------+

| mycall test1 test2                                         |

|   up to 4 mycalls                                          |

|                                                            |

| txdelay 40                                                 |

|   delay after activating the radio transmitter (10ms steps)|

|                                                            |

| german on                                                  |

|   german character set with DIN mutated vowels             |

|                                                            |

| vadapter 0                                                 |

|   video-typ: 0 = automatic, 1 = mono, 2 = color, 3 = LCD   |

|                                                            |

| rs232 1                                                    |

|   number of the serial port                                |

+------------------------------------------------------------+

| Esc: cancel         F1: end + save                         |

+------------------------------------------------------------+

You can manipulate the parameters by using the keyboard.

To move the cursor from one field into an other use Cursor up,

Cursor down, Tab or Enter.

To finish the operation press Esc to terminate the program

without saving. To save the manipulated parameters in the

configuration file, use the F1 key. A backupfile with the

extension .old will be generated from the old version of the

configuration file.

The parameters, which you can manipulate with this program must

have definition lines in the configuration file with the following

structure:

keyword default    ;comment

for example:

rs232 1    ;number of the serial port

Not possible are keywords which include blanks. For example:

sattrib 8 07   ;monitor info field

Prohibited are also keywords with semicolons. The first character of the

keyword must be in the first columne.

===========================================================================

CBED

This program is a simple text editor.

It enables you to edit text files. The columne length is limited to 80

characters. The number of lines is limited to 5000, if your free memory is

large enough.

cbed [filename] [/m]

filename is the file, you want to manipulate. If this parameter is missing,

the program will request you to type in the file name.

/m forces the program to operate in the monochrom display mode. Normally the

program checks which mode is activ. For some Laptops, it can be

necessary to type /n.

Supported keys:

Backspace:  Deletes the character at the left of the cursor

Del:        Deletes the character under the cursor

Ins:        Inserts a new line at cursor's position

Home:       Places the cursor at the first character in the actual line

End:        Plases the cursor at the last character in the actual line

Esc:        Aborts the program (without saving)

F1:         Terminates the program an saves the file

F2:         Saves the file

F3:         Prints the file to PRN

F4:         Deletes the file (in the buffer and on disk)