Multiuser BayCom Packet Terminal Driver Program For IBM-PC/Clones

=================================================================

Copyright Jon Welch - Ver 1.02 21/04/95

---------------------------------------

Introduction

============

The  multiuser/multitasking  BayCom Packet  Terminal  Driver

program   is  a  general  purpose  communications   package   for

communicating  with a BayCom modem attached to your  IBM-PC/clone

in  conjunction  with  the  TFPCX  Baycom  driver  software.  The

software  will  also work with a normal TNC if it  supports  KISS

mode, such as the Kantronics range.

The software includes many features not often found in other

TNC  driver  programs.  These include  integrated  Personal  Mail

System with FBB compatible compressed forwarding, split screen or

full  screen  display, 200 line scroll back  buffer  with  search

facilities,  YAPP  and ASCII file  transfer  facilities,  connect

directory, directory browser with comprehensive file viewer, file

and  notepad editor, clipboard editor with associated  functions,

UUencode  and UUdecode of files, 7PLUS encoding and  decoding  of

files, REQDIR and REQFIL facilities with automatic uuencoding  or

7PLUS  encoding  of  binary REQFIL requests, simple  to  use  yet

sophisticated  script  language for  automated  operation,  timed

execution  of  script  files, a  screen  blanker,  wordwrap  with

optional  auto-justification of type text, FBB  header  broadcast

support plus many more.

The  software  is fully multiuser/multitasking  which  means

that more than one person can be active in any part of the system

at  once  whilst you are still using the software  for  your  own

purposes. The software does not require any external multitasking

software in order to run since it is all integral to the program.

DISCLAIMER OF WARRANTY

THIS  SOFTWARE  AND  MANUAL  ARE PROVIDED  "AS  IS"  AND  WITHOUT

WARRANTIES  AS  TO PERFORMANCE OF MERCHANTABILITY  OR  ANY  OTHER

WARRANTIES WHETHER EXPRESSED OR IMPLIED.  BECAUSE OF THE  VARIOUS

HARDWARE AND SOFTWARE ENVIRONMENTS INTO WHICH THIS PROGRAM MAY BE

PUT, NO WARRANTY OF FITNESS FOR A PARTICULAR PURPOSE IS OFFERED.

MUTNCBAY Driver V 1.02          1                        21/04/95

Distribution Restrictions

=========================

The Multiuser BayCom Packet Terminal Driver program is NOT a

public domain or free package. The software is distributed as  an

'Evaluation Copy' package where all the features of the  software

are  accessible except the on-air PMS. The user can evaluate  the

product  to determine it's suitability for their purpose and  can

subsequently  register to gain access to the on-air PMS  if  they

decide to use the package.

Registration

============

Individual  copies  of  the program  may  be  registered  by

filling  in  the accompanying order form and  sending  a  minimum        ________________________________________

donation of 10 U.K. pounds to me at :

50 Quarrydale Road,

Sutton In Ashfield,

Notts.

NG17 4DR.

Great Britain

If  you are registering outside the U.K., please  send  your

registration fee in U.K. currency if possible (either as cash  or

cheques/money  order).  If you cannot arrange this,  please  send

your fee in cash currency.

I  WILL  NOT ACCEPT CHEQUES MADE OUT IN NON  U.K.  CURRENCY.

These   will   be  returned  to   sender   without   registration

information. It costs me 3.00 U.K. pounds to bank foreign cheques

and  1.50  U.K. pounds to exchange foreign  currency.  Since  the

registration fee is so low, these bank charges take a hefty  lump

out of the fee which I could well do without. International reply

coupons  can only be exchanged for postage stamps and not  cashed

so these are also out I am afraid.

Upon  receipt of your registration information, you will  be

sent your registration number enabling you to gain full access to

the program. Registration entitles you to future upgrades of this

software FREE OF CHARGE so please register and give your  support

to the program.

If  you  are  ordering from outside  Great  Britain,  please

remember to include your country in your address so I know  where

to send the information to !

MUTNCBAY Driver V 1.02          2                        21/04/95

C O N T E N T S

===============

Chapter                                           Page

-------                                           ----

1 What is Multiuser/Multitasking Software ?      4

2 System Requirements                            5

3 Installing The Software                        6

3.1 System Files and Usage                     7

4 Configuring The Software                       9

4.1 Format of TNC.CFG                          9

4.2 Format of TNC.TIM                         13

5 How to use the Software                       14

5.1 General Guidelines                        15

6 Miscellaneous Terminal Driver Commands        17

7 Specialised Terminal Driver Function Keys     19

8 Specialised Terminal Driver Alt Function Keys 20

9 Specialised Terminal Driver Alt Keys          21

10 Directory Browser                             25

11 7PLUS File Encoding                           28

12 Script Files                                  30

12.1 Sample Script Files                      40

12.2 Automatic Script Files                   43

13 Personal Mail System                          44

13.1 Configuring the PMS                      44

13.2 Setting up the PMS                       47

13.3 Logging Onto The PMS                     48

13.4 Using the PMS                            48

13.5 Commands In The PMS                      50

13.6 Maintaining The PMS                      61

13.7 BBS access to PMS                        62

13.8 REQDIR and REQFIL                        64

14 Online Cross Reference Help Facility          66

14.1 Creating a Help File                     66

14.2 Using the Help System                    67

15 FBB Header Broadcast Facility                 68

16 Trouble Shooting                              70

Appendices

----------

A - Summary of Function Keys                    72

B - Summary of Editor Keys                      73

C - Summary of Script Language                  74

D - Summary of PMS Commands                     77

E - TFPCX Configuration Files                   80

F - Closing Remarks                             82

MUTNCBAY Driver V 1.02          3                        21/04/95

1 What is Multiuser/Multitasking Software ?

===========================================

The  principle behind multiuser/multitasking software is  to

enable   the  program  to  run  more  than  one  task   at   once

(multitasking)  and  to  allow more than one person to  be  in  a

particular task at the same time (multiuser).

In this software, a task is taken to be a channel which  can

be configured to be either a PMS Port or a terminal driver port.

You can define up to 10 of any combination of the above  two

tasks  and  the program will then appear to be running  them  all

simultaneously.

For  example,  you could define 7 PMS port  and  3  terminal

driver ports. With this setup, 7 users can be in your PMS at  the

same  time whilst you can be making three outgoing connects.  All

this would be happening simultaneously.

Of course, doing 10 things at once rather than just the  one

task will slow down the computer quite a bit and so depending  on

the speed and processor type of your computer, you will  probably

want  to  restrict the number of tasks to say, two  of  each  for

example.

Another  restriction  on the number of tasks  you  can  also

define is the amount of base memory your computer has. Each  task

you  configure  wants  a certain amount of memory  for  it's  own

personal  workspace.  Refer  to the table below  for  the  memory

requirements of the program and the two tasks.

Task                     Memory Requirements

====                     ===================

BayCom TFPCX Driver              65K

Main Program                    359K

Terminal Driver                  40K

PMS                              12K

When  you are configuring the software, you must  make  sure

that  your computer has enough free memory for the  total  memory

requirements  of the tasks you define. Also, if you want  to  use

the  DOS shell feature of the program as well, bear in mind  that

this  can  require  upto 100K depending on your  DOS  version  so

please  leave  enough  free memory when choosing  the  tasks  you

require.

MUTNCBAY Driver V 1.02          4                        21/04/95

2 System Requirements

=====================

The  software  is designed to run in  conjunction  with  the

TFPCX  Baycom driver software written and placed into the  public

domain  by  Rene Stange DG0FT. The current version of  the  TFPCX

driver also supports KISS mode TNC's enabling this program to run

on a much wider range of TNC's than earlier versions.

The software will work on any 80x86 computer running  MS-DOS

3.00 or above. The more free base memory you have available after

loading  DOS and device drivers etc will mean you  can  configure

more  tasks  for  use by the program. The video  display  can  be

colour   or   mono  and  the  on-screen   colours   are   totally

configurable.  The software has been tested with version 2.10  of

the TFPCX software but any recent version should be compatible.

If  you encounter any problems running the program, it  will

probably be due to your TFPCX configuration file being  different

to  mine. Please compare my TFPCX.INI (given in Appendix E)  with

yours  and  adjust yours to suit. Doing this will sort  out  most

problems  you  are likely to encounter. Also,  read  the  Trouble

Shooting section below for more information on sorting out common

problems reported by users of the program.

MUTNCBAY Driver V 1.02          5                        21/04/95

3 Installing The Software

=========================

To install the software you will need to copy the  following

files TNCBAY.EXE, TNC.CFG and TNC.CLR to either your hard disc or

to a blank floppy disc. You will also need to copy the TNCPMS.CFG

file  to  the PMS directory and the help files to  their  correct

location as shown below.

If  you wish to use the DOS-SHELL feature of  the  software,

you will also need a copy of 'COMMAND.COM' on your floppy disc.

You  will also need to create several sub-directories  under

the  TNC home directory. These will be used to store  any  script

files  you  may create, the PMS system and an area for  your  log

files and other system information. The structure you must create

is therefore :

TNC

|

|

+---------------+-----------------+

|               |                 |

|               |                 |

LOG             PMS             SCRIPT

|

|

+---------------+--------+--------+---------------+

|               |                 |               |

|               |                 |               |

HELP           FORWARD          AREAINFO          MAIL

|

|

OLDMAIL

If  you are installing the software from the  release  file,

this structure will be created for you automatically.

MUTNCBAY Driver V 1.02          6                        21/04/95

3.1 System Files and Usage

--------------------------

The files comprising the system, where they are located  and

their function are as follows :

TNC

---

TNC.CFG     - Configuration File

TNC.CLR     - Colour configuration file

TNC.CT      - Optional CTEXT sent upon a connection

TNC.DIR     - Connect directory

TNC.HLP     - Optional online cross reference help file

TNC.LOG     - Log file of connect and disconnect times

TNC.TIM     - Optional timed event configuration file

TNCBAY.EXE  - Main Program

TNC\LOG

-------

MAMAUTO.001 - Files created by the LOG, ENDLOG script commands

BADAUTO.001 - These can be deleted once read

GB7MAM .FBB - List of grabbed FBB message headers

GB7MAM .IDX - Index for above file

TNC\PMS

-------

TNCPMS.BAD - Callsigns not allowed on PMS  (optional)

TNCPMS.CFG - Configuration file for PMS    (mandatory)

TNCPMS.DAY - Message of the day            (optional)

TNCPMS.HLP - Text file sent for H command  (optional)

TNCPMS.IDX - Message index file            (created by program)

TNCPMS.INF - Text file sent for I command  (optional)

TNCPMS.LOG - PMS usage log file            (created by program)

TNCPMS.OK  - Callsigns only allowed on PMS (optional)

TNCPMS.SYS - PMS system file               (created by program)

TNCPMS.USR - PMS user details file         (created by program)

TNC\PMS\MAIL

------------

MSGnnnnn.TXT - Message files - editable if required

TNC\PMS\OLDMAIL (Only created if OLDMAIL directory exists)

------------

MSGnnnnn.TXT - Archive of killed messages - delete if required

TNC\PMS\FORWARD (All files optional)

---------------

G7JJF.LST - Text file containing lists of message subjects which

G1EQT.LST - the user requests being made available for forwarding

etc.

MUTNCBAY Driver V 1.02          7                        21/04/95

TNC\PMS\AREAINFO (All files optional)

----------------

1.INF - File displayed before listing files in file areas and can

2.INF - be used to describe what sort of files are in area

etc.

TNC\PMS\HELP (All files optional)

------------

TNCPMS.LM - Text file sent for ? LM command

TNCPMS.W  - Text file sent for ? W command

etc.

TNC\SCRIPT (All files optional)

----------

DADAUTO.SCR - General script file

DADAUTO.FWD - Script file for mail forwarding

DADAUTO.MAI - Script file for mail snatch

TNCINIT.nnn - Executed on running program

TNCEXIT.nnn - Executed on exiting program

etc.

MUTNCBAY Driver V 1.02          8                        21/04/95

4 Configuring The Software

==========================

The configuration for the program is stored in four separate

files  called  TNC.CLR,  TNC.TIM, TNCPMS.CFG  and  the  main  one

TNC.CFG. The configuration of the PMS will be discussed in detail

below.

The  file  TNC.CFG  is a standard text file  and  should  be

edited  with the inbuilt file editor or a text editor  that  does

not  add formatting characters to the text (such as  Wordstar  in

non-document  mode). Alternatively, you can use the  online  menu

driven  configuration  option  which will be  described  in  more

detail below.

The  file  TNC.CLR is automatically updated when you  change

the screen colours from the inbuilt configuration menu.

The  file  TNC.TIM is also a standard text  file  containing

time and script file information which will be described below.

4.1 Format of TNC.CFG

---------------------

Line 1

------

This tells the computer the number of terminal driver  ports

you require. You can specify from 1 to 10 ports but bear in  mind

the memory requirements for a terminal driver as discussed above.

Unless  you  are  going to make a lot  of  outgoing  connects,  I

wouldn't recommend you configure more than two or three  terminal

driver ports.

eg.       2

would define 2 terminal driver ports.

Line 2

------

This tells the computer the number of PMS ports you require.

You  can  specify from 1 to 9 ports but bear in mind  the  memory

requirements for a PMS port as discussed above. Because PMS ports

don't take up lots of memory, you can use the remainder of the 10

available ports as PMS ports, ie if you define 3 terminal  driver

ports, you can define 7 PMS ports.

eg.       2

would define 2 PMS servers.

PLEASE NOTE - Until you register the software and the on-air  PMS

ports  become  active,  this parameter MUST be  set  to  zero  or

incoming chat connects will not be recognised.

MUTNCBAY Driver V 1.02          9                        21/04/95

Line 3

------

This tells the computer the default PACLEN value to use when

sending out data, ie the maximum number of characters sent out in

one packet transmission. This value can be between 10 and 256 but

a value around 128 is usual.

eg.       128

would set PACLEN to 128 characters.

Line 4

------

This tells the computer how many hours to subtract from  the

system clock time for different time zones (eg. put 1 when in BST

and 0 when in GMT). For UK users, during BST, you will also  need

to  add  the following line into your AUTOEXEC.BAT  file  (either

hard disc or the floppy disc you boot the computer from).

SET TZ=BST6

It will of course need removing when we are back in GMT.

Line 5

------

This will be your name when you register the software. After

receiving  your registration number, you may edit this  line  and

the subsequent line to the correct text. If you try and cheat  by

guessing a name and registration number, the software will detect

this and revert to evaluation copy mode. Also, make sure you have

configured  TNCPMS.CFG with the callsign in your registered  name

else the software will revert to evaluation copy mode.

Line 6

------

This  will  be  your personal registration number  when  you

register the software. See above for info on editing this line.

Lines 7-16 (for function keys 1 to 10)

----------

These contain the text attached to the function keys.  There

are  four  fields per line. The text is each field  is  currently

limited  to 64 characters for the TNC text and 14 characters  for

the help text.

Field 1 : The  text sent to the TNC when pressing shift  and

the  function key. You can add control characters into the  text,

eg  ^M  is  return, ^[ is the escape character, ^G  is  the  bell

character etc. You can also create a delay of 0.5 seconds by  the

string '^.'.

MUTNCBAY Driver V 1.02          10                        21/04/95

You  can  also include key press emulation as  used  in  the

script  REPLY  command.  See the SCRIPT section  below  for  more

information on using emulated key presses.

Field 2 : The  text to describe the key which will appear on

the help screen.

Field 3 : The text sent to the TNC when pressing control and

the  function  key.  See above for  information  about  including

control characters in the string.

Field 4 : The text to describe the key which will appear  on

the help screen.

Line 17

-------

This line contains a single Y or N character and informs the

software  whether you wish it to generate the TNC.LOG  file.  For

people using only floppy discs instead of hard discs, disc  space

may  be short so you may want to disable the log file  generation

by putting a N here.

Line 18

-------

This line contains the amount of memory you wish to  reserve

for the inbuilt editor to edit a file in. It can be between 1 and

63 which corresponds to 1K to 63K of memory. Do not use a  figure

which  is larger than you need and a value of 16 is  recommended.

If  you  try  and  edit a file  larger  then  the  editor  memory

available,  an  error  will be generated and  you  will  need  to

increase this figure to suit.

Line 19

-------

This line contains the maximum number of lines a file  being

edited  may contain. It can be between 1 and 5000. Do not  use  a

figure  which  is  larger than you need and a value  of  1000  is

recommended. If you try and edit a file with more lines than this

figure, an error will be generated and you will need to  increase

this figure to suit.

MUTNCBAY Driver V 1.02          11                        21/04/95

Line 20

-------

This lines contains the seconds delay of keyboard inactivity

before  blanking the computer screen. A value of 0  will  disable

the  screen  blanker, any other value will  enable  the  blanker.

After  the specified number of seconds of inactivity, the  screen

will  go  blank. For the technically minded, to  accommodate  all

video  adaptors, the blanker routine simply selects video page  1

of  the  current screen mode and clears the  screen.  All  bioses

should support this feature but if the screen doesn't blank,  you

will have to disable the blanker and let me know so I can come up

with  another method of blanking the screen. Nothing  else  fancy

happens  whilst the screen is blanked but space is available  for

rent at reasonable rates ! To unblank the screen, simply press  a

key. The screen will also automatically unblank when an  incoming

connect is detected.

eg.       30

would set the screen blanker to come into operation after 30

seconds.

Line 21

-------

This line contains a single Y or N character and informs the

software  whether you wish to automatically justify text  as  you

type  it in. If you specify a Y here, the program will  pad  each

line  of  text you enter with enough spaces to ensure  the  right

margins  align as demonstrated in this paragraph you are  reading

now.

MUTNCBAY Driver V 1.02          12                        21/04/95

4.2 Format of TNC.TIM

---------------------

The format of each line in the file is given below :

HH:MM <script filename>

where  HH:MM is the time (in 24 hour format) when  you  wish

the  script  file <script filename> (stored in  the  SCRIPT  sub-

directory) to be automatically executed. The times must be stored

in chronological order.

For example, your TNC.TIM file could contain the following :

02:00 GB7DAD_2.SCR

04:00 GB7BSX_4.SCR     etc.

and use the timed execution facility to automatically log on

to  BBS's overnight at set times to download the latest  messages

for you.

When the software is first run, it interrogates the  TNC.TIM

file  (if one is present) to find the next entry to  be  executed

skipping over the times which have already gone. If there are  no

more entries for the current day, it goes back to the top of  the

file and waits for the time to arrive.

When it is time for one of the script files to execute,  the

last  terminal driver port defined in TNC.CFG will  automatically

start  to execute the relevant script file. If the last  terminal

driver port is busy, the script file will be put into a queue and

executed as soon as the terminal driver becomes free once more.

After  executing the script, the program will wait  for  the

next  timed  event,  looping back to the start  of  the  file  if

necessary.  The  time  of the next event is shown on  the  Alt  I

information screen. See below for details on script files.

MUTNCBAY Driver V 1.02          13                        21/04/95

5 How to use the Software

=========================

To  load  the software,  install the  package  as  described

above. If you have a hard disc, you may wish to add the directory

where the TNCBAY files are stored in your PATH statement.

Before running the software, you must configure and  install

the  TFPCX  software first. I will not say any  more  about  this

since  adequate  documentation  comes  with  the  TFPCX  software

telling you how to accomplish this. You can refer to my TFPCX.INI

file  as  shown in Appendix E if you want to see how  I  have  it

configured.

To  run  the  program, enter the directory  where  you  have

installed  the software (unless you have set the PATH), load  the

TFPCX  software  and simply type 'TNCBAY' (and  press  RETURN  of

course  !).  It is much simpler to load the  software  through  a

batch file and I have shown examples of ones I use in Appendix E.

After  loading  the  software,  you  should  shortly  see  a

terminal driver window which is split into two main windows.  The

upper screen is where the text received from the TNC is displayed

and   the   lower  screen  is  where  you  will  type   in   your

commands/text. If something else happens instead, please refer to

the  troubleshooting  chapter  below for  hints  on  getting  the

software up and running.

At  the  top left of the screen is the current time and  the

top  right of the screen shows either the length of  the  current

connection or the key to access the help screen (Alt F1).

Below  this is a window which displays who you are connected

to.  (This  is  picked up by the text  'CONNECTED  TO'  which  is

received from the TNC).

Between  the  two  main text windows  is  a  further  window

containing status information. From left to right, this  displays

the  number  of  buffered  transmitted  packets,  the  number  of

transmitted  packets  waiting  acknowledgement,  the  number   of

retries for the current transmission attempt, the music symbol if

the  connect bell is on, a flashing P if you have unread mail  in

your PMS, the uploading/downloading files status together with  a

bytes to go/bytes downloaded counter and the printer dump  on/off

status.

As  you  can  see,  running the software  puts  you  into  a

terminal  driver window. To monitor what is happening on  another

port  or  to select a different terminal driver  to  use,  simply

press the Alt key. This will bring up a menu of available  ports,

describing  what each is used for and showing who (if anyone)  is

currently  connected to your system.

MUTNCBAY Driver V 1.02          14                        21/04/95

You can select a different port by using the cursor  up/down

keys  then  returning  on the required port  or  more  simply  by

pressing  the  initial letter of the required port.  Selecting  a

different  port  will then swap the screen to what  is  currently

showing on the new port.

When  you are ready to exit from the program, this  must  be

done  from a proper terminal driver window in order to shut  down

the software correctly.

The  TFPCX  host  software  operates  in  two  modes  called

'command mode' and 'data mode'. In 'command mode', you are  shown

a  '*' prompt and are able to issue commands to the TNC  such  as

'C' for connect and 'D' for disconnect etc. In order to issue TNC

commands,  you  must make sure you are in 'command mode'  or  the

commands you type will either be transmitted over air or ignored.

To  enter  command mode if the '*' prompt is  not  shown,  simply

press the Escape key.

The sequence for making a sample connect is thus :

If the '*' prompt is not showing, press ESC.

Issue the connect, eg 'C GB7MAM' and press RETURN.

Use the BBS as required.

When you are ready to disconnect, either enter the 'B'  (BBS

bye command) or press ESC to return to command mode and enter 'D'

and press RETURN to disconnect.

The TFPCX manual describes other commands which you can  use

in command mode but unless you consider yourself a packet expert,

you  will not need to use any of them apart from the Connect  and

Disconnect commands described above.

5.1 General Guidelines

----------------------

When  typing text at the keyboard, eg if you are  sending  a

message,  the program will automatically wrap text as  you  reach

the  right edge of the screen. This means you can just  carry  of

typing  and  the  program  will press  'Enter'  for  you  at  the

appropriate time, moving half typed words onto the line below. If

you  have turned on auto justification in the TNC.CFG  file,  the

program will insert spaces between words to pad out each line  so

the  right  margins  align to the screen edge  giving  a  similar

effect to the layout of this paragraph.

When  entering information into an input box,  full  command

line  editing is available. You can use the cursor keys  to  move

around  the  line, delete backwards with the  backspace  key  and

delete forwards with the Del key. The Insert key toggles  between

insert and overwrite modes with an indicator at the bottom  right

hand side of the box showing the current mode.

MUTNCBAY Driver V 1.02          15                        21/04/95

When  entering  a  filename at any point, you  may  enter  a

wildcard  filename.  If  you do, you  will  enter  the  directory

browser  where  you  can select a  file  matching  your  wildcard

filename.  If  you press RETURN in an empty input box,  you  will

enter  the  directory browser with a default filename  of  '*.*'.

Pressing ESCAPE will cancel the input prompt.

To  select  from  the  light  bar  menus,  use  the   cursor

left/right  keys  to  move the bar and press  RETURN  to  select.

Alternatively, press the key corresponding to the initial  letter

of  the  required choice. You can press F1 to get a  help  screen

reminding you of these keys. ESCAPE will cancel the menu  without

making a choice.

MUTNCBAY Driver V 1.02          16                        21/04/95

6 Miscellaneous Terminal Driver Commands

========================================

Ctrl Cursor Right - Clear the TNC receive window.

Ctrl Cursor Left - Clears 'connected to' window.

TAB  (or Page Up) - This will enter the scroll  back  buffer

mode  from a terminal driver window or the monitor  window.  Here

you will be able to scroll back through the previous 200 lines of

text  received from your TNC on your current port.  Each  defined

terminal driver and monitor window maintains it's own scroll back

buffer. Use the normal cursor keypad to scroll the text. You  can

find text by pressing F and entering the search text. The  search

will only occur on text off the top of the screen. If the text is

found,  the  buffer will scroll to display it at the top  of  the

screen. Once finished browsing the scroll back buffer, press  any

key to exit. (See ALT F2-8 and ALT F10 below for more scroll back

buffer functions and ALT-N for the notepad function).

Shift Tab - This only works in split screen mode and  allows

you  to access the last 8 commands you have entered. Press  shift

tab  to  scroll  back  through the list.  To  choose  a  previous

command,  press  RETURN. This will send the command  to  the  TNC

again  but  leaves the cursor at the end so you  may  modify  the

command if you wish. To cancel this option press the space bar.

ESC - This  is  normally used to return  to  'command  mode'

(denoted by the * prompt) in order to issue commands to the  TNC.

The ESC key is also used to cancel menus and selection boxes.

Ctrl Home - This key will access the Online Cross  Reference

Help Facility which is described in more detail below.

PgDn - This key will access the list of grabbed FBB  message

headers which is described in more detail below.

@ - This is used to automatically enter remote SYSOP mode on

WORLI  and NNA type BBS's, and can also be used to  automatically

enter remote SYSOP mode on this PMS as well.

On  WORLI BBS's and when using this programs PMS,  when  you

normally  enter  a single @ (followed by RETURN),  you  get  back

three  lines of numbers from which you must choose one  line  and

use it to decode your password.

If  you  enter your personal password as the text for  Shift

Function  key  10,  now when you press @<CR>,  the  program  will

automatically  pick one of the three lines and use it  to  decode

the  password which it automatically sends to the remote end.  It

will  first confirm this action by popping up a Yes/No menu  just

to make sure you did not accidentally press the wrong key.

MUTNCBAY Driver V 1.02          17                        21/04/95

On  NNA BBS's, when you normally enter a single @  (followed

by RETURN), you get back three, two digit pairs of numbers  which

index into an 8 by 8 matrix of characters.

If  you  enter your personal password as the text  for  Ctrl

Function key 10 (as a single 64 character string made by  joining

the  eight rows together), now when you press @<CR>, the  program

will automatically detect the three pairs of digits and use  them

to decode the password which it automatically sends to the remote

end.  It  will first confirm this action by popping up  a  Yes/No

menu  just to make sure you did not accidentally press the  wrong

key.

The program detects which type of BBS/PMS you are  connected

to and which password routine to enter by counting the number  of

characters  of  the first line received from  the  remote  system

after sending the @<CR>. If it receives 8 characters, it  assumes

it   is  the  NNA's  three,  two  digit  pairs  and   sends   the

corresponding  password  back.  If more  than  8  characters  are

received,  it captures the next two lines as well and enters  the

WORLI/PMS password routine.

If  you accidentally press @<CR> and say Yes to the  pop  up

menu prompt when you really did not mean to do it, you can  abort

the automatic password sequence by pressing any key.

The  program  also operates a mail snatch facility.  If  the

program monitors a 'Mail For' beacon containing your callsign (as

stored  in  the TNCPMS.CFG file), the last terminal  driver  port

defined  in TNC.CFG will automatically start to execute a  script

file  called  '<callsign>.MAI' (stored in the  SCRIPT  directory)

where <callsign> is the call of the station holding your mail. If

the  last terminal driver port is busy, the script file  will  be

put  into  a queue and executed as soon as  the  terminal  driver

becomes free once more.

eg. if the beacon

fm GB7MAM to BEACON:

Mail For : G7JJF ALL

was  monitored  and your callsign appeared in the  Mail  For

line,  the  script  file  'GB7MAM.MAI'  would  be   automatically

executed. The script file must be stored in the SCRIPT directory.

MUTNCBAY Driver V 1.02          18                        21/04/95

7 Specialised Terminal Driver Function Keys

===========================================

Please Note : F1, F3 and F10 also work on the monitor window.

F1 - ASCII  Download. You will be asked for the name of  the

file to contain the downloaded information. If the file cannot be

created  or  you  do not enter a filename, the  command  will  be

cancelled.  If the file you specify already exists, you  will  be

given  the opportunity to overwrite the existing file  or  cancel

the download.

F2 - ASCII  Upload. You will be asked if you want to  upload

from  a  file or the clipboard. If you select file, you  will  be

asked  for the name of the file you wish to upload. If  the  file

cannot be found or you do not enter a filename, the command  will

be cancelled. Selecting clipboard will send data currently stored

in  the  clipboard  (if it is not empty  !).  During  the  upload

process, the program will periodically wait for the TNC to  empty

its  buffer  before it can send any more data. During  this  wait

period,  you  can still press the Alt key to select  a  different

port to use whilst you are waiting for the upload to finish.

F3 - PAUSE DOWNLOAD. This will temporarily suspend output of

characters to the capture file. Press F3 again to resume capture.

F4 - YAPP  Download. You will be asked for the name  of  the

file to contain the downloaded information. If the file cannot be

created  or  you  do not enter a filename, the  command  will  be

cancelled.  If the file you specify already exists, you  will  be

given  the opportunity to overwrite the existing file  or  cancel

the download. You can abort the download at any time by  pressing

the  ESCAPE  key  (and again if you cannot wait  for  the  remote

station to acknowledge your intention to abort the transfer).

F5 - YAPP Upload. You will be asked for the name of the file

you  wish  to upload. If the file cannot be found or you  do  not

enter  a filename, the command will be cancelled. You  can  abort

the  upload at any time by pressing the ESCAPE key (and again  if

you  cannot  wait  for the remote  station  to  acknowledge  your

intention to abort the transfer).

F9 - This  will  temporarily exit back to  the  DOS  command

prompt  (assuming  you have enough spare memory)  where  you  can

enter  any  DOS command. To return to the  TNCBAY  program,  type

'EXIT' (and RETURN).

F10 - This  key will dump the current TNC text  (either  from

the main screen, the previous screen or a saved screen) to either

your printer or to a disc file. It will first ask you to  confirm

your desire to dump the screen (just in case you caught the wrong

key).  It will then ask where you want to send the  dump  (either

printer  or  disc)  and  if you choose disc it  will  ask  for  a

filename.

MUTNCBAY Driver V 1.02          19                        21/04/95

8 Specialised Terminal Driver Alt Function Keys

===============================================

Please  Note  : Alt F2-F8 and Alt F10 also work  on  the  monitor

window scroll back buffer.

Alt F1 - This will display the main help screen.

Alt F2 - Set  Start Marker.  This is used to set  the  start

marker  for  a  block of text in the scroll  back  buffer.  After

pressing the Alt F2 key, use the cursor up/down keys to  position

the  highlight bar at the start of the required block then  press

RETURN.

Alt F3 - Set End Marker.  This is used to set the end marker

for a block of text in the scroll back buffer. After pressing the

Alt F3 key, use the cursor up/down keys to position the highlight

bar at the end of the required block then press RETURN.

Alt F4 - Clear Markers.  This will erase any markers set  by

Alt F2 or Alt F3.

Alt F5-Alt F8 - These  keys access four screens which can be

used  to save any text received from your TNC.  This is  done  by

accessing the scroll back buffer using the TAB key, scroll to the

required  screen of text then press ALT together with F5 to F8 to

store  the  text in which ever screen you  wish.  To  recall  the

screen at a later time, press the same key again when on the main

program  screen. Please note that all terminal drivers share  the

same four screens.

Alt F9 - Toggle  Sysop  Availability. This will  toggle  the

'sysop available for chat' status in the PMS. If you toggle  this

off, when a PMS user enters C for chat, they will immediately  be

told  the sysop is unavailable and offered the chance of  leaving

him  a  message. If you toggle this option on, when  a  PMS  user

enters  C for chat, the normal chat routine as described  in  the

PMS commands below will be followed. The current toggle status is

shown on the Alt I info screen.

Alt F10 - Dump Block. This key will dump the current  marked

text  in  the scroll back buffer to either your printer,  a  disc

file  or  the clipboard. It will first ask you  to  confirm  your

desire to dump the text (just in case you caught the wrong  key).

It will then ask where you want to send the dump (either printer,

disc  or  clipboard)  and if you choose disc it will  ask  for  a

filename

MUTNCBAY Driver V 1.02          20                        21/04/95

9 Specialised Terminal Driver Alt Keys

=======================================

Please Note : Alt D and Alt E also work on the monitor window.

Alt B - Toggle Connect Bell. When you make a connection, the

bell will normally warble. This key will toggle this action so it

can be turned off if you having a late night session.

Alt C - Connect Directory. This option allows you to make  a

connection  to  either a callsign in the MH list or  one  from  a

personal directory of callsigns.

Choosing  the  'Monitor  Heard'  option,  the  program  will

displays  it's own MH listing in a window. This list will show  a

callsign and a date/time stamp of when it was last heard. You can

then use the cursor up/down keys to select a callsign to  connect

to.  You can of course press the ESCAPE key to leave  the  window

without making a connection.

Alternatively, you can create a directory of call signs  and

corresponding  script files and use this key to pop up  a  window

for selecting someone to connect to. The directory must be stored

in  a  file  called TNC.DIR which must be  located  in  the  same

directory as the TNCBAY.EXE program . The format of each line  of

the file is shown below :

<name>,<script file>

eg.       Jon,g7jjf.scr

When  selecting Jon from the connect directory  window,  the

program  will execute the script file 'g7jjf.scr' which  MUST  be

located  in  a subdirectory SCRIPT below the main  TNC  directory

(see below for more information on script files).

Alt D - Directory Browser. See below for more information.

Alt E - Edit File. Select whether you want to edit a file or

the  current contents of the clipboard. If you select  file,  you

will  be  asked for the name of a file you wish to edit.  If  you

enter  a filename which does not exist on disc, the  editor  will

create a new blank file for you otherwise the editor will load in

the  existing  file. Selecting clipboard will  edit  the  current

contents  of  the clipboard which will initially  be  empty  when

first  running the program. The clipboard can be thought of as  a

temporary file which resides in memory rather than on disc.  This

makes it quicker to access than a disc file.

The  editor  operates  in  full screen  mode,  the  same  as

described below in the directory browser section. This option  is

useful  for creating messages offline ready to ASCII upload  when

online. The editor also multitasks so you can still switch  ports

using the Alt key whilst editing a file.

MUTNCBAY Driver V 1.02          21                        21/04/95

Alt G - Perform Script File. See below for more information.

Alt H - Help  Screen.  This will show a help screen for  the

Alt keys.

Alt I - Info  Screen. This will redisplay  the  registration

screen showing your registered name and number. It will also show

the time when the next script file will be automatically executed

(if  active), the current script file command being executed  (if

active), whether the sysop is available for a chat in the PMS and

finally, if you have any unread mail in the PMS.

Alt J - Monitor  Connects.  This  key only  works  from  the

monitor  window  and  brings up a menu  containing  up  to  three

options.

The  first option will list the last 10 stations  that  were

monitored connected to each other.

The  second  option allows you to monitor both  sides  of  a

connection fullscreen. This is done by stringing together all the

monitored  packets  and displaying them in a  continuous  stream.

Duplicate  packets  that  are sent as retries  are  detected  and

stripped  out  to avoid corrupting the display.  All  other  data

received  by the TNC is still processed and can be viewed on  the

scroll back buffer. To select which connection to monitor, simply

select  it from the list after choosing the monitor  option.  The

connection  list  is updated continuously and  you  can  reselect

another connection at any time.

The  third  option only appears after entering  the  monitor

mode.  This  option  cancels the current  monitor  operation  and

reverts back to displaying all monitored data.

Alt L - 7PLUS Code File. See below for more information.

Alt N - Enter  notepad. This will bring up a quarter  screen

notepad in the top right hand corner of the screen.  It  actually

enters  the  file editor on a file called  NOTEPAD.<port>  (where

<port> is the terminal driver port you are currently using) which

is  created in the home TNC directory. The notepad is  active  on

the  main screen and on the scroll back buffer. The notepad  also

multitasks so you can still switch ports whilst editing a file.

Alt O - Enter PMS locally. This will allow you to connect to

your PMS. See the section below for more information on using the

PMS.

Alt P - Toggle  Printer.  This  will toggle  output  to  the

printer.  When on, all characters received from the TNC  will  be

dumped to the printer.

MUTNCBAY Driver V 1.02          22                        21/04/95

Alt Q - TNC  I/P  from file. This allows you to  replay  TNC

captured  text  through the program as though it  had  just  been

received  again  from the TNC. Upon selecting this  command,  you

will  be  asked for the file to replay. All text from  this  file

will  be  used as subsequent TNC input until the  file  has  been

fully  processed.  The program will then revert  back  to  taking

input from the TNC.

Alt S - Change  Setup.  This  is  the  online  configuration

option.  Here  you can change the port configuration,  the  video

colours,  etc plus enter your registration information  when  you

have registered the software. After altering any of the settings,

the  configuration file TNC.CFG and TNC.CLR are updated to  match

the  new  values. If you alter any of the screen colours,  it  is

advisable  to rerun the software so the software can repaint  the

display in the correct colours.

Alt T - Send Date/Time.  This will send the sequence :

DD-MON-YY  HH:MM:SS

to   the  TNC  - useful  for  date/time  stamping  messages  when

connected to another user.

Alt U - UUencode/decode File.  This option is used to encode

and  decode files conforming to the UUcode protocol.  The encoder

can  split  the  source  file into any  number  of  smaller  file

sections  and  the decoder can subsequently merge any  number  of

consecutive file sections into the original source file.

Selecting  the  encode option will ask you  for  the  source

filename.  You can of course use the browser to select a file  by

entering  a  wildcard file spec. After entering  a  valid  source

filename,  you will be asked for the destination file.  You  must

not enter an extension since the protocol will automatically  add

an  extension starting at .U01 and incrementing by one  for  each

section of the encoded file. You will also be asked for the  size

of each encoded section. This will default to 4K (4096 bytes) but

you can alter this to any size you wish. However, it is advisable

to keep the value low to avoid sending large amounts of data over

the packet airwaves.

After  entering  all the data, the encoded file(s)  will  be

generated.  They will be stored in the current directory, ie  the

one  from where the program was run or the last directory  chosen

from the browser if appropriate.

MUTNCBAY Driver V 1.02          23                        21/04/95

Selecting the decode option will ask you for the filename of

the  first  file  section.  You must specify  the  full  filename

including  the extension.  The initial file is assumed to have an

extension  containing  either a 01 or 1,  eg .U01  or  .UU1.  The

program  will  proceed  to  generate the  original  file  merging

together all the necessary file sections which must be present in

the  same directory.  If any file sections are missing or any  of

the  files  are invalid an error will be produced.  If  all  goes

well,  the decoded file will be written to the current directory,

ie  the one from where the program was run or the last  directory

chosen from the browser if appropriate.

Alt W - Toggle  Connect  Window/Status Line. When  making  a

connection,  the callsign of the person you are connected  to  is

shown  in  the  connect window. (This is picked up  by  the  text

'CONNECTED TO' which is received from the TNC. If you do not wish

this  to be shown, you can toggle the window off  thereby  giving

you  two  more lines for the TNC text window. NOTE :  You  cannot

toggle  the window off whilst there is text in the box. Clear  it

first with CTRL-Cursor Left.

On  full  screen  mode, this command will  also  toggle  the

status  line found on split screen mode and put it at the  bottom

of the screen (reducing screen height to 23 lines).

Alt X - Exit. This will exit the program. You will be  given

the chance to confirm your actions before the program exits. This

command must be issued from a terminal driver window in order for

the software to shut down correctly.

Alt Z - Toggle  Full  Screen.  This will  make  the  program

display  all input/output to a single full screen  window  rather

than two separate windows for user input and tnc output.

MUTNCBAY Driver V 1.02          24                        21/04/95

10 Directory Browser

====================

Choosing this option will bring up a powerful directory file

maintenance  window.  Here you can browse through directories  on

your  discs,  copy files,  rename files,  view files (in HEX  and

ASCII), delete files, edit files and print files etc.

The  following keys operate when using the directory browser

(note  that  the  ^  symbol means  press  CTRL  followed  by  the

following key, eg ^C means press CTRL-C) :

Cursor key pad - Use to move about the current display.  The

home,  end, page up and page down keys also operate to  move  the

cursor in larger steps.

Return - Use  to  select the current  highlighted  filename.

When  over  a  directory,  it will change  the  display  to  that

particular directory. When on a file, if the browser was  entered

from  a filename input box, this will select the current file  as

though it was entered in the input box.

TAB - Use  to  alter the display format.  Pressing TAB  will

toggle between a four column short display showing only filenames

and  tag  markers  and  a long display  showing  two  columns  of

filenames and corresponding file sizes.

ESC - Use  to exit from the directory browser and return  to

the previous screen.

C - Use  to  copy  the  current file (not  directory  !)  to

another  area of your disc,  optionally changing its name  during

the copy process.  You will be asked for a wildcard filename  you

wish to copy the file as.  If you are copying the file to another

directory  or disc,  you can simply default on '*.*' to keep  the

same  name.  Alternatively,  you  could  for example  change  the

wildcard  filename  to  *.BAK  to create a  backup  of  the  file

elsewhere.  You  will  next be asked where you wish to  copy  the

file. This must be the full pathname of the destination directory

which  MUST already exist.  You can specify another drive in  the

pathname if you wish. (When entering the pathname, you MUST enter

the  final '\' character unless you just press RETURN on a  blank

path to copy it to the current directory).  Finally,  you will be

asked if you wish to automatically replace existing files. If you

say YES,  any file with the same name you wish to create will  be

overwritten. If you say NO, you will be prompted to overwrite any

file which already exists of the same name.

^C - Use  to  copy  all  the tagged files  from  the  current

directory to another area of your disc.  (You must of course  tag

the required files first before using this command).  After being

asked  the  same  questions as above,  each tagged file  will  be

copied to the new pathname.

MUTNCBAY Driver V 1.02          25                        21/04/95

D - Use to delete the current file or directory.  If pressed

on  a  filename,  you will be asked to confirm that you  wish  to

delete it.  If pressed on a directory name, only if the specified

directory  is  empty  will it be deleted.  You are not  asked  to

confirm the deletion of a directory.

^D - Use to delete all the tagged files.  (You must of course

tag the required files first before using this command). You will

be  asked if you wish to automatically delete all files.  If  you

answer  YES,  all  tagged files will be deleted otherwise if  you

answer  NO,  you  will be asked to confirm the deletion  of  each

tagged file.

E - Use to edit the current file. The file editor is a  full

screen editor based on the Wordstar command structure which seems

to be the most common in existence. A summary of the keys used in

the editor is given in Appendix B.

F - Use to change the current drive,  directory and wildcard

filespec.  Upon  using this command,  you will be allowed to edit

the current path as shown at the top of the window.

L - Use this to quickly log onto another drive without going

through the F option described above.  You will be presented with

a  menu of all available drives including those defined with  the

SUBST  command.  Simply  select a drive to log onto  its  current

directory showing all files in it.

M - Use to make a new directory under the current directory.

You will be asked to enter the new directory name.

P - Use  to print the current file to your  printer.  Whilst

the file is being printed, you may abort the printout by pressing

a  key.  After  the printout is complete,  a page throw  will  be

performed on the printer.

^P - Use to print all tagged files to your printer. (You must

of  course  tag  the  required  files  first  before  using  this

command).  Each  file will be printed as described above for  the

single file print option.

R - Use  to rename the current file.  You will be  asked  to

enter  the wildcard filename you wish to rename the current  file

as.

^R - Use to rename all the tagged files.  (You must of course

tag the required files first before using this command). You will

be  asked  to enter the wildcard filename that each  tagged  file

will be renamed as.

T - Use  to  tag  the current file.  When a  file  has  been

tagged,  a  diamond will appear after the filename on the screen.

After a file has been tagged, it can be used with all the control

key functions described in this section.

MUTNCBAY Driver V 1.02          26                        21/04/95

^T - Use to tag ALL files in the current directory. See above

for information about tagged files.

U - Use  to  untag the current file.  When a file  has  been

untagged,  the  diamond  will  disappear after  the  filename  on

screen.   Also,  the file will no longer be available for use  by

the control key functions described in this section.

^U - Use  to untag ALL files in the  current  directory.  See

above for information about untagged files.

V - Use to view the current file in ASCII/HEX mode. The file

viewer  is similar to the one provided in XTREE.  Again,  use the

cursor keys to scroll around the file,  press S to set a  marker,

press  G  to go to a marker, press H to  toggle  HEX/ASCII  mode,

press  T to set the tab spacing and press RETURN to exit back  to

the browser.

MUTNCBAY Driver V 1.02          27                        21/04/95

11 7PLUS File Encoding

======================

The  program supports an option to encode and  decode  files

conforming to the 7PLUS protocol which was designed by Axel Bauda

DG1BBQ  of Germany to whom I am grateful for putting  the  source

code  into  the public domain which makes it possible for  me  to

incorporate it into my software.

The  encoder  can split the source file into any  number  of

smaller file sections and the decoder can subsequently merge  any

number  of  consecutive file sections into  the  original  source

file.

The  advantages  of  7PLUS  encoding/decoding  is  that  the

protocol  can detect and automatically correct  single  character

errors  in  transmission  and also  generate  'error  files'  and

'correction  files' which allow you to only re-send the parts  of

file  that were corrupted in transmission rather than  the  whole

file again.

To  encode  a file, use the Alt L option and  select  Encode

File.  You will then be asked for the source filename,  the  file

section  size and the file end string. You can of course use  the

browser to select a source file by entering a wildcard file spec.

If  the  source file is larger than the file  section  size,  the

encoded file will be split into portions, each approximately  the

file  section size in length. This will default to about  4K  but

you can alter this to any size you wish. However, it is advisable

to keep the value low to avoid sending large amounts of data over

the packet airwaves. The file end string will be appended to each

file  section  and  will default to '/ex' which  is  useful  when

uploading the files to a BBS. However, you can delete this out if

required.

After  entering  all the data, the encoded file(s)  will  be

generated.  They will be stored in the current directory, ie  the

one  from where the program was run or the last directory  chosen

from  the  browser if appropriate. If the encoding  process  only

resulted  in  a single file, it will have an extension  of  .7PL.

However,  if multiple files were generated, the  extensions  will

start at .P01 and increment in hex upto .PFF.

To  decode  a file, use the Alt L option and  select  Decode

File. You will then be asked for the source filename and the file

end string. You can of course use the browser to select a  source

file  by entering a wildcard file spec. Select the .7PL  file  if

there is only one section or the .P01 file if there are  multiple

sections. The program will proceed to generate the original  file

merging  together all the necessary file sections which  must  be

present  in the same directory. If any file sections are  missing

an  error  will be produced. If all goes well, the  decoded  file

will be written to the current directory.

MUTNCBAY Driver V 1.02          28                        21/04/95

If  any  errors were detected in the decoding  process,  the

program  will  either  automatically correct them if  it  can  or

generate an 'error file' called '<source_filename>.err'. You  can

then  send this file to the originator of the file to obtain  the

corresponding 'correction file'.

After   receiving  the  'correction  file'  back  from   the

originator,  to  correct your files, you must select  the  Decode

File  option  again and put <source_filename>.cor as  the  source

filename.  This will correct your files ready for  decoding  once

more. If the original corruption was bad, further error files may

be  produced and you will have to repeat the above process  until

all  errors  are removed. You can combine the  above  correction,

decoding  procedure  be specify the source  filename  without  an

extension.  If  the file with the .cor extension  exists  in  the

current  directory,  the program will automatically  perform  the

correction and decoding in one step.

If after uploading a 7PLUS encoded message to a BBS, a  user

sends  you  an 'error file' due to message corruption,  you  will

need  to  generate  a correction file for him. This  is  done  by

choosing  the 'Generate Correction File' off the Alt L menu.  You

will be asked for the source filename, the error filename and the

file  end  string. This source filename must be the same  as  the

original  file you uploaded. The error filename will be  the  one

sent  from  the  user and you can of  course  use  the  directory

browser  to select it. The file end string is again optional  and

will be appended to the resulting correction file. After entering

all  the  data,  the program will generate  the  correction  file

called <source_filename>.cor which you can then send to the  user

to correct his file corruption.

If  several users send error reports, you can  combine  them

together  and  produce a single correction file for  both  users.

This is done by choosing the 'Join Error Reports' option from the

Alt  L  menu. You will be asked for the source filename  and  the

second filename. Specifying both filenames will merge the  second

file  into  the  source file. If you leave  the  second  filename

blank,  the program will look for <source_filename>.e01 and  join

it to the source file, then look for <source_filename>.e02 and so

on.

Finally,  to  aid  extracting separate 7PLUS  files  from  a

capture log, you can choose the final option from the Alt L  menu

which  is  called 'Extract File'. You will be asked for  the  log

filename  and  a search string. If you leave  the  search  string

blank,  all  7PLUS  from the capture log will  be  extracted  and

placed  in the current directory. If you specify a search  string

however,  only those files containing the search string in  their

filename will be extracted.

MUTNCBAY Driver V 1.02          29                        21/04/95

12 Script Files

===============

Script   files  are  very  useful  for  creating   automatic

procedures  which  can  be used for anything  from  automatically

logging  onto a BBS,  spooling all your messages to a  disc  file

then  disconnecting  to automatically forwarding mail to  another

user. The uses are only limited by your imagination.

The  basic  idea  of a script file is  for  the  program  to

monitor  the incoming text for a certain string or any one  of  a

selection  of strings then automatically replying with  something

else  or  performing a certain action. To do this,  you  need  to

create a text file containing lists of strings to monitor for and

strings to reply or actions to perform. The string you reply with

can  also  contain function or Alt key presses to  perform  their

normal action.

The format of the file consists of lines beginning with  the

following keywords :

WHEN                     REPLY

DO                       UNTIL

GOTO                     :<label>

DISPLAY                  PRINT

READ                     IF

SCAN                     ENDSCAN

STOP                     ;

LOG                      ENDLOG

WRITELOG                 CLS

COLLECT                  FORWARD

REVERSE                  RUN

FBBCOLLECT               FBBREAD

CMD

You must make sure they are spelt correctly with exactly one

space after the keyword if it uses a parameter.

The syntax of each of the keywords is given below :

CMD

---

This keyword forces the TNC back to command mode and must be

used before issuing a Connect or Disconnect command.

WHEN <text>

-----------

This  keyword halts execution of the script file  until  the

<text> is monitored in the incoming TNC text. If the text you are

monitoring for contains a variable number, eg a port number in  a

BBS  prompt, you can use the # wildcard character in  the  <text>

field which will match any number in the incoming text.

MUTNCBAY Driver V 1.02          30                        21/04/95

For example, if your BBS prompt is :

(2) GB7MAM>

you could use the following command to monitor for it :

WHEN (#) GB7MAM>

REPLY <text>

------------

This keyword sends the <text> to the TNC. See below for  the

allowed characters in <text>.

DO

--

This  keyword  has  no parameters and forms a  loop  in  the

script  file  in  conjunction  with  the  UNTIL  <text>   keyword

described below.

UNTIL <text>

------------

This  keyword  signifies the end of the DO ..  UNTIL  <text>

loop.  The  statements between DO and UNTIL  are  executed  until

<text>  is monitored in the incoming TNC text. You can  nest  the

DO..UNTIL  loop up to 10 levels. See below for an  example  using

the  DO..LOOP  keywords. The <text> can contain  the  #  wildcard

character as described in the WHEN command above.

GOTO <label>

------------

This keyword causes execution of the script file to continue

at the line defined with <label>. The search for the label starts

at the top of the file and continues until the label is found  or

the  end of the script file is reached. An error is generated  if

the specified label cannot be found.

:<label>

--------

This  keyword is used in conjunction with the  GOTO  keyword

and identifies a place in the script file.

eg.

:TOP

DISPLAY Hello World !^M

GOTO TOP

will send 'Hello World !' to the display forever !

MUTNCBAY Driver V 1.02          31                        21/04/95

DISPLAY <text>

--------------

This keyword will display <text> in the user window  without

sending it to the TNC. This is useful for displaying any  message

to  the user about the outcome of script file operation as  shown

above !

PRINT <text>

------------

This  keyword  will  print <text>  to  the  printer  without

sending it to the TNC. This is useful for displaying any  message

to  the user about the outcome of script file operation as  shown

above !

READ <prompt>,<subject1>,<subject2>,...

---------------------------------------

This  is  a very powerful command and is used  when  listing

mail  on  a BBS. With this command, you can get  the  program  to

automatically  issue  'read message' commands for  every  message

that contains subject information that interests you.

eg.

REPLY L^M

READ GB7BSX,IBM,BBC,DANPAC,MODS,G3ZHI

This would issue a message list command to the BBS and  then

monitor  all  the  incoming  text until  the  GB7BSX  prompt  was

monitored  AT THE START OF A LINE. For every line that  contained

any  of  the subjects listed in the READ command, a  'R  <num>^M'

command  would  be  issued where <num>  was  the  message  number

displayed at the start of each message header.

For  example,  if the following text was received  from  the

initial L^M command,

Msg# Stat Size  To    From   @ BBS  Date/Time Title

5613 B     322 ALL    GW4AZW GBR    1028/1151 Have 2 813's new,swap.

5612 B     995 BBC    G3UDA  GBR    1028/1149 MUSIC 5000 AMPLE Users

5611 B     509 ALL    G6FUM  GBR    1028/1148 AR-2001 RX MODS HELP.

5609 BI    238 ALL    G3ZHI  GBR    1028/1113 LEICESTER SHOW PARK

5607 B     490 ALL    G8VHB  GBR    1028/0935 anyone using standard C58

GB7BSX MBX (B,C,D,H,?,I,J,K,L,N,R,S,T,U,V,W) [2598] 2>

the program would send :

R 5612^M

R 5611^M

R 5609^M

MUTNCBAY Driver V 1.02          32                        21/04/95

to  the  BBS to read the specified messages. The  next  read

command  is only sent after the last message had been  completely

read  (ie  when  the BBS prompt as defined in  the  initial  READ

command is monitored again.) This stops some BBS systems aborting

the  current message if you issue another read command before  it

has finished sending it.

If  your BBS prompt changes each time it is  displayed,  for

example  if it contains the connection port number, you  can  use

the # wildcard character in the prompt field which will match any

number in the BBS prompt. For example, if your BBS prompt is :

(1) GB7BSX MBX >

the above READ command would be :

READ (#) GB7BSX,IBM,BBC,DANPAC,MODS,G3ZHI

IF <text>

---------

This  keyword is used as part of the SCAN..ENDSCAN loop  and

will  be described below. The <text> can contain the  #  wildcard

character as described in the WHEN command above.

SCAN

----

This  keyword  has  no parameters and forms a  loop  in  the

script file in conjunction with the IF <text>, ENDSCAN  keywords.

Using  this  loop,  you can scan for up to  10  different  <text>

strings  at a time, and depending which one is found,  execute  a

separate  series  of script keywords. This is  very  useful  when

logging  onto  a BBS through a node for example,  where  you  can

either  monitor  for the BBS prompt or a busy  reply  and  either

continue with an automatic mail download or logoff from the  node

if the BBS is busy.

An example will best illustrate this construct.

CMD

REPLY C GB7DAD^M

SCAN

IF busy

REPLY B^M

DISPLAY GB7DAD is busy !!^M

IF GB7DAD>

REPLY LM^M

WHEN GB7DAD>

REPLY B^M

ENDSCAN

MUTNCBAY Driver V 1.02          33                        21/04/95

(The  indentation  can be used to add clarity  to  the  loop

constructs in the script files as the initial spaces are  ignored

by the program).

This  script file will issue the connect request  to  GB7DAD

and enter the SCAN, ENDSCAN loop. The incoming text will then  be

monitored until one of the text strings after the IF keywords  is

encountered. Once one is found, the script keywords following the

IF  statement  is  then  executed until  another  IF  or  ENDSCAN

statement  is  found.  Execution then  continues  with  the  next

statement after the ENDSCAN keyword.

Thus,  if  the  text  'BBS is busy,  please  try  later'  is

received,  the  first  IF takes control, logs off  the  node  and

display  the message to the console. It then encounters an IF  so

jumps  out  of the SCAN..ENDSCAN loop and stop execution  of  the

script file because there are no more commands to execute.

However, if the connection is made and the normal BBS prompt

is  received, the second IF takes control and continues  to  list

all  your  messages before logging off. It  then  encounters  the

ENDSCAN  statement  so jumps out of the  SCAN..ENDSCAN  loop  and

stops  execution  of the script file because there  are  no  more

commands to execute.

The  SCAN..ENDSCAN  loops can be nested up to  any  required

depth for very complicated conditional script file execution.

ENDSCAN

-------

This  keyword  has  no parameters and forms a  loop  in  the

script  file in conjunction with the IF <text> and SCAN  keywords

described above.

STOP

----

This  keyword has no parameters and stops execution  of  the

script file without completing it.

;

-

This  keyword tells the program to ignore the  remainder  of

the characters on the line and so is used to add comments to  the

script file.

MUTNCBAY Driver V 1.02          34                        21/04/95

LOG <file>

---

This command is used in conjunction with the ENDLOG command.

It  is  similar to the 'REPLY {F1}capture.log^M' command  but  is

more useful. Using the REPLY command above, the filename is fixed

all  the time so you cannot use the same script  file  containing

this  command  more  than  once, say  overnight,  to  log  on  at

different times to download your mail. Using the LOG command, you

specify  the filename, and the program automatically  creates  an

extension  for  you, starting at .000 (or if some  files  already

exist,  with  the next unused extension number)  which  increases

sequentially  every time the LOG command is issued. The log  file

is  stored in the LOG directory and all subsequent text  received

will  be  stored  in it as is normal.  Thus  the  command  'REPLY

{F1}capture.log^M' could be replaced by 'LOG capture'.

ENDLOG

------

This  keyword has no parameters and is used  in  conjunction

with the LOG command described above. It is equivalent to  'REPLY

{F1}' and closes the LOG capture file.

WRITELOG <text>

---------------

This command is used in conjunction with the LOG command. It

appends  <text>  to  the current log file  opened  with  the  LOG

keyword. If a log file is not active, this command is ignored.

As well as containing normal text, the <text> parameter  can

also  contain optional macros which when expanded can be used  to

write the current date or time to the log file as well, eg

WRITELOG Today is $D and the time is $T

will  write 'Today is 21/04/95 and the time is 19:19' to the  log

file.

CLS

---

This  command simply clears the TNC output window and  moves

the  cursor to the top of the screen.

MUTNCBAY Driver V 1.02          35                        21/04/95

COLLECT <label>,<whoto1>,<whoto2>,...

-------

This keyword is used in conjunction with the FORWARD keyword

and  is used to automatically collect together mail from the  PMS

mail  files  which need forwarding to another user  or  BBS.  You

specify  which  mail  you  wish  to  forward  with  the   <whoto>

parameters.   These  are  matched  with  all  un-forwarded   mail

addressed  <whoto> @ <whoto> (matches if <whoto> is contained  in

either  callsign or home BBS). If there is mail to  forward,  the

script file continues to execute, else a GOTO <label> command  is

performed to skip the forwarding part of the script file. You may

use  the GOTO to skip to the next forward routine in  the  script

file or to exit the file.

FORWARD (BBS) (FBB)

-------

This keyword is used in conjunction with the COLLECT keyword

and performs the actual forwarding part of the procedure. If  you

specify the optional BBS or FBB parameter, either FBB  compressed

forwarding  or normal BBS forwarding will take  place,  otherwise

the  messages will be sent as though typed live at the  keyboard.

Upon  executing  the FORWARD command, the program will  send  the

messages, kill them and mark them as being forwarded.

eg. if your PMS had the following mail,

Msg# ST  Size    To    @ BBS    From  Date/Time Title

3      295   G0HMZ  GB7FLG   G7JJF 0201/1912 Forward test

2      154   G6YAL           G7JJF 0201/1912 Hello

1     1049   G1EQT  GB7FLG   G7JJF 0201/1912 Test

the script file :

COLLECT NONE,GB7

CMD

REPLY C G6YAL-8^M

WHEN ?

REPLY X GB7FLG^M

WHEN GB7FLG>

FORWARD

REPLY B^M

:NONE

DISPLAY Finished !!

would log onto GB7FLG and forward messages 1 and 3.

You would normally create a large script file containing the

forwarding routines for all likely users and add an entry to  the

TNC.TIM  file to forward all your mail overnight to the  relevant

people.

MUTNCBAY Driver V 1.02          36                        21/04/95

NOTE : Unless  you consider yourself to be an expert packet  user

who  really know's what's what, you will probably need to  ignore

the next section.

If  you  are friendly with your local BBS sysop,  you  could

arrange  for him to make you a BBS on his board. Also ask him  to

edit  White Pages and the local user database so your homebbs  is

shown   as   your   full  hierarchical  address,   eg   mine   is

G7JJF.GB7MAM.#23.GBR.EU.  This would then allow you to do  proper

forwarding  and reverse forwarding between his BBS and your  PMS.

If  this is the case, you would then need to alter to  syntax  of

the FORWARD command above to be :

FORWARD BBS

or

FORWARD FBB

if the remote BBS performs FBB compressed forwarding.

You  would  also need to alter the script file slightly  to  send

different prompt's when you are a 'BBS' user.

The  following  script file will reverse/forward mail  using  BBS

forwarding :

CMD

REPLY C GB7MAM^M       } Normal connect

WHEN GB7MAM BBS >      } bit at top

; Start of reverse/forward bit

REPLY [BAY-1.02-HM$]^M

WHEN >

REVERSE BYE            } See below

COLLECT NONE,GB7MAM,G1EQT,EU

FORWARD BBS

:NONE

REPLY B^M

:BYE

STOP

and the following script file will reverse/forward mail using FBB

compressed forwarding (much simpler !) :

CMD

REPLY C GB7MAM^M       } Normal connect

WHEN GB7MAM BBS >      } bit at top

; Start of reverse/forward bit

REPLY [BAY-1.02-BFHM$]^M

COLLECT NONE,GB7MAM,G1EQT,EU

:NONE

FORWARD FBB

MUTNCBAY Driver V 1.02          37                        21/04/95

Upon executing either script file, the program will  receive

any waiting mail and send any local messages (either as bulletins

or  as  personal  messages), kill them and  mark  them  as  being

forwarded.

Note  that the FORWARD FBB command  automatically  downloads

mail waiting for you so there is no need for the REVERSE command.

Also,  you  are  automatically disconnected  from  the  BBS  when

forwarding is finished so the commands to disconnect you as shown

in  the  first script file are not required. This  makes  writing

script  files to perform FBB compressed forwarding  much  simpler

and straight forward.

REVERSE <label>

-------

This  command is again only for expert users and is used  to

request  messages  or bulletins from BBS's to  be  collected  and

stored in the local PMS. You will need to arrange with your local

BBS sysop for him to create a forwarding file for you.

Upon executing this command, the program will issue requests

for messages and stores them in your PMS until there are no  more

to   be  received.  I  have  noticed  that  I  occasionally   get

disconnected when downloading messages this way due to slow links

so  I  have added a label parameter. If you do get cut  off,  the

program  will automatically issue a GOTO <label> to skip  to  the

next bit of the script file.

If you are performing FBB compressed forwarding, you do  not

need to use the REVERSE command to request mail since the FORWARD

FBB  command  described above automatically  downloads  all  mail

waiting for you.

RUN <command>

---

This   keyword   will  execute  <command>  to   run   either

a  .EXE, .COM, .BAT file or an internal DOS command whilst  still

remaining  in the TNC driver program. If you do not  have  enough

free  memory when using this keyword, the <command> will  not  be

executed.

eg.       RUN DEL MYFILE.DAT

MUTNCBAY Driver V 1.02          38                        21/04/95

FBBCOLLECT <label>,<subject1>,<subject2>,...

----------

This keyword is used in conjunction with the FBBREAD keyword

and is used to automatically scan the list of grabbed FBB  Header

broadcasts  for  mail  which interests you (see  below  for  more

information on header broadcasts). You specify which messages you

wish to read with the <subject> parameters which are matched with

all unscanned message headers. If there are messages which  match

any  of  the  parameters in your subject list,  the  script  file

continues  to  execute,  otherwise  a  GOTO  <label>  command  is

performed to skip the next part of the script file.

FBBREAD <label>

-------

This  keyword  is used in conjunction  with  the  FBBCOLLECT

keyword  and  is used to download the matched messages  from  the

local BBS. If the download process gets disconnected due to  slow

links,  the  program will automatically issue a GOTO  <label>  to

skip to the next bit of the script file.

The following script file shows an example of the FBBCOLLECT

and FBBREAD commands in operation :

FBBCOLLECT NONE,GB7MAM,JJF,MUBAY,BPQ,INTERNET,CDROM,CD-ROM

CMD

REPLY C GB7MAM^M

WHEN >

REPLY [BAY-1.02-$]^M

WHEN >

FBBREAD NONE

REPLY B^M

:NONE

The  scanned message pointers are automatically  updated  as

each  message is downloaded so next time the script file is  run,

only new message headers will be scanned.

MUTNCBAY Driver V 1.02          39                        21/04/95

12.1 Sample Script Files

------------------------

A  simple  example may illustrate the use  of  script  files

better.

REPLY {F1}capture.log^M       (or LOG capture)

CMD

REPLY C GB7DAD^M

WHEN GB7DAD>

REPLY LM^M

WHEN GB7DAD>

REPLY B^M

REPLY {F1}                    (or ENDLOG)

The first line shows how to include function keys in  script

files.  You  can include cursor, function, shift  function,  ctrl

function,  alt  function  or alt keys in this way.  Each  key  is

enclosed in { } characters.

A shift function key is defined by {SF1} to {SF10}

A ctrl function key  is defined by {CF1} to {CF10}

An alt function key  is defined by {AF1} to {AF10}

An alt key           is defined by {AA} to {AZ}

Cursor up            is defined by {CU}

Cursor down          is defined by {CD}

Cursor left          is defined by {CL}

Cursor right         is defined by {CR}

Cursor home          is defined by {CH}

Cursor end           is defined by {CE}

Page up              is defined by {PU}

Page down            is defined by {PD}

You  can also use other keys which are not included  in  the

above list by specify the high byte of its extended key code as a

number  enclosed in { } 's, eg. the extended key code for  shift-

Tab (to recall the history list) is 0x0f00. You would enter  {15}

to simulate pressing the shift-Tab key.

The  first line of the example script file above thus  says,

simulate F1 to start downloading a file.  The filename is entered

as capture.log followed by return.  Note the use of control  keys

in  the text strings.  You can use ^A to ^Z,  ^[ is escape and ^.

pauses for 0.5 seconds (excellent for creating rolling demos).

The second line forces the TNC back to command mode.

The  third  line  then issues a  connect  to  GB7DAD,  again

followed  by  return. You must remember to put ^M at the  end  of

each line where you would normally press return.

MUTNCBAY Driver V 1.02          40                        21/04/95

The  fourth line then monitors the incoming text  until  the

GB7DAD>  prompt is detected. A LM command is then issued to  list

all  my messages. The prompt is then monitored for, to know  when

all  the  text has been displayed. The B (bye)  command  is  then

issued  and the download file is closed by simulating the F1  key

again.

You  may find it useful to build up a collection  of  script

files for connecting to users through nodes which requires  cross

connects or port specifiers.

You could thus have a script file such as

CMD                      (Back to command mode)

REPLY C G6YAL-8^M        (Issue connect)

WHEN Help ?              (Wait for prompt line)

REPLY X GB7DAD^M         (Issue cross connect)

You  can get very complicated when using script  files,  for

example,  checking for busy nodes or no BBS ports available  when

connecting.  I  will  leave you to work out  what  the  following

script file does !

CMD

REPLY C G6YAL-8^M

SCAN

IF ?

;   Connected

IF busy

STOP

ENDSCAN

REPLY X GB7DAD^M

SCAN

IF GB7DAD>

;   Connected

IF RETRIED

REPLY B^M

STOP

IF ###LINK BUSY AT NODE G6YAL-8

REPLY X DANPAC^M

SCAN

IF DANPAC   #

REPLY BBS^M

SCAN

IF Please try later

REPLY B^M

STOP

IF GB7DAD>

;           Continue

ENDSCAN

IF busy

REPLY B^M^[D^M

STOP

ENDSCAN

ENDSCAN

MUTNCBAY Driver V 1.02          41                        21/04/95

LOG DADAUTO

REPLY L^M

READ GB7DAD,G7JJF,DANPAC,SOF,IBM,BBC,MOD,NOD,NET,SYSOP

ENDLOG

REPLY B^M

DISPLAY Finished !!^M

STOP

I have had numerous request from people asking for copies of

script files which I use to perform my mail forwarding and  other

bits.  The following script files are ones I use on a day to  day

basis.

BYE.SCR

-------

REPLY ^MJon.                                 Message Sent {AT}^M

REPLY /=====================================\^M

REPLY | G7JJF @ GB7MAM.#23.GBR.EU      AX25 |^M

REPLY | jon@g7jjf.demon.co.uk      Internet |^M

REPLY +-------------------------------------+^M

REPLY |      Providing Support For :        |^M

REPLY |TNCV142, MUTNC205, MUBAY102, BPQAX25 |^M

REPLY \=====================================/^M

REPLY /EX^M

I  use this script file when signing off my messages  and  I

call it by pressing Ctrl-F1, setup with the following :

"{AG}C:\TNC\SCRIPT\BYE.SCR^M","Exit MSG"

GB7MAM.FWD

----------

CMD

REPLY C MAMBBS^M

WHEN >

REPLY [BAY-1.02-BFHM$]^M

COLLECT NONE1,G1EQT,GB7,.EU,DANPAC,.USA,GBR,.CAN,NZL

:NONE1

FORWARD FBB

This   script   file   connects  to   my   local   BBS   and

reverse/forwards all mail using FBB compressed forwarding.

MUTNCBAY Driver V 1.02          42                        21/04/95

GB7MAM.FBB

----------

CMD

REPLY C GB7MAM^M

FBBCOLLECT NONE,GB7MAM,JJF,MUBAY,BPQ,INTERNET,CDROM,CD-ROM

REPLY [BAY-1.02-$]^M

WHEN >

FBBREAD NONE

REPLY B^M

:NONE

I  sometimes  use this script file to scan the list  of  FBB

header  broadcasts and automatically connect to my local  BBS  to

grab  them.  However, I prefer to use  the  interactive  facility

described below.

12.2 Automatic Script Files

---------------------------

You  can get the program to automatically perform  a  script

file for each terminal driver when started by running the program

as :

TNCBAY -S<filename>

where <filename> is the name of a script file, eg.

TNCBAY -Sstartup.scr

Alternatively,   if   the  program  finds  a   file   called

TNCINIT.<port>  in a directory called SCRIPT under the  TNC  home

directory, this will also be executed automatically at start  up.

Using  this method, you can have a separate TNCINIT  script  file

for   each   terminal  driver  port.  Also,   the   script   file

TNCEXIT.<port>  will be automatically executed when  the  program

exits  for each port if one exists in the SCRIPT  directory.

I personally don't use the TNCINIT/TNCEXIT script files  but

you may find a use for them.

MUTNCBAY Driver V 1.02          43                        21/04/95

13 Personal Mail System

=======================

Included  in  the  TNCBAY driver program  is  an  integrated

Personal  Mail System. It includes all the normal BBS  facilities

for  message sending, YAPP/ASCII file upload and downloads,  mail

forwarding, 'Mail For' beacon, chat mode, REQDIR, REQFIL etc.

Due  to the current licensing conditions in the UK, it  also

disallows  third party mail by only accepting incoming  mail  for

the  sysop.  However, the sysop can send mail  to  anyone.  Other

countries  may  allow  third  party  mail  so  this  facility  is

configurable.

The following sections will describe how to configure, setup

and run the PMS.

13.1 Configuring the PMS

------------------------

The  configuration  for the PMS is stored in a  file  called

TNCPMS.CFG. It is a standard text file and should be edited  with

the  inbuilt  file  editor or a text editor  that  does  not  add

formatting  characters  to  the text (such as  Wordstar  in  non-

document mode).

The  first  line is your callsign and is  used  to  identify

yourself when using the Alt O command to log onto the PMS locally

and also when using the mail snatch facility. This callsign  must

be  the  same as the one in your registered  name  otherwise  the

software will revert to running in evaluation copy mode with  the

on-air PMS disabled.

eg.                 G7JJF

The  second line is the name you wish to give to  your  PMS.

This  is displayed on the welcome screen when a user connects  to

your PMS.

eg.        Jon's Multiuser Personal Mail System

The  following lines identify the file upload  and  download

areas  you  wish to set up. Each line comprises of  five  fields,

each separated by a comma.

Field  1  - File Area Designator. This is a  four  character

string  which uniquely identifies the area. The order you  define

the  file  areas is the order they are display to  the  user.  In

keeping  with  the standard for identifying file areas,  I  would

recommend you use numbers from 1-9999.

MUTNCBAY Driver V 1.02          44                        21/04/95

Field 2 - File Area Directory. This is the complete pathname

(including  drive if necessary) of the directory  containing  the

files for this area.

Field 3 - File Area Description. This text string is used on

the  W command to tell the user what type of files are  contained

in the file area.

Field  4  - Download Flag. This can be either a Y or  N  and

tells the system if file downloads are allowed from this area.

Field 5 - Upload Flag. This can be either a Y or N and tells

the system if file uploads are allowed to this area.

eg.  1,C:\TNC\PMS\DOWNLOAD,Download Area,Y,N

You must terminate the list of file areas with a single line

containing a '*'.

The  next  three  lines in the configuration  file  are  the

prompt lines sent out for each of the three user levels : normal,

expert  and sysop. They can contain any text you wish as well  as

certain  pieces  of  system information. The  following  list  of

macros will include the corresponding piece of system information

at the relevant point.

$A - Number of active PMS messages

$C - Callsign of user

$D - System date

$H - Home BBS of user

$M - Next PMS message number

$N - Name of user

$T - System time

$U - Number of PMS users

eg.       Cmd:B,C,D,E,G,H,I,K#,KM,L,LM,R#,RM,S,U,V,W,X >

Cmd: >

Ok $N, wot now ?

The first line is for normal users and displays all possible

commands.

The  second  line  is  for  expert  users  and  displays  an

abbreviated prompt line.

The last line is for Sysops and display a more user friendly

prompt.

The  next  line  in the configuration file  is  your  normal

beacon  text setting. This is needed so that a 'Mail For'  beacon

can be appended to it and used as your current beacon text.

The  next  line  in  the configuration file  is  a  Y  or  N

character  which decides if the PMS accepts third party  mail  or

not. For normal UK operation this MUST be set to N.

MUTNCBAY Driver V 1.02          45                        21/04/95

The  next  line  in  the  configuration  file  is  a  string

identifying  your QTH. It is used in the routing  information  on

forwarded  mail and can be anything you like really but limit  it

in length to about 20 characters. You must make sure though  that

the total length of the routing information is not longer than 80

characters  which  can be checked by monitoring  your  forwarding

sessions.

eg.       [Sutton In Ashfield]

The next line contains a single Y or N character and informs

the software whether you wish it to generate the TNCPMS.LOG file.

For  people using only floppy discs instead of hard  discs,  disc

space  may  be  short so you may want to  disable  the  log  file

generation by putting a N here.

The  next  line is a number telling the  software  the  size

(in K)  of  the  largest file which a user may  REQFIL.  I  would

recommend a value of 40 here but it can be anything you want.

The  next line is a number telling the software the  maximum

size (in K) of a message which can be sent to a user in  response

to  a REQFIL request. If the requested file is larger  than  this

figure, the file will be split over several messages, all of this

size.  The  actual message size will be only  approximately  this

figure due to the extra information the uucode and 7plus encoding

routines add to the message. I would recommend a value of 4  here

but it can be anything you want.

The  final line is the time interval in minutes for  sending

out the 'Mail For' beacon. Putting a value of 0 will disable  the

beacon altogether.

A sample TNCPMS.CFG file is thus :

G7JJF

Jon's Multiuser Personal Mail System

100,C:\TNC\PMS\DOWNLOAD\GENERAL,General Download Area,Y,N

101,C:\TNC\PMS\UPLOAD,Upload Area,N,Y

102,C:\TNC\PMS\DOWNLOAD\TNC,TNC Driver Support,Y,N

*

Cmd:B,C,D,E,H,I,K#,KM,L,LM,R#,RM,S,U,V,W,X >

Cmd: >

Ok $N, wot now ?

G7JJF (Jon) S-In-A, Notts. ** KAM 10 Port Multiuser PMS Ver 1.02 on Test **

Y

[Sutton In Ashfield]

Y

40

4

15

MUTNCBAY Driver V 1.02          46                        21/04/95

13.2 Setting up the PMS

-----------------------

There  is very little to do in setting up the Personal  Mail

System.

If  you have not done so already, create the  PMS  directory

structure  as  shown  in  section  2  above  and  configure   the

TNCPMS.CFG file accordingly.

You   may  also  want  to  customise  the  main  help   file

TNCPMS.HLP,  the ? help files HELP\TNCPMS.xxx and the  info  file

TNCPMS.INF

You  can create an optional 'Message of the Day' file.  This

is a plain ASCII text file which is dumped to the user when  they

log  on. This must be called TNCPMS.DAY and is stored in the  PMS

directory.  You can alter the contents of this file as  often  as

you wish.

For  all  your file areas, you can also create  an  optional

'area information' file. Again, this is a plain ASCII text  file,

must  be called <area>.INF where <area> is the name given to  the

area in the TNCPMS.CFG file and stored in the directory AREAINFO.

When the user issues a 'W <area>' command, the <area>.INF file is

also displayed below the header line and before the list of  file

names.  This feature can be used to tell the user,  for  example,

that  all  files  in  the  area  are  .ZIP  files  and  must   be

decompressed with PKUNZIP before use.

If you want to bar anyone from your PMS, you may create  the

file  TNCPMS.BAD  which  is an ASCII  text  file  containing  one

callsign  per  line which cannot access the PMS. You  can  either

specify  a  full callsign or part of a callsign. If  you  specify

only  a part of it, any callsign trying to access the  PMS  which

starts with the callsign you specified will be denied access.

The  PMS  can also operate as a closed system  whereby  only

callsigns  you  specify  are allowed access to the  PMS.  If  you

create  an  ASCII  text  file  called  TNCPMS.OK  containing  one

callsign  per line, only those callsign may access the  PMS.  You

can either specify a full callsign or part of a callsign. If  you

specify  only a part of it, only callsigns which starts with  the

callsign you specified will be allowed access.

If  you  require automatic mail forwarding,  you  must  also

create  the mail forwarding script files as discussed  under  the

script  file section above and amend the TNC.TIM file to run  the

forwarding files at the required times.

If  you want other users to be able to reverse forward  mail

you  may  need to create a forward file list for  them.  This  is

simply a file containing a list of subject matters (one per line)

which  interest the user. These are then called '<call>.lst'  and

are stored in the TNC\PMS\FORWARD directory.

MUTNCBAY Driver V 1.02          47                        21/04/95

eg.  if G7JJF wanted to be able to reverse forward  messages

relating  to  DANPAC, TCPIP, IBM, you would then  create  a  file

called G7JJF.LST which contained :

DANPAC

TCPIP

IBM

Any  message that was addressed either to or at any  of  the

above  subjects  would then be considered for forwarding  if  the

particular message had not been read or forwarded already.

If  no forward list file exists for a particular  user  when

they try to reverse forward mail, they will only receive messages

addressed to their callsign.

The final operation to perform is to log on locally for  the

first time and identify yourself as the sysop of the PMS.

13.3 Logging Onto The PMS

-------------------------

The PMS can be accessed in one of two ways.

The  first is by using the Alt O function to log on  locally

as the sysop.

The second is for other users to log on. This is done by the

remote user connected to your callsign as though wanting a  chat.

If you are not about, the user can then type 'PMS' (or 'BBS')  to

enter the mail system. You may wish to alter your TNCPMS.CT  file

(which  contains  your CTEXT information) to inform the  user  of

this, eg.

If I do not reply within 30 seconds, please type PMS

to enter my Personal Mail System.

Because  you may want a small initial chat to tell the  user

about  your new PMS, the PMS will be accessed if the remote  user

types  PMS  on a blank line within the first five lines  that  he

types.

13.4 Using The PMS

------------------

After  the user logs on to the PMS, one of three things  may

happen.

1. If the users callsign is one contained in the  TNCPMS.BAD

file,  he will be told that he cannot access the PMS and will  be

logged  off.  If the TNCPMS.OK file is being used and  the  users

callsign  is not contained in it, he will be told that he  cannot

access the PMS and will be logged off.

MUTNCBAY Driver V 1.02          48                        21/04/95

2.  If the user has been excluded from the PMS with  the  EU

<call>  E  command described below, he will be told that  he  has

been excluded from the PMS and will be logged off.

3. The user will be welcomed to the PMS !

If it is the first time the user has called the PMS, he will

be asked to enter his name (it must be at least three  characters

long).

The  user will then be shown some systems statistics  before

being given the command prompt.

All  the  time the remote user is logged onto the  PMS,  the

sysop  still has control over the keyboard as well when  switched

to the port the user has connected on. This means that the  sysop

can  enter  commands as though they were coming from  the  remote

user.  This  can be used to guide new users through  the  PMS  by

showing  them  what commands do what, or the remote user  can  be

logged off if he mis-uses the PMS etc.

Whilst  the  user  is in the PMS, you  will  see  on  screen

exactly  what the remote user is seeing. The PMS window  operates

in  split  screen mode so the users commands will appear  in  the

bottom window with the PMS output in the top window.

A user can be given one of three different levels.

A  normal  user  and  expert  user  have  the  same  command

privileges  but  the  expert user can have  a  different  command

prompt as configured by the sysop. The expert/normal user  status

can be altered by the user.

The third level is the sysop level. The sysop has access  to

more  commands used for maintaining the PMS and also a  different

command prompt (if configured).

MUTNCBAY Driver V 1.02          49                        21/04/95

13.5 Commands In The PMS

------------------------

@ - Enter sysop mode. (sysop only command)

If a user has been configured as a sysop, he may enter sysop

mode with this command. Please note that a sysop logging into the

system  remotely  (including you !) does not gain access  to  the

sysop  facilities  until he uses the @ command. This is  to  stop

people  borrowing  your own callsign and wrecking havoc  on  your

system.

After  entering the @ command, the user will get back  three

lines of numbers from which he must choose one line and use it to

decode  the password. The password string he uses is the one  you

generated for him using the EU <call> P command described  below.

To  enter  the  password, for each of the eight  numbers  in  the

chosen  line,  the  users  counts up to  that  character  in  the

password  string  (starting at 0 being the first  character)  and

enters it. After typing all 8 characters and pressing return,  if

the  user got the password correct he will enter sysop  mode.  If

the user got the password wrong, he will stay as a normal user on

the PMS.

*DOS Commands (sysop only commands)

-----------------------------------

*DIR (directory)(filespec) - Display file directory

This  command  will  display the files  in  the  current  or

specified directory which match the optional filespec (defaulting

to *.*)

*TIME (HH:MM:SS) - Set or display the computers time

With  no  parameter, this command will display  the  current

system time of the computer.

Specifying  a  time  will set the  computers  time  to  that

required.

*DATE (DD-MM-YY) - Set or display the computers date

With  no  parameter, this command will display  the  current

system date of the computer.

Specifying  a  date  will set the  computers  date  to  that

required.

The format of the displayed date and the format in which you

must  enter a new date to change it will depend on  your  country

and should default to your normal method of entering dates.

MUTNCBAY Driver V 1.02          50                        21/04/95

*CD (directory) - Set or change the current drive/directory

With  no  parameter, this command will display  the  current

logged on drive and directory of the computer.

Specifying  a  directory (with optional drive  specifier  as

well) will set the computers current directory to that required.

*MD <directory> - Create a new directory

This  command  will create the specified  directory  on  the

computer if it can.

*RD <directory> - Remove a directory

This  command  will delete the specified  directory  on  the

computer if it can.

*TYPE <filename> - Display a file on-screen

This  command  will display the contents  of  the  specified

ASCII  file  on  screen. Remember that you can still  use  the  A

(abort listing) command described below to stop the screen output

if you wish.

*DEL <filename> - Delete a file

This command will delete the specified file on the  computer

if it can. Please note that you cannot use wildcard here and only

single files can be deleted at a time.

*REN <oldfile> <newfile> - Rename a file

This command will rename the specified file on the  computer

to the new filename given. With this command you can also  rename

the file to a different directory on the same drive but not  onto

another drive. Please note that you cannot use wildcard here  and

only single files can be renamed at a time.

*COPY <oldfile> <newfile> - Copy a file

This command will copy the specified file on the computer to

the new filename given. Please note that you cannot use  wildcard

here and only single files can be copied at a time.

*HELP - Display DOS command help summary.

This will display a summary of the above commands on  screen

and give you the syntax for each of them for reference.

A - Abort.

The  current  listing being received from the  PMS  will  be

aborted and the user will be returned to the command prompt.

MUTNCBAY Driver V 1.02          51                        21/04/95

B - Log off the PMS.

Disconnecting has the same effect.

The 'Mail For' beacon will also be updated.

C - Chat to the sysop if he is available.

(Command not available in local mode)

If  the  'Sysop Available' toggle is active  (Alt  F9),  the

sysop will be paged by the bell ringing once a second for  thirty

seconds. If the sysop is around, he can switch to the correct PMS

port and start typing at the keyboard to chat to the remote user.

If  the  sysop does not reply within the thirty seconds,  or  the

'Sysop  Available'  toggle is inactive, the remote user  will  be

told  the  sysop  is  unavailable  and  he  will  be  given   the

opportunity of sending him a message instead. Either the user  or

the sysop can press Ctrl Z when finished with the chat to  return

to the PMS.

CM <message num> <callsign> - Copy message.

This  is  used  to copy a message to  another  user  without

retyping it in. Sysops can copy a message to anyone but users can

only copy messages to the sysop. The user will be asked for a new

subject but the content of the message will remain unaltered.

eg. CM 5 G7JJF

DA <area> <file> - Download file.

This  is used to download a file using ASCII  transfer.  The

area can be up to a four character string as defined by the sysop

in the TNCPMS.CFG file.

eg. DA 1 HELP.TXT

DS - Display PMS stats.

This will display several stats about the PMS, eg number  of

users, active messages who is currently using the PMS, what  port

they  are  connected  on and what operation  they  are  currently

performing etc.

DU - Display all user stats.

This will display stats about all the users of the PMS,  eg.

callsign, name, homebbs, number of times called etc.

DU <callsign> - Display user stats.

This will display stats about an individual PMS user in  the

same format as the DU command above.

eg. DU G7JJF

MUTNCBAY Driver V 1.02          52                        21/04/95

DY <area> <file> - Download file.

(Command not available in local mode)

This  is  used to download a file using YAPP  transfer.  The

area can be up to a four character string as defined by the sysop

in the TNCPMS.CFG file.

eg. DY 1 HELP.COM

EC - Toggle colour status

This  is  used  to  toggle the  status  of  a  user  between

accepting colour from the pms or not. At the moment, the pms does

not  operate  in  colour  but  future  versions  may  add  colour

capability.

eg. EC

EH <homebbs> - Edit home bbs.

This will change the home bbs of the current PMS user.

eg. EH GB7BAD.#23.GBR.EU

EN <name> (H <homebbs>) - Change user info.

This  will change the current PMS users name and  optionally

his home bbs.

eg. EN Jon              or

EN Jon H GB7BAD.#23.GBR.EU

EM is for sysops only

---------------------

EM <num> U - Make unread

This will make the status of the message unread, ie. it will

appear on a RM command, etc.

eg. EM 1 U

EM <num> R - Make read.

This will make the status of the message read, ie. the  same

as reading the message.

eg. EM 1 R

MUTNCBAY Driver V 1.02          53                        21/04/95

EM <num> F - Make forwarded.

This  will  make  the status of the  message  to  have  been

forwarded, ie. it will not be COLLECTed in the forwarding  script

file.

eg. EM 1 F

EM <num> N - Make not forwarded.

This  will make the status of the message not to  have  been

forwarded, ie. it will be COLLECTed in a forwarding script file.

eg. EM 1 N

EM <num> A - Make alive.

This will reverse the affect of the kill message command.

eg. EM 1 A

EM <num> K - Make killed.

This is the same as the kill message command.

eg. EM 1 K

EM <num> P - Toggle private/public status of message

If  the  message  was originally a private  one,  this  will

toggle to make it accessible to the public and vice versa.

eg. EM 1 P

EM <num> S - Set next message number

This  command  sets the next message number to the  one  you

specify.  You cannot set it below a message number  that  already

exists in the pms unless the pms is empty.

eg. EM 100 S

EU is for sysops only

---------------------

EU <call> N <name> (H <homebbs>) - Create/Edit user

This  is used to create a new user or edit the details of  a

current  PMS user. You can specify his name and  optionally,  his

home bbs.

eg. EU G7JJF N Jon                      or

EU G7JJF N Jon H GB7BAD.#23.GBR.EU

MUTNCBAY Driver V 1.02          54                        21/04/95

EU <call> H <homebbs> - Create/edit user.

This  is used to create a new user or edit the details of  a

current PMS user. You need to specify his home bbs.

eg. EU G7JJF H GB7BAD.#23.GBR.EU

EU <call> D - Delete user.

This  is  used  to  delete a PMS  user.  You  cannot  delete

yourself (ie the first user in the PMS system)

eg. EU G7JJF D

EU <call> B - Toggle BBS status

This is used to toggle the status of a user between being  a

BBS and not being a BBS. This is used to allow a BBS to enter the

PMS  from  a  connect without typing the  PMS  string  and  going

straight  into  the mail transfer routines. See  below  for  more

information on BBS's and mail transfer.

eg. EU GB7BAD B

EU <call> S - Toggle sysop status

This  is used to toggle the status of a user  between  sysop

level  and non sysop level. Use with caution since  anything  you

can do, a remote sysop user can also do.

eg. EU G7JJF S

EU <call> X - Toggle expert status

This  is used to toggle the status of a user between  expert

user and normal user. At the moment, the only difference  between

an  expert  user  and a normal user is that  they  get  different

command prompts as defined in the TNCPMS.CFG file.

eg. EU G7JJF X

EU <call> P - Generate Password

This  is used to generate a password for a user in order  to

enter sysop mode. The password is unique to each user and is sent

to  a disc file called <callsign>.pwd in your PMS directory.  You

should then make the user aware of this password then delete  the

password  file  since it is no longer needed. The  same  password

will be generated for each user every time this command is used.

eg. EU G7JJF P

MUTNCBAY Driver V 1.02          55                        21/04/95

EU <call> C - Toggle colour status

This  is  used  to toggle the status  of  the  user  between

accepting colour from the PMS or not. At the moment, the PMS does

not  operate  in  colour  but  future  versions  may  add  colour

capability.

eg. EC G7JJF C

EU <call> E - Toggle exclude status

This  is used to temporarily exclude a user from the PMS  if

he misbehaves for example. The user record is still maintained on

the  PMS and you can reinstate the user by issuing  this  command

again. You cannot exclude yourself with this command.

eg. EU G7JJF E

F(V) <num> <area> <filename> - Convert message to a file.

This is used to convert a mail message to a named file in  a

particular  file  area. The area can be up to  a  four  character

string  as  defined  by the sysop in the  TNCPMS.CFG  file.  This

command  is  a  sysop only command as well.  If  the  optional  V

parameter  is added as well, the message routing  information  is

stored in the file as well.

eg. F 4 123 GAME.BAS

H - Display help file.

This  command  will display the PMS\TNCPMS.HLP file  if  one

exists.

? <command> - Display help on <command>.

This command will display the PMS\HELP\TNCPMS.<command> file

if one exists.

eg. ? LM

I - Display info file.

This  command  will display the PMS\TNCPMS.INF file  if  one

exists.   Use   it  to  show  your   stations   hardware/software

configuration and anything else you may wish to tell the user.

J - Show MH list.

This  command will display a list of the last  20  callsigns

and the date and time they were heard.

MUTNCBAY Driver V 1.02          56                        21/04/95

K <num> - Kills message.

This  will kill a message if it exists. Users can only  kill

message either sent to them or sent from them.

eg. K 327  Kills message number 327.

KM - Kill my mail.

This will kill all mail addressed to you that you have read.

K 0 - kill all message base (sysops only).

This  command  will delete all the messages in  the  system.

They cannot be recovered after being deleted.

L ( <num> <num> ) - Lists messages.

This will list all messages in reverse order, ie. newest  to

oldest which you can read. If two message numbers are  optionally

specified,  only  those messages between the two limits  will  be

listed.

LL <num> - Lists last messages.

This will list the last <num> messages in reverse order, ie.

newest to oldest which you can read.

LM - List Mine.

This will list all messages addressed to you.

LK - List killed. (sysops only).

This   will  list  all  messages  that  have  been   killed.

Individual  messages  can be recovered by using the  EM  <num>  A

command.

L< <callsign> - List messages from user.

This will list all messages that are addressed from the user

you specify which you can read.

L> <callsign> - List messages to user.

This  will list all messages that are addressed to the  user

you specify which you can read.

R <num> - Read message.

This command will read a message.

eg. R 325 reads message 325

MUTNCBAY Driver V 1.02          57                        21/04/95

RK <num> - Read killed message (sysops only).

This command will read a killed message.

eg. RK 325 reads killed message 325

RM - Read Mine.

This command will read all your unread messages.

RA - Read all messages.

This command will read all messages that you are allowed  to

read.

NOTE  -  In  any of the above four read commands,  if  the  R  is

replaced with a V, the equivalent read command is performed but a

verbose  listing of the message routing information is  displayed

as well.

S <call> - Same as SP <call>

SP <call> - Send personal message to station <call>.

This is used to send a personal message to another user. The

PMS  will prompt for the message title and then for  the  message

text. The text entry is ended with a ctrl-Z or /EX. Only the user

who sent the message or the recipient of the message can read  it

(as well as Sysops of course).

eg. SP G7JJF

SB <call> - Send bulletin to station <call>.

This  is  used to send a bulletin to another user.  The  PMS

will prompt for the message title and then for the message  text.

The  text entry is ended with a ctrl-Z or /EX. Normal  users  can

only  mail  the sysop but a sysop can mail to  anyone  (including

ALL) if the third party mail option is disabled otherwise  anyone

can send mail to anyone. Anyone can read a bulletin message.

eg. SB ALL

SR - Send reply to last message you read.

This  will  send a message to the user who  wrote  the  last

message  you read. It will automatically address the  message  to

the user @ his home bbs as found in the user database and  retain

the same subject title as before. You then enter the message text

as described under the S command above.

eg. SR

MUTNCBAY Driver V 1.02          58                        21/04/95

SPR - Send personal reply to last message you read.

This will send a personal message to the user who wrote  the

last message you read. It will automatically address the  message

to  the  user @ his home bbs as found in the  user  database  and

retain  the  same  subject title as before. You  then  enter  the

message text as described under the S command above.

eg. SPR

SM <port> <msg> - Send on-line message.

This  command allows a logged on user to send a single  line

message to another user who is currently logged into the PMS. The

<port> is the port number of the other logged on user as shown in

the  DS  command and <msg> is the message he wishes  to  send.  A

sysop  can also specify ALL for <port> and will send the  message

to all logged on users.

eg. SM 1 Hi Ian

would  send a message to the user logged onto port 1 and  display

on his screen the following next time the command prompt was also

displayed :

From Jon (G7JJF) Port 6 : HI IAN

SD <call> - Send directory listing to <call>. (Sysop only)

This  is  used to manually reply to a  REQDIR  request  from

another  user. The PMS will prompt for the directory  area  (with

optional  filespec)  which  the user wants  information  on.  The

resulting personal message(s), in the same format as the W <area>

(filespec)  command, will then be placed in the PMS ready  to  be

forwarded out .

eg. SD G7JJF @ GB7MAM

Area: 102 *.doc

will send a message listing all .DOC files in file area  102

to  G7JJF  @  GB7MAM. The resulting listing will  be  split  into

several messages if there are a lot of files.

If  you  just  RETURN at the Area  prompt  without  entering

anything,  the user will be sent a message listing all your  file

areas instead in the same format as the W command.

SF <call> - Send file to <call>. (Sysop only)

This  is  used to manually reply to a  REQFIL  request  from

another  user.  The PMS will prompt for the area and  file  (with

optional  encoding  flag)  which  the  user  wants  sending.  The

resulting  personal  message(s) will then be placed  in  the  PMS

ready to be forwarded out.

MUTNCBAY Driver V 1.02          59                        21/04/95

eg. SF G7JJF @ GB7MAM

File: 102 ANALYSE.ZIP 7

will send message(s) containing the file ANALYSE.ZIP encoded

using  7PLUS.  The resulting message(s) will be split  into  file

sections as configured in the TNCPMS.CFG file.

See  the  section below for more information on  REQDIR  and

REQFIL.

T - Tidy message base (sysops only).

This  will remove all killed messages from the system.  They

cannot be recovered after being deleted. If you have created  the

OLDMAIL  directory  under  the  PMS\MAIL  directory,  the  killed

message will be archived here.

UA <area> <file> - Upload file.

This is used to upload a file using ASCII transfer. The area

can  be up to a four character string as defined by the sysop  in

the TNCPMS.CFG file.

eg. UA 1 MYFILE.TXT

UY <area> <file> - Upload file. (Not available in local mode)

This is used to upload a file using YAPP transfer. The  area

can  be upto a four character string as defined by the  sysop  in

the TNCPMS.CFG file.

eg. UY A MYPROG.EXE

V - Show version.

This will show what version of the PMS is running.

W - List file areas.

This will list the file areas as configured by the sysop  in

the  TNCPMS.CFG  file. Each area will indicate  whether  you  can

download and upload to it.

W <area> (filespec) - List files (matching filespec).

This  will list all the files matching the  optional  filespec

(defaulting to all files) contained in that particular area.  The

area can be upto a four character string as defined by the  sysop

in the TNCPMS.CFG file.

eg. W 1 *.ZIP

MUTNCBAY Driver V 1.02          60                        21/04/95

X - Toggle expert level.

This will toggle the level of the user between expert  level

and  normal  level. Expert level normally gives  shorter  command

prompts whereas normal level gives full command prompts.

13.6 Maintaining the PMS

------------------------

How  you maintain your PMS is really up to you. I  can  only

give you common sense advice which may or may not seem obvious to

you.

It  is advisable to keep your message base tidy by  removing

killed  messages using the T - tidy command. Also, kill and  tidy

any messages which have been forwarded or read.

Also,  check  any  uploads made by users  for  viruses  etc,

before  putting  them into a download area  for  general  access.

Check  that there is no breach of copyright on the program  being

offered for download.

Analyse the log file TNCPMS.LOG to see what users have  been

doing  in  terms  of uploading/downloading etc. to  see  what  is

popular and what is not. If you are feeling adventurous, you  may

want  to write programs which will automatically analyse the  log

file  and  perhaps send thank you messages  to  people  uploading

files,  and  warnings  to  people who are  hogging  the  PMS  for

downloads  etc. You could also write programs to produce top  ten

lists  of PMS users or files etc. The log file may be deleted  at

any time but you may wish to archive it off for later analysis.

If  you  wish  to  totally start the  PMS  from  scratch  by

deleting users and messages, you must delete all three files :

TNCPMS.SYS, TNCPMS.USR and TNCPMS.IDX

and everything from the MAIL directory.

None  of the above three files are user editable  so  please

don't  try and modify them as you will most probably destroy  the

system information contained in them.

Finally,  PLEASE don't reboot your computer  whilst  running

the  program or you will probably corrupt the PMS data files.  If

you  find  the PMS asking for your name again  after  logging  in

before,  the  files have been zapped. To recover from  this,  you

will  need  to  restart  the PMS  again  by  deleting  the  files

mentioned above.

MUTNCBAY Driver V 1.02          61                        21/04/95

13.7 BBS access to PMS

----------------------

The  PMS  has  the ability to  perform  reverse  and  normal

forwarding between itself and other BBS's (including other  users

of this program).

In  order to perform the mail transfer, the other  user  (or

BBS)  has to enter the PMS. Depending on the status of the  user,

the  connection  can take place in one of two ways.  (The  status

being a BBS or not as defined with the EU <call> B command).

If  the  status  of the user is a  BBS,  after  the  initial

connect, if the user sends it's BBS id string, eg.  [NNA-1.09-$],

it will automatically enter the mail transfer routines.

If the characters BF appear in the BBS id string, the system

will enter FBB compressed forwarding mode and mail transfers will

progress  automatically.  The  mechanics of  FBB  forwarding  are

beyond the scope of this manual (good get-out clause that !)  and

any interested parties are advised to consult the manual for  the

FBB program for more information.

If the user is not a BBS, after the initial connect and  log

on sequence, he must send his BBS id string, eg.  [BAY-1.02-HM$].

The  prompt  will then change to a > and we are  ready  for  mail

transfer.

To  reverse forward mail, the user types 'F>'.  The  program

then finds the first message in the PMS that the user has not yet

received. It will send out a header line identifying the message.

The  user will then send either 'OK' if it wants to  receive  the

message or 'N-BID' if not. If the user wants it, the program will

send  the message and mark it in the PMS as being forwarded.  The

user  then keeps issuing a 'F>' until there are no more  messages

to  be  received  at which the point  the  program  replies  with

'*** DONE'.

To forward mail, the user sends the header line  identifying

the  message and the PMS replies 'OK' or 'N-BID' accordingly.  If

OK, the user then sends the message etc and repeats the above for

each message that needs to be sent.

Once all mail has been transferred, the user then types  the

'B' command to log off the PMS.

NOTE : Once inside the mail transfer routines, all other PMS

commands are disabled except for 'B'.

MUTNCBAY Driver V 1.02          62                        21/04/95

This procedure for reverse/forwarding mail is  automatically

handled  by  the use of the REVERSE and FORWARD BBS  keywords  if

transferring  between  two copies of this program.  Normal  BBS's

should also have this procedure built into their software.

An example script files to transfer mail between two  copies

of this software is thus :

CMD                      } Enter command mode

REPLY C G7JJF^M          } Issue connect

WHEN my PMS              } When get CTEXT

REPLY PMS^M              } Enter PMS

WHEN now ?               } When get PMS prompt

REPLY [BAY-1.02-HM$]^M   } Send ID

WHEN >                   } When get prompt

REVERSE NONE             } Collect all my mail

COLLECT NONE1,G7JJF      } Any for Jon ?

FORWARD BBS              } Send

:NONE1

REPLY B^M                } Bye when finished

:NONE

DISPLAY Finished !!^M

or if you want to perform compressed mail forwarding between  two

copies of this software, you can use :

CMD

REPLY C G7JJF^M          } Issue connect

WHEN my PMS              } When get CTEXT

REPLY PMS^M              } Enter PMS

WHEN now ?               } When get PMS prompt

REPLY [BAY-1.02-BFHM$]^M } Send ID

COLLECT NONE,G7JJF       } Any for Jon ?

:NONE

FORWARD FBB              } Send and collect mail

MUTNCBAY Driver V 1.02          63                        21/04/95

13.8 REQDIR and REQFIL

----------------------

The PMS supports REQDIR and REQFIL requests from users  with

optional automatic encoding of binary REQFIL requests into UUcode

or 7PLUS code format.

To  activate  the REQDIR or REQFIL requests,  the  PMS  must

receive  a  message  addressed  to  you  with  the  subject  line

detailing  the specific request. The message content is  ignored.

The  message  can enter the PMS either through a  direct  connect

using  the  SP command or from a message been forwarded  in  from

another BBS.

REQDIR - Request a directory listing

------

To  request a directory listing, the subject must be of  the

format :

REQDIR <area> (filespec) @ <users homebbs>

where  specifying <area> will list all files  (matching  optional

filespec)  in that particular area or leaving <area>  blank  will

give a list of available file areas.

eg.

REQDIR @ GB7MAM

will put a message into your PMS, addressed to the users callsign

at his homebbs of GB7MAM, listing all available file areas.

REQDIR 102 *.DOC @ GB7MAM

will put a message into your PMS, addressed to the users callsign

at  his homebbs of GB7MAM, listing all DOC files in area 102.  If

there  are a lot of files in a particular area, the user will  be

sent multiple messages, each one containing part of the  listing.

This is to keep the individual message size down to a  reasonable

level.

REQFIL - Request a file

------

To  request  a particular file, the subject must be  of  the

format :

REQFIL <area> <filename> (U)|(7) @ <users homebbs>

where  <area> is a valid area, <filename> is a file in that  area

and  U  or  7  tells the program to send  the  file  back  either

UUencoded  or  7PLUS encoded. If a U or 7 is not  specified,  the

file is sent as ASCII.

MUTNCBAY Driver V 1.02          64                        21/04/95

eg.

REQFIL 102 ANALYSE.ZIP 7 @ GB7MAM

will  put  message(s)  into  your PMS,  addressed  to  the  users

callsign  at his homebbs of GB7MAM, containing the 7PLUS  encoded

file ANALYSE.ZIP stored in area 102.

REQFIL 102 INDEX.TXT @ GB7MAM

will put a message into your PMS, addressed to the users callsign

at  his  homebbs of GB7MAM, containing the ASCII  file  INDEX.TXT

stored in area 102.

You  can place a limit on the size of the files  transferred

and define the size of each encoded file section as described  in

the TNCPMS.CFG section above.

After the messages have been put in your PMS, they should be

forwarded to the user via your local BBS or collected by the user

if they can connect direct.

You can manually perform a REQDIR or REQFIL operation in the

PMS  using  the SD and SF commands. This is useful  is  the  user

sends you a message asking for a file or directory listing rather

than using the 'official' way of doing it.

MUTNCBAY Driver V 1.02          65                        21/04/95

14 Online Cross Reference Help Facility

---------------------------------------

The  program  supports  an on  line  cross  referenced  help

facility  which  the  user can use for  whatever  purpose  he  so

desires.  Typical examples of use would be to create a help  file

of  TNC  commands  and  their usage  or  a  file  containing  BBS

callsigns,  sysop's details, their location, software  they  use,

etc.

The help system has been made compatible with the help files

produced for use with the PAKET4 program so if you have a copy of

that  as  well, you can plug in the existing help  files  without

resorting to creating your own.

The  idea  of  the  help system is to  create  a  text  file

containing   a  subject  line  followed  by  a  short   narrative

describing that particular subject and repeating this process for

however  much information you want to include in the  help  file.

Upon  calling  the help system, the program displays  a  list  of

subject  headers  and the user can scroll through  this  list  to

select  a subject of interest. Once found, the program will  then

display  the  narrative  associated  with  the  subject.  If  the

narrative  also contains cross referenced keywords, you can  also

automatically see the narrative of any keyword you wish to select

rather than going through the subject index.

14.1 Creating a Help File

-------------------------

The  help  file (called TNC.HLP and stored in the  main  TNC

directory)  is  a standard ASCII text file which can  be  created

with  the inbuilt file editor or a text editor that does not  add

formatting  characters  to  the text (such as  Wordstar  in  non-

document mode). Please note though that these help files can  get

very  large  and there is a 64K file size limit to  the  internal

editor.  If  you find your help files are nearing this  limit,  I

would suggest that you try an alternative editor which can handle

larger  files.  There are several such editors available  in  the

shareware  market and perhaps one of the more popular ones  is  a

program  called QEDIT which can be found on most  bulletin  board

systems.

Throughout the following discussion, it may be advisable  to

read it whilst the sample help file included with the  evaluation

copy is on screen. You can use the command 'ED TNC.HLP' whilst in

the TNC directory to view/edit the file.

The  first  two lines of the help file are  reserved  for  a

footer  and  header to the help menu box. They can  each  contain

whatever information you require and are usually used to identify

the version of the help file in use and also as a title line  for

the subject headers.

MUTNCBAY Driver V 1.02          66                        21/04/95

Below  this  is  the first subject line  (ignore  the  lines

starting  with  '*'s.  These only act as comment  lines  and  are

ignored by the program). Each subject line is identified with the

first  character  being the '@' symbol. The text  following  this

character is then used as the subject line and is displayed in on

help  subject index screen. The format of the line is  free  text

but make sure any columns of information line up with the  header

line from the top of the file if required.

Below  the  subject line, you can specify up to  20  further

lines  which  are used as narrative to  describe  the  particular

subject.  Again, this is format is free but make sure  the  lines

are no longer that 76 characters.

To  include keywords in the narrative, enclose the  relevant

text in [ and ] characters. The text must match exactly with  the

start  of  a subject line in order for the keyword search  to  be

effective. See the example of the Function Keys narrative to  see

how this is achieved.

After specifying what narrative you require for the subject,

you can then repeat the sequence with another subject etc.  until

your help file contains all the information you require.

14.2 Using the Help System

--------------------------

To access the help system, simply press the Ctrl-Home key to

bring up the subject index. The first time you access the system,

there will be a short delay whilst the program builds an internal

index  into  the help file. Subsequent accesses during  the  same

program run however, will be instantaneous.

You can use the cursor keys to scroll through the help index

until   you   find  a  subject  you   require   information   on.

Alternatively,  if  you  start  typing  text,  the  cursor   will

automatically jump to the subject line which starts with the text

you  specify. As you continue adding more characters, the  search

will be refined until you find the subject you require.

Once you have found a subject of interest, press the  return

key to bring up the narrative about the subject. If the narrative

contains  keywords, you can use the cursor keys to select one  of

them. You can then press the return key to automatically take you

to  the  narrative  screen for that  particular  keyword  without

having to go through the index again.

To return to the subject index screen, press the escape key.

You  can  then  select other subjects of interest  or  press  the

escape key again to return to the program.

MUTNCBAY Driver V 1.02          67                        21/04/95

15 FBB Header Broadcast Facility

================================

The  FBB-BBS  software has a facility  for  sending  unproto

lists  of  messages  headers which are  broadcast  every  time  a

message is received by the BBS.

The  TNC driver software monitors for these  broadcasts  and

builds up a list of message headers in a disk file. You can  then

pull  up this list and navigate it using the cursor keys  marking

messages  of interest which the software can  then  automatically

download from the BBS and place in your PMS.

The broadcasts are in the form :

fm GB7MAM to FBB ...

2089   B   3233 PROPAG@EU     OK1HH  950421 FORECAST ONDREJOV

2090   B   3265 C64   @EU     PA2NJC 950421 to: disk maker

etc..

Every time the software monitors a packet addressed to  FBB,

it  checks to see if the headers have already been  recorded  (by

checking  if  the message number of the last header  received  is

below that of the current header) and if not, it appends them  to

a  disc file called GB7MAM.FBB stored in the LOG directory.  Each

BBS  broadcasting  in  this way will have its  own  log  file  of

course.

The  program also maintains an associated index file  called

GB7MAM.IDX (also stored in the LOG directory) which contains  the

message number of the last header received and the message number

of  the  last  header  scanned with  the  FBBCOLLECT  command  or

downloaded  with FBBREAD/interactively, whichever occurred  last.

This  second  message number is therefore used  to  identify  new

messages  which  have  been  captured since  the  last  scan  was

performed.

Please  note that both these files are not editable. If  you

try and edit them, corruption will possibly occur. After a  time,

the  .FBB file will grow quite large after several days worth  of

headers have been stored. Simply delete both .FBB and .IDX  files

to start again.

Before  browsing  the list of message headers,  if  you  are

going  to actually select messages to download, you will need  to

manually  connect to the BBS in question first. This was done  to

avoid having to write further script files to connect to the  BBS

before accessing the list. Makes life simpler thats all !

MUTNCBAY Driver V 1.02          68                        21/04/95

To pull up the list of message headers, press the Page  Down

key. You will be presented with a directory browser defaulting to

LOG\*.FBB  which will show the available files to  browse.  After

selecting  a  file  (or press ESC to  cancel),  the  screen  will

display (in reverse message number order) a list of the last  200

message  headers  received with a help bar at the bottom  of  the

screen.

Each header is displayed in the following format :

2090   B   3265 C64   @EU     PA2NJC 950421 to: disk maker

2089   B   3233 PROPAG@EU     OK1HH  950421 FORECAST ONDREJOV

etc..

You  can  scroll through the list using  the  normal  cursor

keys. To mark a message for retrieval, select it with the  cursor

keys  and  press the space bar which will show a  diamond  marker

beside  the message header. Press the space bar again  to  remove

the marker. After marking all the messages you want  downloading,

press  the  Return  key. This will  start  the  message  download

process  which  retrieves each message in turn and places  it  in

your local PMS where you can review them later. If you decide not

to  retrieve any marked messages, press Escape which will  cancel

the operation.

MUTNCBAY Driver V 1.02          69                        21/04/95

16 Trouble Shooting

===================

It  is  very unlikely that you will encounter  any  problems

when running this software. However, due to the complex nature of

writing  and  testing  multiuser/multitasking  software,  it   is

virtually  impossible to test out all combinations of  everything

that  could  possibly  happen at the same time and  so  I  cannot

guarantee  that there are no small bugs lurking somewhere in  the

code. If you do find something strange that is happening,  please

let me know about it so I can correct it if a problem exists.

There  are, however, errors than can occur due to  incorrect

configuration of the software.

When you run the program, a box may appear on screen  saying

'TFPCX Code Not Loaded'. This will be because you have  forgotten

to  load the TFPCX software before running the TNCBAY program.  I

get around this problem by using a batch file to load TFPCX first

followed  by the TNCBAY program, ie I have a file  called  GO.BAT

containing :

tfpcx -f

tncbay

D:             } change to drive and directory

CD \btnc\obj   } where your prog is stored

tfpcx -u

(Remember,  you can configure tfpcx to run from a  different  com

port  or  baud  rate  by  adding  the  -Pport[:xxx]  and   -Bnnnn

parameters to the 'tfpcx -f', eg if you want to operate 300  baud

HF packet from COM 2, change the line to read :

tfpcx -PCOM2 -B300 -f

Read the tfpcx documentation for more information )

You  may also get an 'out of memory' error message. This  is

self  explanatory  and you must reduce to  number  of  configured

tasks  in the TNC.CFG file before re-running the software. As  an

aid  to seeing how much memory you have available whilst  running

the  software, if you run the program by typing 'TNCBAY -d'  this

will  then  show  on the Alt I info screen  the  amount  of  free

memory.

You may also get an occasional 'system error' box pop up  if

you  try to log onto an empty drive in the directory browser  for

example  or turn on printer output without a  printer  connected.

This  will  give  you  to opportunity  to  Retry  or  Ignore  the

operation and by pressing R or I enough times, the error box will

disappear.

Other pop up error message may also occur but these will  be

self explanatory.

MUTNCBAY Driver V 1.02          70                        21/04/95

Several users have reported problems in getting the  program

to monitor any packet information although the system appears  to

be  transmitting  ok.  This can be cured  by  altering  the  bias

resistors  connected  to pin 7 of the TCM3105 chip  to  make  the

voltage on pin 7 2.6V +/- 0.1V. In earlier designs of the  Baycom

modem, this voltage can be altered by trimming a pot.

MUTNCBAY Driver V 1.02          71                        21/04/95

Appendix A - Summary of Function Keys

==========   ------------------------

F1  - ASCII Download           Alt F1  - Help Screen

F2  - ASCII Upload             Alt F2  - Set start block marker

F3  - Pause Download           Alt F3  - Set end block marker

F4  - YAPP Download            Alt F4  - Clear markers

F5  - YAPP Upload              Alt F5  - Save/recall screen 1

F9  - DOS Shell                Alt F6  - Save/recall screen 2

F10 - Dump screen              Alt F7  - Save/recall screen 3

Alt F8  - Save/recall screen 4

Alt F9  - Toggle Sysop Available

Alt F10 - Dump block

Alt B - Toggle connect bell    Alt O - Enter PMS locally

Alt C - Connect directory      Alt P - Toggle printer output

Alt D - Directory browser      Alt Q - TNC I/P from file

Alt E - Edit file              Alt S - Online configuration

Alt G - Perform Script         Alt T - Send DD-MON-YY  HH:MM:SS

Alt H - Display ALT help       Alt U - UUcode File

Alt I - Info Screen            Alt W - Toggle connect window

Alt J - Monitor Connects       Alt X - Exit program

Alt L - 7PLUS Code File        Alt Z - Toggle Full Screen

Alt N - Enter notepad

MUTNCBAY Driver V 1.02          72                        21/04/95

Appendix B - Summary of Editor Keys

==========   ----------------------

Key Action                 Primary Key               Secondary Key

------------------------------------------------------------------

Character left             P: <CtrlS>                S: <Lft>

Character right            P: <CtrlD>                S: <Rgt>

Word left                  P: <CtrlA>                S: <CtrlLft>

Word right                 P: <CtrlF>                S: <CtrlRgt>

Line up                    P: <CtrlE>                S: <Up>

Line down                  P: <CtrlX>                S: <Dn>

Scroll up                  P: <CtrlW>                S:

Scroll down                P: <CtrlZ>                S:

Page up                    P: <CtrlR>                S: <PgUp>

Page down                  P: <CtrlC>                S: <PgDn>

------------------------------------------------------------------

Cursor to left side        P: <CtrlQ><CtrlS>         S: <Home>

Cursor to right side       P: <CtrlQ><CtrlD>         S: <End>

Top of screen              P: <CtrlQ><CtrlE>         S: <CtrlHome>

Bottom of screen           P: <CtrlQ><CtrlX>         S: <CtrlEnd>

Top of window              P: <CtrlQ><CtrlR>         S: <CtrlPgUp>

Bottom of window           P: <CtrlQ><CtrlC>         S: <CtrlPgDn>

Top of block               P: <CtrlQ><CtrlB>         S:

Bottom of block            P: <CtrlQ><CtrlK>         S:

------------------------------------------------------------------

New line                   P: <CtrlM>                S:

Insert line                P: <CtrlN>                S:

Toggle insert mode         P: <CtrlV>                S: <Ins>

Delete line                P: <CtrlY>                S:

Delete line right          P: <CtrlQ><CtrlY>         S:

Delete right word          P: <CtrlT>                S:

Delete current character   P: <CtrlG>                S: <Del>

Delete left character      P: <CtrlBks>              S: <CtrlH>

------------------------------------------------------------------

Begin block                P: <CtrlK><CtrlB>         S:

End block                  P: <CtrlK><CtrlK>         S:

Toggle block display       P: <CtrlK><CtrlH>         S:

Copy block                 P: <CtrlK><CtrlC>         S:

Move block                 P: <CtrlK><CtrlV>         S:

Delete block               P: <CtrlK><CtrlY>         S:

Import from clipboard      P: <CtrlK><CtrlI>         S:

Export block to clipbrd    P: <CtrlK><CtrlE>         S:

Read file into window      P: <CtrlK><CtrlR>         S:

Write block to file        P: <CtrlK><CtrlW>         S:

------------------------------------------------------------------

Find pattern               P: <CtrlQ><CtrlF>         S:

Find and replace           P: <CtrlQ><CtrlA>         S:

Find next                  P: <CtrlL>                S:

Tab                        P: <CtrlI>                S:

Exit editor                P: <CtrlK><CtrlD>         S:

Abandon editor             P: <CtrlK><CtrlQ>         S:

Save file and continue     P: <CtrlK><CtrlS>         S:

Insert control char        P: <CtrlP>                S:

Editor help                P: <F1>                   S:

------------------------------------------------------------------

MUTNCBAY Driver V 1.02          73                        21/04/95

Appendix C - Summary of Script Language

==========   --------------------------

CMD

---

This keyword returns the TNC to command mode.

WHEN <text>

-----------

This  keyword halts execution of the script file  until  the

<text> is monitored in the incoming TNC text.

REPLY <text>

------------

This keyword sends the <text> to the TNC.

DO

--

This  keyword  has  no parameters and forms a  loop  in  the

script file in conjunction with the UNTIL <text> keyword.

UNTIL <text>

------------

This  keyword  signifies the end of the DO ..  UNTIL  <text>

loop.  The  statements between DO and UNTIL  are  executed  until

<text> is monitored in the incoming TNC text.

GOTO <label>

------------

This keyword causes execution of the script file to continue

at the line defined with <label>.

:<label>

--------

This  keyword is used in conjunction with the  GOTO  keyword

and identifies a place in the script file.

DISPLAY <text>

--------------

This keyword will display <text> in the user window  without

sending it to the TNC.

MUTNCBAY Driver V 1.02          74                        21/04/95

PRINT <text>

------------

This  keyword  will  print <text>  to  the  printer  without

sending it to the TNC.

READ <prompt>,<subject1>,<subject2>,...

---------------------------------------

This keyword is used to automatically issue 'read  messages'

commands  from a BBS message list, monitoring each received  line

for requested subject matter until the prompt is received.

IF <text>

---------

This keyword is used as part of the SCAN..ENDSCAN loop.

SCAN

----

This  keyword  has  no parameters and forms a  loop  in  the

script file in conjunction with the IF <text>, ENDSCAN  keywords.

Using  this  loop,  you can scan for up to  10  different  <text>

strings  at a time, and depending which one is found,  execute  a

separate  series  of script keywords.

ENDSCAN

-------

This  keyword  has  no parameters and forms a  loop  in  the

script file in conjunction with the IF <text> and SCAN keywords.

STOP

----

This  keyword has no parameters and stops execution  of  the

script file without completing it.

;

-

This  keyword tells the program to ignore the  remainder  of

the characters on the line and so is used to add comments to  the

script file.

LOG <file>

---

This  command  is  used in create  a  sequentially  numbered

capture file stored in LOG.

MUTNCBAY Driver V 1.02          75                        21/04/95

ENDLOG

------

This closes the file created by the LOG keyword.

WRITELOG <text>

--------

This command writes <text> to the current log file.

CLS

---

This command clears the TNC output window.

COLLECT <label>,<whoto1>,<whoto2>,...

-------

This keyword collects together all mail who address contains

any of the whoto's specified in the command ready for forwarding.

If there is no mail, a GOTO <label> command is performed.

FORWARD (BBS) (FBB)

-------

This  keyword forwards mail, performing BBS type  forwarding

if  the BBS parameter is specified and FBB compressed  forwarding

if the FBB parameter is specified.

REVERSE <label>

-------

This  keyword  reverse forwards mail to BBS's.  If  you  get

disconnected during the forward, a GOTO <label> is executed.

RUN <command>

---

This   keyword   will  execute  <command>  to   run   either

a .EXE, .COM, .BAT file or an internal DOS command.

FBBCOLLECT <label>,<subject1>,<subject2>,...

----------

This  keyword  scans the list of FBB header  broadcasts  for

messages containing any of the subjects specified in the  command

ready  for  grabbing.  If there is no messages,  a  GOTO  <label>

command is performed.

FBBREAD <label>

-------

This keyword downloads messages found during the  FBBCOLLECT

scan. If you get disconnected during the download, a GOTO <label>

is executed.

MUTNCBAY Driver V 1.02          76                        21/04/95

Appendix D - Summary of PMS Commands

==========   -----------------------

@ - Enter sysop mode. (sysop only command)

*DOS commands for sysops only

-----------------------------

*DIR (directory)(filespec) - Display file directory

*TIME (HH:MM:SS) - Set or display the computers time

*DATE (DD-MM-YY) - Set or display the computers date

*CD (directory) - Set or change the current drive/directory

*MD <directory> - Create a new directory

*RD <directory> - Remove a directory

*TYPE <filename> - Display a file on-screen

*DEL <filename> - Delete a file

*REN <oldfile> <newfile> - Rename a file

*COPY <oldfile> <newfile> - Copy a file

*HELP - Display DOS command help summary.

A - Abort.

B - Log off the PMS.

C - Chat to the sysop if he is available.

(Command not available in local mode)

CM <message num> <callsign> - Copy message.

DA <area> <file> - Download file using ASCII.

DS - Display PMS stats.

DU - Display all user stats.

DU <callsign> - Display user stats.

DY <area> <file> - Download file using YAPP.

(Command not available in local mode)

EC - Toggle colour status

EH <homebbs> - Edit home bbs.

EN <name> (H <homebbs>) - Change user info.

MUTNCBAY Driver V 1.02          77                        21/04/95

EM is for sysops only

---------------------

EM <num> U - Make unread

EM <num> R - Make read.

EM <num> F - Make forwarded.

EM <num> N - Make not forwarded.

EM <num> A - Make alive.

EM <num> K - Make killed.

EM <num> P - Toggle private/public status.

EM <num> S - Set next message number

EU is for sysops only

---------------------

EU <call> N <name> (H <homebbs>) - Create/Edit user

EU <call> H <homebbs> - Create/edit user.

EU <call> D - Delete user.

EU <call> B - Toggle BBS status

EU <call> S - Toggle sysop status

EU <call> X - Toggle expert status

EU <call> P - Generate Password

EU <call> C - Toggle colour status

EU <call> E - Toggle exclude status

F(V) <num> <area> <filename> - Convert message to a file.

(sysops only)

H - Display help file.

? <command> - Display help on <command>.

I - Display info file.

J - Show MH list.

K <num> - Kills message.

KM - Kill my mail.

MUTNCBAY Driver V 1.02          78                        21/04/95

K 0 - kill all message base (sysops only).

L ( <num> <num> ) - Lists messages (optionally between range).

LM - List Mine.

LK - List killed. (sysops only).

LL <num> - List last <num> messages.

L< <callsign> - List messages from <callsign>.

L> <callsign> - List messages to <callsign>.

R <num> - Read message.

RK <num> - Read killed message (sysops only).

RM - Read Mine.

RA - Read all mail.

NOTE - Use V instead of R for verbose read command

S <call> - Send private message to station <call>.

SP <call> - Send private message to station <call>.

SB <call> - Send bulletin message to station <call>.

SR - Send reply to last message you read.

SPR - Send personal reply to last message you read.

SM <port> <msg> - Send on-line message.

SD <call> - Manually reply to a REQDIR

SF <call> - Manually reply to a REQFIL

T - Tidy message base (sysops only).

UA <area> <file> - Upload file using ASCII.

UY <area> <file> - Upload file using YAPP.

(Command not available in local mode)

V - Show version.

W - List file areas.

W <area> (filespec) - List files (matching filespec).

X - Toggle expert level.

MUTNCBAY Driver V 1.02          79                        21/04/95

Appendix E - TFPCX Configuration Files For TFPCX V2.10

==========   -------------------------

TFPCX.INI

---------

I   G7JJF                Change to your own callsign !!

C   MAIL

Y   10

R   1

F   6

N   16

@I  80

@T3 30000

U   0

P   0:64

W   0:20

T   0:20

@D  0:0

O   0:4

@T2 0:50

@T4 0:1

M UISC

JHOST1                   Must be last command !!

You  may  also need to add the @C command if you  are  using

software data carrier detect with your Baycom modem. This can  be

entered on any line before the JHOST1 line as :

@C 0:25   (or whatever value is required)

Read the tfpcx documentation for more information on setting

the software data carrier detect value.

Sample Batch Files For Running Software

=======================================

GO.BAT - Runs off COM 1

------

tfpcx -f

tncbay

D:             } change to drive and directory

CD \btnc\obj   } where your prog is stored

tfpcx -u

GO2.BAT - Runs off COM2

------

tfpcx -PCOM2 -f

tncbay

D:             } change to drive and directory

CD \btnc\obj   } where your prog is stored

tfpcx -u

MUTNCBAY Driver V 1.02          80                        21/04/95

GO300.BAT - Runs off COM 1 at 300 Baud

------

tfpcx -B300 -f

tncbay

D:             } change to drive and directory

CD \btnc\obj   } where your prog is stored

tfpcx -u

The  program can also operate with a KISS mode TNC. You  can

either put your TNC into KISS mode before running the program  or

use the KISSINIT program included with the distribution file.

GOKISS.BAT - Runs off COM 2 Using Kiss Mode TNC (KAM)

----------

KISSINIT -PCOM2 -SKAM40 -N

TFPCX286 -F -PKISS2 -N

TNCBAY

D:             } change to drive and directory

CD \btnc\obj   } where your prog is stored

TFPCX286 -U -N

KISSINIT -PCOM2 -K -N

The  TFPCX286  is  a  version of  TFPCX  which  is  compiled

especially  to take advantage of the superior instruction set  of

286  processors and above. Replace with TFPCX if you are  running

on a 8086/8088 processor.

See  the  KISSINIT documentation  for  further  information.

Although  it  is in German, you should be able to  pick  out  the

relevant command line options should you need to alter anything.

PLEASE NOTE : I cannot give technical advice on the operation  of

TFPCX or KISSINIT since I did not write these pieces of software.

Consult  the documentation accompanying the software  or  contact

their original authors for advice.

MUTNCBAY Driver V 1.02          81                        21/04/95

Appendix F - Closing Remarks

==========   ---------------

If  you have any problems using the software, first  of  all

please  read this manual thoroughly. The answer to most  of  your

questions  will  be hidden somewhere. If all else  fails,  please

contact me at the address given below or G7JJF @ GB7MAM for  help

and advice.

I  would  also appreciate any input you would care  to  give

concerning  the program.  If you have any ideas or comments  that

would make it a better program, then please let me know.

*****************************************************

**** Remember To Support Your Local Packet Group ****

*****************************************************

**** If You Have Registered - Thank You, If Not - Please Do! ****

Jon Welch,                    AX25 : G7JJF @ GB7MAM.#23.GBR.EU

50 Quarrydale Road,           inet : jon@g7jjf.demon.co.uk

Sutton In Ashfield,           CompuServe ID : 100012,2607

Notts.

NG17 4DR.                     Tel : 01623 511712 after 6pm

Great Britain

21st April, 1995

MUTNCBAY Driver V 1.02          82                        21/04/95

##########