BPQC.DOC	July 12, 1993

SIMPLE TRANSPARENT DATA COMPRESSION BETWEEN G8BPQ DRIVER AND APPLICATIONS.

Written by:

David Farrell N1HAB

34 Vista Drive

Danvers MA 01923

(508) 887-3795 (Topsfield MA)

Compuserve:73530,2701

Primarily for message forwarding, although end users of BPQ or WA8DED

may take advantage.

This is mostly to serve as an example of data compression, no analysis

of algorithms best suited to the amateur community was performed. I hope

some mathematician out there will derive a good algorithm.

How it works:

G8BPQ driver is intercepted with my driver (so load mine after the BPQCODE).

All data send for a specified stream is compressed then passed to the

BPQ driver.  Up to 31 consecutive data frames are compressed, then a 32cd

non compressed ID frame is sent to notify people monitor why they cant

read the grabage on the screen.  This 32cd frame resets the LZW algorithm

so people who wish to monitor may start every 32cd frame. (This may expand

to 48th or 64th since LZW ability to compress is limited if you dont pass

enough data through).

All data received is passed through my driver and de-compressed. Every 32cd

frame the receiver expects to see the uncompressed identification message.

In full duplex data setups, two separate compression tables are maintained,

one for each direction.  Each may have its frame reset occur at different

times!

The 32cd frame: (see limitations below)

The Uncomressed 32cd I-frame shall display and only display.

"COMPRESSED V01.00"

The V##.## represents the compression type & version.

The two digits before the decimal point represent the algorithm used

to compress the data.

The two digits after the decimal point represent versions.

Simple changes, like changing from 32 to 64 compressed frames per reset

should imply a version change, not algorithm change. Thus V01.00 would become

V01.01

NOTE!  Unless the general public has access to the decoding software

this scheme may be considered ciphers.  If you change the algorithm in any

way, make available a program to decode your algorithm on local HAM BBS

and Compuserve.  Your program should be able to automatically detect your

code and use previous codes so the user doesn't have to figure out which

algorithms to try.

Sending data in this way is no different than using YAPP (or such) to

transmit a binary (ZIP) file.  In both cases you should never intend to

obscure the meaning of the data your transmitting.

Limitations:

Non-library real time compression rely on past data transmitted, so anyone

who wishes to monitor our transmissions would have to monitor from the very

first one!  This effect would provide ciper/code system which we should avoid.

My solution to this is to send a uncompressed message every 32nd frame

which also is used to reset the LZW algorithm.  Any user wishing to

monitor would have to wait for the next "reset" cycle to occur.

This 32 frame reset limits the efficiency of compression, but until a

better compression scheme is devised, I think it helps encourage compression

which is initially more important that compression efficiency.

Implementation Notes:

Data is best send using large blocks.  You must be careful not to

send pre-compressed data as the output data size will be larger!  You

can't compress data twice!  Since compression has the effect of making

transmissions shorter, you should allow at least 4 outstanding packets.

This allows 4 packets to be send on one transmission without having to

re-aquire the frequency (via DCD detect & persist...).

Sometimes the packet length doesn't shrink, but grows with compression.  Since

this may happen, we must provide margin in the paclen parameter.  The BPQ

code should be set for paclen of 250 bytes, while the main program shouldn't

send more than 75% of paclen (187) bytes.  This provides ample margin for the

worst case.  (I like to leave several bytes available in paclen in case some

changes, like the compressed flag below, are needed.)

Possible additions:

Compressed flag as the 1st byte of the message.

Before transmitting you measure the size of the compressed and uncompressed

packets.  If sending un-compressed packets results in a smaller packet size,

then set the flag to non-compressed and transmit.

This is the tricky part:

Whether or not you transmit the packet compressed, the packet should add to

the LZW data tables at both ends!.  I'm not sure at this point how best

to do this (at the receive end), but for LZW to continue working, if needs

as much data as possible!  (the frame resets should still occur as before).

What is intercepted in BPQCODE:

RECEIVE:

- Receive a packet to buffer A.

- BPQC decodes packet in buffer A, stores in local buffer B.

the size of packet in buffer B is returned.

- application reads buffer B message.

TRANSMIT:

- caller sends packet in buffer C.

- BPQC codes packet in buffer C, stores in buffer D.

- BPQ transmits the buffer D packet.

What I would like to do:

Implement the level2 T2 timer (see 2.4.7.1.2 or AX.25 version 2) and don't

transmit any packet to BPQ until sufficient time is allowed for additional

packets to be queued.  This would probably effect transmission time more than

compression since aquiring the frequency is the most time consuming task.

I would have to duplicate BPQ's buffer results and tell fake the application

into thinking BPQ has transmitted packets.