��������  �������  ������     ����   ��    ��

�۱�۱�۱  �۱��۱  �۱���   �۱���  �۱   �۱

۱��۱ ۱  �۱  ۱  �۱ �۱ �۱�  ۱  ��  �۱�

� �۱  �  �۱�  �  �۱ �۱ �۱    �   ���۱�

�۱     ���۱    ����۱� �۱         �۱�

�۱     �۱۱    �۱���  �۱        ����

�۱     �۱ �    �۱     �۱   �   �۱���

�۱     �۱      �۱      ��  �۱ �۱�  ��

����    ����     ����       ���۱� �۱   �۱

����    ����     ����       ����   ��    ��

The Firmware PC Extended

Version 2.71 (23. March 1998)

Resident AX.25-Controller for PC

and BayCom-Modem, SCC-Boards, PAR96, YAM96

PA0HZP-OptoPcScc Board, KISS

with WA8DED-Hostmode-Interface

and DRSI cards

by Henk de Groot, PE1DNN

Based on TFPCX 2.21 by Ren� Stange, DG0FT

Based on TF2.7b by NORD><LINK

English translation and update by PE1DNN

Editorial corrections by PE1MHO

Contents

--------

1.      Foreword

2.      A word about the documentation

3.      Introductions since version 2.0

4.      Quick-start tips

5.      Installation

6.      Invocation and configuration

6.1.    General options

6.2.    Options for resident loading

6.2.1.  Port and baudrate configuration

6.2.2.  TFPC and DRSI-interface

6.2.3.  Other options

6.3.    Clearing TFPCX from the RAM

6.4.    Terminal-mode

7.      Installation

7.1.    SP (DL1MEN)

7.2.    GP (DH1DAE)

7.3.    THP (DL1BHO)

7.4.    TERM (DL5FBD)

7.5.    CT (K1EA)

7.6.    DIEBOX (DF3AV)

7.7.    WINPR (DG6BI)

7.8.    TOP (DF8MT)

7.9.    FBB (F6FBB)

7.10.   MS-Windows, OS/2 and others

8.      Operation

8.1.    Multi-Port expansion

8.2.    Individual commands

8.3.    KISS

8.4     DAMA

APPENDICES

1.      Overview of commands

2.      Error-correction (Modem Operation)

2.1.    Send and receive problems

2.2.    Problems with other programs

2.3.    Hardware problems

3.      Hardware connections

3.1.    Serial modems

3.2.    BayCom USCC-card

3.3.    BayCom PAR96 and PICPAR modem

3.4.    YAM96 modem

4.      Information for software developers

4.1.    Program-interface

4.1.1.  TFPC-interface

4.1.2.  DRSI-interface

4.1.3.  Special functions

4.2.    Format of reports

4.3.    Extended hostmode

4.4.    Previous versions

1. Foreword

In  1997  DG0FT  published  his  work  under  the  GNU  General Public

Licence, and stopped maintaining the code. Although this TFPCX  worked

reasonably well, it was not suitable for use with DAMA stations.  This

release used an old version  of TheFirmware by NORD><LINK. During  the

next  few  years  the  Dutch  packet  network will be converted to use

DAMA.  There are  still a lot of  users using TFPCX, and  therefore it

seems to be a good idea  to exchange the TheFirmware version in  TFPCX

for the latest version, which is TF2.7b.

There is an alternative to TFPCX, which is TFX by DB7KG. But they  are

not quite the same.   The main advantage of  TFPCX is the  possibility

to  use  different  modems  at  the  same  time (different independant

ports). Some of the modem  implementations in TFX are better,  but for

others  the  TFPCX  implementation  are  preferable. Replacing the old

version of TFPCX with this  one will ease the change-over  to software

supporting the DAMA protocol.

Legal rambling

The legal status of this product  is a bit strange. Both the  ALAS and

the GNU  General Public  License are  applicable. This  work shall  be

seen  as  an  aggreate  work  as  defined  in  the  GNU General Public

License.  The source code is  distributed in such way that both  parts

are clearly distinguishable - therefore we don't confirm or deny  any-

body's  right.   Both  parts  can  be  treated  according to their own

license.  The resulting executable  has to follow ALAS since  that one

poses the most restrictions.   Since the source code is  independently

available in un-merged form:   this does not deny  anybody the use  of

the  part  which  was  under  GNU  GPL  control.  Before you burn your

fingers  to  put  me  right  -  it  is  highly questionable if the GNU

General Public License  is applicable to  TFPCX anyway; it  contains a

big portion of NORD><LINK's TheFirmware and violates the ALAS  license

which was applicable for it. Anyway this is the current situation  and

if  you  want  to  know  more,  read both LICENCE.DOC and ALAS.TXT and

contact your lawyer.

Now  I  want  to  give  the  word  to  Rene,  DG0FT,  who  made  TFPCX

possible.....

----------------------------------------------------------------------

"The additions in this version are  listed in chapter 3. In chapter  4

some  advice  is  given  for  those  who are upgrading from an earlier

version.

Note that I can not guarantee that TFPCX will run without any  problem

on every PC  when using a  serial modem:   especially when using  a PC

with  a  slow  CPU  you  may  encounter problems during reception. XTs

running with  a clock  frequency below  8MHZ are  not usable  (or with

limitations). Some  TSR's (Terminate  and Stay  Resident) programs are

known  to  cause  problems  (See  Appendix  2.1.). Since this is not a

commercial product I take this  for granted. I think this  program wil

work for most users with some compromises .

I would  like to  thank Peter  (DB2OS), Asko  (DG2BRS), Denis (G0KIU),

Henk (PA0HZP),  Rob (PE1CHL),  the BayCom-Team,  all contributers  and

all the users who have helped me with advice and recommendations."

73 De Ren�, DG0FT                      Strausberg, 20th. November 1993

----------------------------------------------------------------------

And I like to  thank Ren� (DG0FT) and  the NORD><LINK team for  making

all this  software and  making it  available so  I could  extend it. I

also want to thank the Beta testers for their valuable feedback.  Note

that although  I updated  the document,  the major  part is written by

Ren� DG0FT,  so 'I'  in this  document will  refer to  him, not to me!

Give the  credit to Ren�.  Thanks to  Nico IV3NWV for  the information

about the YAM96 modem and testing.

73s Henk, PE1DNN                              Apeldoorn, 23 March 1998

2. A word about the documentation.

I assume you have  a suitable Modem or  TNC. This can be  any modem or

TNC which  is supported  by TFPCX.   Furthermore, you  need a suitable

Terminal Program (capable of handling  TFPCX).  Finally you also  need

some knowledge  about the  TNC-Software, which  is TheFirmware version

2.7b by  NORD><LINK.   As reference  I enclosed  the documentation  of

TheFirmware  2.7b  which  is  built  into  this version of TFPCX. This

document  is  however  only  available  in  German,  so polish up your

foreign languages  (and if  you happen  to be  German the same applies

since this document  is written in  English).  You  can also wait  and

hope that somebody  will translate it  for you.   Only the differences

compared to TNC-Firmware will be considered here.

If  you  get   TFPCX  running  without   reading  this   documentation

completely you may save some time,  but you may have overlooked a  few

tricks  which  will  come  in  handy.   If  you  have any questions or

problems,  please  find  a  solution  by  reading this document first.

Practice has shown  that the same  questions are raised  over and over

again. I have kept the frequently  asked questions in mind when I  was

writing this document  but this document  will never be  complete.  Of

course this documentation  is not a  basic course in  MS-DOS or packet

radio. A little  hint:  you  can quickly find  a keyword by  using the

search function of a Text Editor.

This  document  mentions  certain  hard  and  software  which has been

designed by other Radio Amateurs:   most of the time their call  signs

are enclosed between brackets.

At some  places (especially  appendix 1)  10 is  mentioned as  default

value for the number of  channels. This value is however  configurable

and is only used as indicator.  When using the '-CH' option the  value

'10' has to be replaced by the value parameter.

Concepts and Abbreviations:

Port       a packet radio interface consisting of a port ( COM, LPT

or SCC-Port), modem and a tranceiver. When using more

ports it is called multiport-operation.

Channel    one of the maximum 10 simultaneous Connects. When using

a BayCom modem the meaning of Port and Channel is exactly

reversed. Confused?

Frame      one complete data unit transmitted by packet radio

(packet), consists of an address field, a control field,

data and a checksum.

Interrupt  interruption of the currently running program due to a

hardware-event (e.g. a key press, elapse of a defined

time-interval).  Software-interrupts are not caused by

hardware but created by executing a particular program

instruction.

KISS       (KA9Q and others) stands for 'Keep It Simple Stupid' and

defines a simple Data Format for the transfer of Frames

and TNC-Parameters over an asynchronous Serial Port. The

original goal was to move the protocol processing from the

TNC to the Terminal-CPU in order to run protocols which

were not supported by the TNC. KISS is implemented in many

TNCs: it also enables direct connection to the computer.

SMACK      (DL5UE and DK5SG) is the abbreviation of 'Stuttgarts

Modified Amateur radio-CRC-Kiss' and extends the on error

free transmission based KISS with a checksum (CRC) so

transmission errors can be recognised.

0x         Prefix of Hexadecimal Number (e.g. 0x300 = 300H)

3. Changes since 2.0

- The YAM 9k6 modem is supported now.

Option -PYAMn:<Base>:<IRQ>  n = 1-4 (Number of the COM-Port)

The  default  <Base>  address  is  the  address  known  by the BIOS.

default <IRQ>  4 (COM1  or COM3)  or 3  (COM2 or  COM4) will be used

when not otherwise specified.

The  YAM  modem  does  not  support  Soft-DCD.  The  @C will have no

effect. The parameter @D (Duplex) will be ignored.

- Added -SA option. For transmission the  'frame-sammler' function  is

off by default. By using -SA you can switch it on again.

- At link-setup  time timer T1  (frame-acknoledge timer) did  not run,

this resulted in an  unusable link if at  startup the first I  frame

was  missed  (T1  regulates  retransmission).  This  problem occured

freqently when the UA response to  SABM at setup was missed and  the

CTEXT (U command) was send  directly following the UA (occurs  often

when using SP or GP).

- Before generating a  RR response a check  is done if DCD  is active.

If so  the generation  of the  response is  postphoned.  This allows

the use of much smaller @T2  values then those needed for KISS  use.

This will  make the  link faster  and should  be compatible now with

TFPCX221 and earlyer.

- Changed TF2.3b to TF2.7b - corrects the usage for DAMA

- Due to the use of TF2.7b, the "Framesammler" is supported

- When connected to a DAMA master the DCD is now ignored like in

TFX and TheFirmware 2.7b based TNCs (with the latest TF2.7b).

- When using the -ST option on the commandline the DOS-Time will be

used to set the clock in the TNC. This will give the real

time to the timestamps (K command)

- When you wanted to start Windows95 after TFPCX has been loaded and

unloaded, Windows95 would complain about a non-conformant TSRs and

use a so-called 'compatibility mode', in effect using 16-bit DOS

drivers instead of 32-bit Windows95 drivers. This has now been

solved.

- The BayCom PAR96 and PICPAR modems are supported

Option -PPARn:<Base>:<IRQ>  n = 1-4 (Number of the LPT-Port)

The default <Base> address is the address known by the BIOS.

<IRQ> 7 will be used when not otherwise specified.

The Soft-DCD can be activated by the @C command. All values other

than 0 have the same effect. The value only has influence on the

DCD-indicator (more stable with larger values). The parameter @D

(Duplex) will be ignored.

- RMNC-CRC-KISS is supported when using -PKISS and automatically

activated. The command @ST shows for both SMACK and RMNC-CRC-KISS

a "+" symbol.

- The BayCom Digi SCC-card is supported (only Ports 0 to 3).

Option -PDSCC:<Base>:<IRQ>:<Modems> (Default -PDSCC:300:7:2222)

- The TheFirmware K command (Timestamp) can be used. The flag -ST

should be given on the command line to enable it.

- FlexNet frames with header-compression are shown in the monitor.

The command M +/- is used here.

Format of the monitor-header:

fm #<Qso> to <Call> ctl ...

fm <Call> to <Call> ctl SABM+ #<Qso>

fm <Call> to <Call> ctl UA- #<Qso>

<Qso> = FlexNet QSO Number

- For KISS, BayCom PAR96-Modem and PA0HZP OptoPcScc-card, the

real-time clock will be used as timebase for internal timers so the

time measurement will be more precise (not under Windows).

- The option -ND has been removed.

- KISS-Mode (including SMACK support) for up to 4 Ports and 57600

baud is supported (Option -PKISS). Frame errors (e.g. through

character loss) will be ignored and counted (See Section 6.2.1 and

8.3.). By means of TFPCX and KISS the program GP (DH1DAE) can now

control more TNCs simultaneously.

- The PA0HZP OptoPcScc-card is supported (option -POSCC, see section

6.2.1).

- The BayCom 9K6 USCC-card now functions with TFPCX (Option -PUSCC,

see section 6.2.1.). When using some BayCom USCC-cards, meaningless

data was transmitted because of timing problems. This problem has

been avoided by using another way of driving the SCC-controller, .

- For SCC-Cards and COM-Ports (with KISS) AT-IRQs (9, 10, 11, 12, 14,

15) can be used.

- The number of free buffers (Option -BU) and the number of channels

(Option -CH), and therefore the required RAM-Space, can now be

configured. (Default values: 600 Buffers/10 Channels, see section

6.2.3.)

- The initialisation file (option -F) may now contain empty lines

and comments. ESC-Characters will be recognised automatically and

no longer require defining with  '^' (See section 6.2.3).

- When using the DRSI-interface (Option  -DR), the incompatibilities

with respect to the DRSI TNCTSR-driver have been removed. These

incompatibilities previously lead to  problems with the monitor and

heard-list in combination with FBB  (F6FBB). In cases where these

modifications now give problems with other programs (e.g. with

TOP), using the option -DX  will  enable the previous (modified)

DRSI-Interface again (See section 6.2.2.1).

- The new @PO command enables selective allocation of a channel to

a port, so the channel can only be connected from this port. This

is useful when using the Program TOP (DF8MT) or TstHost (IK1GKJ).

(See section 7.8 and 8.2.).

- By means of a Transparent-Mode during reception (Command @M) and

switchable character echoing (Command @E). #BIN#-Reception in is

possible in Terminal-mode (e.g. with TERM (DL5FBD)). (See section

7.4 and 8.2)

- Internal connects (Loopback) can be switched off when needed

(Option -NL, see Section 6.2.3.).

- TXTAIL (Command @TA) in combination with the serial modem and

SCC-Card will be optimally installed according to the baud-rate and

timer inaccuracy. Previously it gave some problems with operation

at 300 Baud.

- If the Option -P is not specified, it will not longer default to

a serial modem on COM1 as before , but to no port at all, which is

only meant for test purposes with internal connects. Therefore -P

should always be specified in normal cases.

- From now on, the option -D (Debug) is on only applicable to the

most recently specified option -P (Port). This will enable

monitoring of a specific port, even during multiport operation (See

section 6.2.3.).

- Disconnecting in the background mode or terminal mode is possible

by means of the remote command '//Q' (Command U, see section 8.2.)

- Because of a DAMA-change (since TF 2.6) the known "complaint-

messages" from of TheNetNode-Digis during multiconnect should now

be eliminated.

- The Extended Host-Mode (DG3DBI) is now supported and enables

faster communication with a terminal program.

- The default-values for the Parameters F, N, P, R, T, U, @I, @T2,

@T3, AND @TA have been changed.

4. Quick Start

Because  of  the  many   different  configuration  settings  and   the

existence  of  many  different  terminal-programs no general 'recipes'

can  be  given.   In  most  cases  it  will  be sufficient if you read

sections 6.2.1  and 7  (depending on  the program  used).   In case of

trouble read appendix 2.   For multiport operations, the sections  8.1

and 8.2 are  important.  In  section 8.3 important  tips for operation

using KISS are given.

If you used a previous version without the '-P' option and were  using

the default COM-1 port you now have to include the option '-PCOM1'.

If you  previously used  version 2.0  with more  than 10 channels, you

now have to supply the  option '-CHnn' during startup of  TFPCX, where

nn is the number of channels required.

If you  used the  option 'DR',  and this  version introduces  problems

with your monitor or heared  list, you should supply the  option '-DX'

instead of '-DR'(e.g. with TOP)

If you worked with version 1.1x up  till now and only want to use  one

modem you can additionally specify the option '-C' when loading  TFPCX

if you want  the send-/receive indicator  - it is  not switched on  by

default anymore. If you used the  option 'NC' up 'til now you  have to

delete that.

Command 'TFPCX -H' will show all allowed options in a shorthand  form.

Command 'TFPCX -U' removes TFPCX from RAM.

5. Introduction

A few years ago packet  radio was mainly operated using  Terminal-Node

Controllers  (TNCs)  which   took  care  of   the  execution  of   the

PR-specific protocol (AX.25), and used the computer as a tool to  look

at the data in a convenient  way. The TNC itself is nothing  more than

a small  computer with  a serial  port and  a modem.   Due to the wide

distribution  of  software  for  the  TNC2  (WA8DED-Firmware  or   the

compatible  TheFirmware  from  NORD><LINK)  many  popular  PC programs

support the software-interface of this firmware (WA8DED-Hostmode).

Since powerful PCs are not longer expensive (in fact they are easy  to

get second-hand),  it becomes  feasible to  let the  PC take  over the

work  of  the  TNC  and  save  the  investment in a TNC. Additionally,

portable operation using a laptop  or a notebook is much  more elegant

if you do  not have to  carry an additional  'black box' arround.  For

this purpose the BayCom  system (Terminal, Modem or  USCC-Card) offers

a quick, cheap and easy to understand solution. But the BayCom  system

is a closed system without the support for TNC terminal programs.

At  this  point  TFPCX  comes  into  the  picture,  since TFPCX is the

connection  between  the  existing  hardware  (serial modem, SCC-card,

Par96 or PicPar)  and the terminal  programs. TFPCX is  a kind of  RAM

resident TNC which is loaded into  the PC once.  It will  conveniently

sit in memory  and will perform  actions in the  background, driven by

the constantly occurring  interrupts in the  PC.  It  will receive and

transmit data and will react to  commands from the user. The data  and

command interface  with the  terminal program  (software interrupt) is

provided in a  similar way as  to that used  by TNCs and  is also (and

this is important) clearly documented.  It is therefore easy to  adapt

an existing program  to get it  running with TFPCX.   This is  already

done in most popular programs.

When  TFPCX  is  loaded  the  system  behaves  as  if  a  real  TNC is

connected,  so  you  can  be  connected  from outside and all incoming

messages will be  stored.  Since  TFPCX runs in  the background, other

programs can  be used  (for exceptions  see appendix  2.2). A blinking

rectangle in the upper right corner will draw your attention if  there

is unread information pending.  The unread information will appear  on

the screen as soon as a  terminal program is started. You can  compare

TFPCX with L2 (DL8MBT) of the BayCom system.

TFPCX  does  not  support  TCP/IP  operation  and  runs  only  on IBM-

compatible PCs.  TCP/IP can be operated with BayCom-compatible  modems

uning the AX25DRV packet driver from Pawel Jalocha (SP9VRC).

6. Invoking and Configuration of TFPCX

TFPCX will be activated by the following Command Line:

TFPCX [ -N ] [ <load options> | -T | -U ]

All  options  start  with  a  '-'  sign  and  are seperated by a space

character.   The  space  character  shall  not  appear  as part of the

option. Options are case-insensitive (no difference between upper  and

lower  case.   Certain  options   (e.g.  '-P')  have  more  than   one

parameter,  separated  by  a  ':'.  For  those  values  which  are not

explicitly specified default values will be used.

Example:

Option '-PUSCC::5' will be interpreted as '-PUSCC:300:5:1103', since

the omitted values default to 300 and 1103.

It makes sense to start TFPCX  from a batch-file so you won't  have to

type in the same options over  and over again. All options are  listed

in short-hand form, which will  also be presented as help-text  if you

enter 'TFPCX -H', The <load  options> are only relevant while  loading

TFPCX as a TSR, and remain valid until TFPCX is unloaded from memory.

<general options>

-N no messages

-T terminal mode

-U unload

<load options>

-P<port>[:xxx:nn:nnnn]  packet port [addr:IRQ:<clock>]

-Bnnnn[:nnnn ...]       baud rate (1 number/port)

-F[file]  read init file      -D  debug mode

-C[xx]    show DCD [color]    -DM use DRSI messages

-Ixx      TFPCX interrupt     -DR emulate DRSI driver

-BU[nnnn] number of buffers   -DX modified DRSI interface

-CHnn     number of channels  -NB no blinking rectangle

-ST       time stamp          -NL no loopback

-SA       sammler on for TX

<port>  COMn | LPTn | PARn | YAMn | KISSn | DSCC | OSCC | USCC (n = 1-4)

<clock> 0 = disable    2 = hardclock    4 = PA0HZP port  (1 digit/

1 = softclock  3 = DF9IC modem  5 = PA0HZP timer  channel)

<legend>

[]  optional use

|   alternative command

x   Hexadecimal number

n   Decimal number

6.1. General Options

These options can be used together with every other option.  Currently

there is only one:

-N  Suppress Messages

In  case  messages   from  the  progam   are  not  wanted   (e.g.   in

batch-files),  they  can  be  suppressed  by  using this option. Error

messages are not supressed.

6.2. Options for loading as TSR

TFPCX  will  always  load  as  a  resident program in RAM, except when

either option  '-T' or  '-U' are  specified, or  when TFPCX,  TFPCR or

DRSI-TNCTSR are  already running  resident in  memory. If  you want to

use multiple drivers simultaneously, TFPCX has to be the first TSR  to

be loaded. I  cannot guarantee that  it will work  without problems in

this case.

The TFPCX program can also be loaded high in upper-memory (UMB)  using

the LOADHIGH command  if there is  sufficient room. Keep  in mind that

there  are  some  problems  when  using  EMM386-drivers  however  (see

appendix 2.1.).

In order to addapt  TFPCX to different hardware,  transmission speeds,

terminal programs  and user  preferences there  are a  lot of options,

which will be descibed here.

After loading, the following report will appear (example)

��������������������������������������Ŀ

� TFPCX v2.71  (Mar 23 1998) by PE1DNN �

�  Based on TF2.7b/TNC2 07Jun95 code   �

�      TheFirmware by NORD><LINK       �

��������������������������������������Ĵ

� Contains portions of TFPCX by DG0FT  �

� published under GNU GPL Licence.     �

� All other portions Copyright         �

� (C) by NORD><LINK e.V. 1988-1995.    �

� Es glit die allgemeine Lizenz f�r    �

� Amateurfunk Software (ALAS).         �

� ONLY for non-commercial usage.       �

��������������������������������������Ĵ

� 5 Port(s), 10 Channels, TFPC-Int FD  �

��������������������������������������Ĵ

� 0: COM1 (3F8/00),  1200 Bd, MODEM    �

� 1: SCC0 (300/07),  1200 Bd, SOFTCLK  �

� 2: SCC1 (301/07),  1200 Bd, SOFTCLK  �

� 3: SCC3 (303/07),  9600 Bd, DF9IC    �

� 4: COM2 (2F8/03),  9600 Bd, KISS     �

����������������������������������������

followed by the DOS  prompt. TFPCX is now  loaded and occupies a  part

of memory.  In  the lower box the  number, the interface, the  address

and when applicable the IRQ, the baud rate and types of the  connected

modems are displayed.

6.2.1. Port- and Baud Rate-Configuration

-P  Specification of the used ports

This option can be specified  multiple times:  twice times  for serial

modems, once for -SCC cards,  once for BayCom PAR96 or  PICPAR modems,

once for  the YAM96  modem and  4 times  for KISS  ports, but not more

than 8 times in  total.  Do not  underestimate processing load on  the

PC when using serial modems.

The allocation of  port numbers will  follow the same  sequence as the

order in  which the  ports are  defined on  the commandline, where the

order of SCC-ports  is fixed. The  report given above  as example will

appear when starting TFPCX with:

TFPCX -PCOM1 -PUSCC -PKISS2

If the option '-P'  is not specified not  any port is used,  this only

makes sense using internal connects.

The optional port adresses should be  in the range 0x100 to 0x3F8  and

be divisible by  8. With SCC  and KISS, use  of the IRQs  2-5, 7, 9-12

and 14-15 are possible (When using XTs only IRQs below 8). ATs do  not

actually have  an IRQ  2. Instead  of this  IRQ 9  is used.  Each port

must have an unique IRQ.

-PCOMn or -PLPTn modem connected to COMn or LPTn (n = 1-4)

The base address of the port will be read from the BIOS-data area  and

has  to  be  present  there.   Most  BIOS-versions  don't  keep   this

information for COM3 and  COM4. In that case  the address can also  be

specified explicitly using '-P<Port>:xxx'.

Example:

TFPCX -PCOM3:338

Using this connamd a modem connected to COM3 will be used, using  base

address 0x338.  This address should  be be looked up in the  manual or

desciption of the interface. The  number of the port (in  this example

3) will be ignored if the base adress is explicitly specified but  has

to be present and between 1 and  4. The IRQ of the interface is  of no

interest to TFPCX since it is not used.

When using 2 modems, the first modem specified shall be the one  which

is  most  frequently  used  because  the  first  modem  has  a  higher

priority. It is evident that  other programs can't use the  same ports

as used by TFPCX.

-PUSCC:<Base>:<IRQ>:<Modems>  use BayCom USCC-card

-POSCC:<Base>:<IRQ>:<Modems>  use PA0HZPcOptoPcScc-card

-PDSCC:<Base>:<IRQ>:<Modems>  use BayCom Digi SCC-card

The base address of the SCC card, the IRQ and a 4 digit number can  be

specified as parameters. The 4 digit number specifies the type of  the

SCC ports (up to  4).  The following  settings are possible (see  also

appendix 3.2.):

0  Disable      Port disabled (Switched Off)

1  Softclock    The transmit and receive clocks are created internally

for use with AFSK-Modems (duplex mode not possible)

2  Hardclock    The transmit clock will be generated by the modem,

the receive clock is created internally (e.g. G3RUH)

3  DF9IC-Modem  The transmit and receive clock will be generated by

the modem, NRZ-Mode

4  PA0HZP-Port  The receive clock will be created internally,

divided externally by 32 and passed back to

the SCC-Controller as transmit clock. (for

OptoPcSCC-card)

5 PA0HZP-Timer  This port generates a timing reference for timing

purposes (Only for OptoPcSCC-card).

The modem types 1-3 are reserved  for the USCC- and DSCC card,  type 4

only functions with the OptoPcScc-Card.

Number 5 has a special  meaning.  TFPCX requires a  regular timer-tick

for internal timing. This timer-tick can be deliverd by the  OptoPcScc

card, but  not when  running under  Windows. In  this case  the system

timer of  the PC  will be  used, which  is, however,  not so accurate.

This  can  cause  a  problem  for  some  parameters  (e.g. TXDELAY and

TXTAIL).  TFPCX  offers  the  possibility  to  use an otherwise unused

SCC-Port  for  generation   of  an  accurate   timer-tick,  which   is

recommended when not all ports are in use.

Examples:

TFPCX -PUSCC:300:7:1103

Base address  is 0x300  and IRQ  is 7.   USCC-ports 0  and 1 are using

Softclock for use with normal  AFSK-modems (the two '1' digits),  port

2 is disabled and port 3  is used with a DF9IC-modem (the  '3' digit).

This is also the default setting when only '-PUSCC' is supplied.

TFPCX -PUSCC:300:7:31

USCC-Port 0 is now setup for a DF9IC modem. Port 1 uses the  Softclock

and ports 2 and 3 are  switched off. This setting is required  for the

9K6-USCC  Card,  which   offers  only  2   SCC-ports.   If  no   clock

specification is supplied  '1103' is used  by default (as  above), but

then a setting is  given using less than  4 digits the missing  digits

will interpreted as '0'.

TFPCX -POSCC:150:3:4445

OptoPcScc-Card with base address  0x150, IRQ 3. Port  0, 1 and 2  will

be used  as modem  ports with  an external  clock. Port  3 deleivers a

timer-tick.   This is  also the  default setting  when using  '-POSCC'

without additional parameters.

TFPCX -PDSCC:300:7:2222

BayCom-Digi-SCC-Card with  base address  0x300, IRQ  7. All  ports are

used with  G3RUH modems,  which will  also deliver  the clock. This is

also  the  default  setting  when  using  '-PDSCC'  without additional

parameters.

-PKISSn:<Base>:<IRQ> KISS-Port on COMn (n = 1-4)

The  base  address  will  be  determined  automatically,  but  can  be

overrulled if  a base-address  is specified.   IRQ 4  will be  used by

default  for  COM1  and  3,  IRQ  3  for  COM2 and 4. When this is not

according  to  the  assingment  in  your  PC  you  have to specify the

correct IRQ.

Examples:

TFPCX -PKISS1

KISS-Port on COM1, base  address determined automatically and  default

IRQ for COM1 used. For COM1  and COM2 this setting will be  sufficient

in most cases.

TFPCX -PKISS3:338:5

KISS-Port on COM3, base address 0x338 and IRQ set to 5

-PPARn<Base>:<IRQ>

BayCom PAR96 and BayCom PICPAR-Modem  connected to LPT 'n', where  'n'

is the number  of the LPT  port. The base  address will be  determined

automatically, but  can be  overridden by  specifying a  base-address.

The default  IRQ is  set to  7.   When this  is not in accordance with

your installation, you have to specify the correct IRQ.

Examples:

TFPCX -PPAR1

BayCom  PAR96  or  PICPAR  modem  on  LPT1,  base  address  determined

automatically and default IRQ 7 used.

TFPCX -PPAR2::5

BayCom  PAR96  or  PICPAR  modem  on  LPT2,  base  address  determined

automatically and IRQ set to 5.

-PYAMn<Base>:<IRQ>

YAM96-Modem connected to COM 'n', where  'n' is the number of the  COM

port. The base  address will be  determined automatically, but  can be

overridden  by  specifying  a  base-address.   IRQ  4  will be used by

default  for  COM1  and  3,  IRQ  3  for  COM2 and 4. When this is not

according  to  the  assingment  in  your  PC  you  have to specify the

correct IRQ.

Examples:

TFPCX -PYAM1

YAM96  modem  on  COM1,  base  address  determined  automatically  and

default IRQ 4 used.

TFPCX -PYAM2::5

YAM96 modem on  COM2, base   address determined automatically  and IRQ

set to 5.

-Bnnnn[:nnnn ...] specification of the baudrate for each port

When using multiple ports the  values separated by a ':'  are assigned

to  each  port  in  order  (first  value  for port-0, second value for

port-1 and so on). The following values can be specified:

Default

Serial Modem   300, 1200, 2400 or 4800 Baud            1200

SCC Softclock    50-38400 Baud                        1200

PA0HZP-Port  50-38400 Baud                        1200

Hardclock    50-38400 Baud                        9600

DF9IC-Modem   1-65535 Baud (without significance) 9600

KISS              2400, 4800, 9600, 19200,             9600

38400 or 57600 Baud

PAR96/PICPAR      9600, 19200 Baud                     9600

YAM               45-19200 Baud (without significance) 9600

When  using  serial  modems,  only  the  above  specified  values  are

possible; when using SCC cards intermediate values are also  possible.

When using Hardclock  the delivered transmit-clock  shall be equal  to

the  specified  value.   When  using  a  DF9IC  modem  the  value   is

meaningless because  the clocks  are generated  externally, but  it is

adviseable to  specify the  correct value  anyway, because  that value

will be  displayed when  using the  'P' command.  When using the YAM96

modem the given  baudrate is cosmetic  too (just like  for DF9IC), the

communication speed to the  YAM96 modem is fixed  at 19200 Baud.   The

initialisation of the modem (using YAMINIT - supplied with the  modem)

determines the actual speed on the air interface.

Example:

TFPCX -PCOM1 -PUSCC:::1003 -B300::19200

Modem on COM1  at 300 baud,  USCC-Port 0 with  Softclock at 1200  baud

(Default, '::') and USCC-Port 3 with DF9IC-modem at 19200 baud.

Which  baudrates  are  possible  on  a  particular  PC, depends on its

computing power (see  table in appendix  2.1.). When using  a a serial

modem at  300 baud  the system  clock will  lose half  a minute  every

hour.

-ST

This option will enable the  use of timestamps on status  messages and

monitor messages by  means of the  'K' command. This  option will also

cause setting of  the TNC time  according to the  DOS time on  the PC.

The default date  style is european  style. This can  be changed using

the 'K' command but  you have to supply  the current date yourself  in

that case.

-SA

This  option   will  enable   the  so   called  'frame-sammler'    for

transmission (TFPCX's reception always uses this feature). If  enabled

TFPCX will only re-transmit the I frame for which a REJ was  received.

If the receiver also has a 'frame-sammler' build in the receiver  will

'remember' subsequent  I frames  and will  acknoledge those immediatly

after the 'lost' I frame has been received.

If the receiver does not have this feature you should not specify  -SA

at  the  command-line  since  re-transmission  takes much more time in

that  case.  When  not  specified  TFPCX  will  (re)transmit as much I

frames as possible.

6.2.2. TFPC- and DRSI-Interface

For communication  with a  terminal program  TFPCX offers  4 different

variations from  which one  has to  chosen, depending  on the program.

Appendix  4.1   describes  the   calling  and   parameterparsing   for

programmers.

The TFPC-Interface was developed by Sigi (DL1MEN) for his  KISS-driver

TFPCR and is widely supported by many programs (e.g. SP, GP,  TSTHOST,

THP, TERM,  DIEBOX). Therefore  this method  is also  selected as  the

default interface for  TFPCX.  The  disadvantage of this  variation is

the  limited  Multi-port  capability,  because  the  transfer  of port

numbers (e.g.  of the  Port on  which one  has been  connected) is not

possible.   Because  of  this,  connections  and  monitor  frames  are

displayed as if they were  originating from one frequency. If  you are

using only one port this variant is recommended.

-DR  DRSI-Interface usage.

This  interface  uses   the  TNCTSR  driver   which  belongs  to   the

DRSI-PCPA-Addapter.   Since  TFPCX  now  supports  multiport  usage, a

method had to be  found for the terminal  program to be able  separate

the different ports. This variant was a suitable alternative since  it

was already fully supported  by SP and FBB.   The difference with  the

TFPC  interface  is  mainly  the  parsing  of the port numbers in link

status reports  and monitor  information.   This option  does not  use

hardware support of a DRSI-PCPA-Adapter.

-DX Modifies DRSI-Interface operation

This  option  corresponds  to  the  'DR'  option  of TFPCX v2.0x. That

version  introduced  an  incompatibillity  with the TNCTSR-Driver when

using  DRSI-Interfaces,  which  has  been  corrected  later.   If that

correction leads to problems in other programs (e.g. TOP), the  option

'-DX' should be used instead of '-DR'.

-DM  Modifies TFPC-Interface operation

This  variant  is  specificly  designed  for  multiport operation with

programs  which  so  far  only  support  the  TFPC  interface. It will

deliver the same type  of messages as the  DRSI interface (so it  will

include port numbers) but uses the already known TFPC interrupt.   The

usage of this variant is a compromise. With many programs (e.g.  TERM)

it does not lead  to any problem but  there are some side-effects  and

some programs (e.g. THP) can not handle the port numbers at all.   The

best solution is to try your program with or without the '-DM'  option

and hope for an update of the program to support multiport  operation.

The program GP forinstance was changed to suppors this.

WARNING!

Please do not overload the developer of your terminal program (or  me)

with messages, if, while using the '-DM' option, something screws  up.

This  option  is  a  compromise  and  has  caused strange effects with

certain programs in the past. Hopefully is has been corrected by now.

6.2.3. Other Options

-BU[nnnn] Size of the TFPCX-Buffer

TFPCX  saves  most  data  in  64  byte  buffers. The number of buffers

needed may be totally different from one application to the other.  If

TFPCX has a small number of buffers  only a few frames can be kept  in

intermediate buffers and every now  and then 'TNC BUSY' messages  will

be reported, which  leads to a  disconnect by the  terminal program in

most cases. Too many buffers waste memory.

The  buffer  size  changed  from  32  bytes  in  TF2.3b to 64 bytes in

TF2.7b. The designers  of TF2.7b decided  to keep the  reported number

of buffers  compatible with  earlier versions,  so when  the number of

buffers is defined it is divided by  2, and when it is reported it  is

multiplied  by  two.   The  same  is  applied  to TFPCX. The number of

buffers is still defined as if  the buffers are still 32 bytes.   With

this option  the number  of buffers  set between  400 and approx. 1400

which will result in 200 to 700 buffers of 64 bytes internally.   When

reporting the  number of  free buffers  the internally  kept number of

free buffers is multiplied by  2, the reported number will  never have

an odd  value. The  maximum value  depends on  the number  of channels

specified and will  be allocated when  the option -BU  is used without

an actual number.

The  default  value  of  600  buffers  (300  real  buffers) is usually

adequate. If many channels are  in use, Gateway-Functions are used,  a

Mailbox is serviced or large  Files transferred on fast Digi-links  it

makes sense to increase the value.   When using SP (except for  V7.50)

you should  not specify  more than  999 buffers  (see Section 7.1). If

there is  very little  RAM available,  then also  a smaller  number of

buffers can be used.

-CHnn Number of Connect-Channels

With this option,  the required number  of channels can  be specified.

The number of channels  should be the same  as the number of  channels

selected in the  Terminal Program, if  defined more it  wil just waste

valueble  RAM  space.   Selection  of  4  to  40 channels are possible

(Default 10).

-C[xx] Transmit-/Receive indicator switch

When using this option  an indication of the  transmit-/receive status

will be  displayed while  in Host  mode. The  indicator appears at the

top right hand corner of the screen ('S' for transmit (Send), 'R'  for

receive).   When more  ports are  to be  used, each  port will  have a

separate indicator, the left indicator  will be the the indicator  for

port 0. When using KISS mode the indicator will display the status  of

the  connection  between  PC  and  TNC.  The  indication only works in

textmode.  The additonal parameter specifies a color attribute.

Example:

TFPCX -C17

^ Foreground (here White)

^  Background (here Blue)

Numbers of the Colour Attributes:

0 Black    4 Red      8 Dark-Grey   C Light-Red     Monochrome:

1 Blue     5 Magenta  9 Light-Blue  D Light-Magenta

2 Green    6 Brown    A Light-Green E Yellow        07 Normal

3 Cyan     7 White    B Light-Cyan  F Light-White   70 Inverse

^             ^

only foreground

-F<File> File for the setup parameters (default TFPCX.INI)

When using this  option (and only  then!) the specified  file (or when

no file name  is given TFPCX.INI)  will be read  during the starup  of

TFPCX and send character by  character to the TheFirmware core  inside

TFPCX.   This makes  it possible  to give  a pre-defined  value to the

parameters.   Normally  this  option  can  be  omitted,  because  most

terminal programs will initialise the TNC themselfs. The file will  be

looked for in the current directory if no pathname is given.

The  file  can  be  created  using  normal text editor and may contain

comments (preceded by '#' or ';')  and empty lines.  For each  command

found  an  escape-character  will  be  sent first automatically. Older

versions required a  '^' first. This  is not longer  required and will

be ignored when found,  so old files can  still be used. Tabs  will be

handled as spaces and  will be ignored at  the beginning and end  of a

Line.   An  example-file  is  included  in  the distrubtion package of

TFPCX.

-Ixx  Software-Interrupt for TFPCX-Interface (40-FF)

The  communication  between  TFPCX  and  the terminal-program uses the

sepecified software-interrupt.  Default  value is interrupt 0xFD.  You

only need to change this if the specified interrupt is already in  use

by another  program or  when the  terminal program  only works  with a

specific setting (e.g. '-IFF' with CT (K1EA) and FBB (F6FBB)).

-NB  Switch off Status Flash

When TFPCX  has unread  information or  status changes  in its buffers

and  is  not  running  in  hostmode  (so  it  is running background) a

rectangle will start  flashing in the  upper right hand  corner of the

screen.   This will  allert the  user that  for example  a connect  is

pending. The user can now start  his terminal program to react to  the

connect. This will not work if a DOS-Shell is started from a  terminal

program  which  will  keep  TFPCX  in  hostmode (as e.g. with SP). The

flashing can suppressed using this option.

-NL Internal Connects (Loopback) Switch Off

In normal  cases all  transmitted frames  are handled  as if they were

also received: this enables internal  connects for test purposes.   In

some cases  this is  not wanted  (e.g. during  tests with  an external

loopback)  and  therefore  this  option  can  be used to prevent this.

Under high load the use of  this option will also reduce the  CPU load

a  little  because  the  transmitted  frames  don't have to be handled

twice.

-D  Test Mode (Debug)

This  option  is  valid  for  the  most  recently supplied '-P' option

issued before the '-D' option on the command-line. It will activate  a

test-mode for this port or  these ports. Each interrupt for  this port

will  lead  to  a  voltage  change  on  the  internal PC speaker which

results in a ticks (cracks) or a tone.

The  test  mode  is  mainly  used  for  fault-finding  when  there are

transmit or  receive problems  during modem  operation. (see  Appendix

2.1.). When using 1200 baud  you should hear a 1800  Hz-Tone (baudrate

*  1.5).   This  tone  should  be  clear and without interruptions. An

unclear tone is caused by  delayed interrupts. If the qualtity  of the

tone sounds disturbed or interrupted then the CPU is overloaded.  What

is  and  what  is  not  acceptable  is hard to define: some background

noice is most likely still acceptable.

When using this  option with a  KISS, SCC-Card, PAR96/PICPAR  or YAM96

modem  you  can  check  if  interrupts  are  generated  at  all. These

interrupts  can  occur  constantly  or  only  during  transmission and

reception.

When the option '-D' is specified  in before any '-P' option then  you

will hear ticks in time with the system clock.

6.3. Removal of TFPCX from RAM

By using  the command  'TFPCX -U'  you can  remove the  resident TFPCX

from the memory.

6.4. Terminal Mode

The command 'TFPCX  -T' will start  a simple terminal  so you can  use

TFPCX without additional terminal  sofware. TPFCX should already  have

been loaded  (as described  previously). This  means that  the program

should be invoked  twice with different  parameter settings (first  to

load it resident and the second  time using this -T option), in  order

to get into the terminal mode.

The  key  combination  ALT-X  will  stop this terminal program. Before

doing that you should switch off the monitor (if activated) using  ESC

'MN', because otherwise the  internal buffers will be  exhausted quite

quickly, and it may cause  problems when starting a host-mode  program

afterwards.

7. Installation

In this section  some advice is  given for the  installation with some

programs using  TFPCX. In  all cases  TFPCX has  to be loaded resident

before the  terminal program  is started.  Additionally, some settings

in configuration files  or menus are  needed.  On  the other hand  the

specified baudrates  in these  files or  menus are  irrelevant most of

the time: the option '-B' at the start of TFPCX takes care of this.

Some  programs  have  problems  with  multiport use (option '-DM'). In

these cases you  should skip this  option and with  it the display  of

port  numbers.  The  use  of  port  numbers  is  still possible in the

commands so  you can  still use  multiport operation  anyway. You just

don't  see  on  which  port  somebody  connects or on which port which

monitor-frame is received. When using these kind op programs it  makes

sense to use the '@PO' command (see chapter 8.2).

You should be  aware that TFPCX  occupies a part  of menory, so  it is

not free  for use  by the  terminal program  anymore. This may require

you  to  decrease  the  number  of  buffers  and the number of connect

channels to get both TFPCX and the terminal program in memory.

7.1. SP (DL1MEN)

Depending on the number of ports, two variants are available:

When only  one port  is required  you can  opt for the TFPC-Interface,

which is supported since SP v5.0.  In this case you should install  SP

according to  the SP-manual  or using  the INSTALL  program install SP

for TFPCX  (or TFPCR)  usage. The  file CONFIG.SP  (previously SP.CFG)

shall include  the line  'CFG=PORT0:5H'. In  practice there  is hardly

any difference between TFPCX and TFPCR.

For  multiport  use  you  should  use  the  DRSI-Interface.   The file

CONFIG.SP shall include  the line 'CFG=PORT0:DH'  and TFPCX has  to be

started using  the '-DR'  option. Apart  from that  you should keep in

mind  that  everything  said  about  the  DRSI-PCPA-Addapter in the SP

manual is also applicable to TFPCX, with the following exceptions:

- SP-Commands 'DR' and 'HB' do work, the parameter will be set using

the normal TNC-Commands with and additional port designation (see

Section 8.1).

- The port selection mechanisme (described in chapter 8.1) without an

explicit specification of the port does not work for some commands.

SP will send those to the monitor channel 0 instead of to the

actually selected channel. When you are not sure, always specify

the port number explicitly.

- Commands 'TP' and '//par' will show the correct baud rate (and not

a number)

When using the DRSI variant with  SP, the SP program will allocate  an

extra QRG for each  of the 8 ports  , except for the  monitor channel.

Switching  off  ports  can  be  accomplished  through  'C  1:', on the

monitor  channel  this  will  select  the  unproto  port. The selected

default port will be selected automatically when a connect command  is

given, but can also be specified directly.

TFPCX can also be  used in parallel using  TNCs in hostmode. How  well

this will function depends on the computing power of the PC.

When using SP6.0  (or earlier), the  QRG indication is  a bit unstable

in some cases and the connect bell doesn't sound the same compared  to

the sound you hear when using a  normal TNC.  This is normal and  does

not cause problems.

With early versions of SP v7.00 a bug may cause problems when using  a

modem. Another  problem arises  if, while  using this  SP version, you

use 2 modems and a TNC  in parallel (which I'm sure somebody  will do)

and are  unsing the  DRSI interface.   At the  startup of  SP you will

immediately  get  a  Resync  message  and  it will refuse to run. This

problem can be solve with a patch for SP.EXE.

When using  SP v7.00  (or earlier)  you should  not allocate more than

999 TFPCX buffers (option '-BU') because these versions have  problems

displaying  more  than  4  digits,  and  the transmission will be very

slow. In SP version 7.50 this problem is solved.

7.2. GP (DH1DAE)

GP  will  use  TFPCX  automatically  if  it  is  loaded,  therefore no

additional configuration settings are  required in normal cases.   For

multiport use you have keep the following in mind:

- GP version 1.50 or better is required. Earlier versions can only

cope with one port, and they also have problems with higher

baudrates.

- You have to start TFPCX using the option '-DM'.

- You must specify at least one PORT command in the file NAMES.GP,

because without this command all connect attempts are terminiated

with an error message.

Example:

PORT0 = DB0BLN,438.450

PORT1 = DB0BLO,438,300

With the  use of  TFPCX,   GP is  able to  control more  than one  TNC

simultaneously. For  this to  work, these  TNCs have  to support  KISS

mode operation (see section 8.3.).

7.3. THP (DL1BHO)

THP supports the  TFPC-Interface (that is,  version 3.03 did).  In the

configuration file CONFIG.PR  you have to  specify port number  '5' in

the 'TNC-Configuration'  section. In  the file  CMD.PR a  few commands

are normally  executed which  lead to  an error  message in TFPCX (for

example 'A' and 'Z'). The best solution is to comment these lines  out

by putting a '#' a the beginning of the offending lines.

Multiport-operation using  the option  '-DM' gains  nothing since  the

port numbers are not correctly displayed.

7.4. TERM (DL5FBD)

This  TFPCX-version  works  best  with  TERM  version  9.71  or later.

Earlier  versions   of  TERM   used  another   method  for   transmit-

handshaking. This method is not supported by TFPCX anymore.

When you want to use more ports you should load TFPCX using the  '-DM'

option. Since  TERM does  not process  the TFPCX  messages itself,  it

does not exhibit problems with the port numbers.

After starting TERM, press the  key combination ALT-P: this will  open

the parameter-menu. The following settings will work for TFPCX:

V24-COMx:     5

Handshake:    S

Protect Time: 0 (without BREAK)

It is  possible to  receive #BIN#  files with  this version  of TFPCX.

Proceed as follows (tested with TERM v9.98):

- Switch off character echoing by using the 'E0' command. This

version off TFPCX will always run in 8 bit mode (transparent) so

the command '@M' is not longer supported.

- start the BIN reception using key combination ALT-U and answer the

questions (read command as last).

- restore character echoing at the end of the transfer by means of

the 'E1' command.

For more information read the TERM documentation.

7.5. CT (K1EA)

The  contest  program  by  K1EA  also  supports  TFPCX (tested with CT

v6.26).   During  CW-Contests  you  can  only  use  KISS or a SCC Card

because CT will  otherwise have problems  with the correct  generation

of morse code.

The options  '-DR' and  '-IFF' are  required when  starting TFPCX.  It

also makes  sense to  set up  the parameters  in TFPCX  using the '-F'

option.

In the first dialogue box, the item 'TNC' shall be set to 'Local'  and

when using a modem the 'Mode'  should be set to 'SSB'. No  adjustments

are requiredi  n the   communications setup  (CT will  not access  the

modem-port  itself).  The  command  'DRSI'  should  be  entered in the

QSO-screen, and then by using key combination ALT-T the TALK mode  has

to be selected, and ENTER has to be pressed once. Now you can  connect

to the  DX-Cluster and  using key  combination ALT-A  will display the

DX-Information in the  opened ANNOUNCE-Window. You  will have to  find

out the rest  for yourself.   It is advisable  to set CT  up as Single

Unlimited  since  the  Single  Operator  setting  will  prohibit  some

options.

7.6. DIEBOX (DF3AV)

You can use DIEBOX with TFPCX by using the TFPC-Interface.  Of  course

you  can  hardly  run  a  Box  with  only  one  simple modem, but this

operation makes sense  with a SCC-Card  .  You  can also use  the KISS

mode to  establish a  direct connection  to a  digipeater (e.g. RMNC),

this setup will save a TNC.

TFPCX  should  be  started  without  the  '-DR'  option.  I don't know

whether or not the '-DM' option will work: you have to check this  out

yourself.   It  makes  sense  to  increase  the number of the channels

(option '-CH')  and the  number of  buffers (Option  '-BU'). I can not

give more information about  the configuration of the  DIEBOX software

because I have not tested this myself.

7.7. WINPR (DG6BI)

WINPR version 2.0 supports TFPCX via the TFPC- Interface. Since  WINPR

runs under Microsoft Windows it only works on a fast computer using  a

KISS  TNC  or  a  SCC-Card  (see  Section 7.10.). Multiport use is not

supported by WINPR. The option '-DM' should not be used with WINPR.

TFPCX should be loaded before  Windows starts up, or started  from the

WINSTART.BAT file. Windows has to  run in 386 Enhanced Mode.  The File

WINPR.INI requires the settings:

Port = Com5

TfpcrInq = 253

7.8 TOP (DF8MT)

The TOP documentation describes  how to use TFPCX  so I will stick  to

the items which  are new.   For multiport operation,  TFPCX has to  be

started with the '-DX' option instead of '-DR' option/

Under TOP,  the different  TFPCX-ports are  handled as  separate TNCs,

and each channel is directly  allocated to a single port  in sequence.

That is why a  connect command does not  need a port-number. This  may

seem  fine  but  up  'til  now  it  did not work consistently, connect

requests from ouside just used  the lowest available channel with  the

correct MYCALL, regardless of the channels-to-port allocation in  TOP.

Using the command '@PO' (see section 8.2.) will fix this problem.

Example:

You want to use  TFPCX with 2 ports  and 20 channels with  10 channels

on each  port.   In addition  to the  setup of  TNC Configuration, you

should enter the following INI-Command in TOPSET:

@PO 00000000001111111111

In this case you should   load TFPCX using the '-CH20' option  because

the default number of channels is only 10.

7.9  FBB (F6FBB)

The  F6FBB-BBS-Software  supports  both  TFPC-  (from v5.15) and DRSI-

Interfaces. Since  the DRSI  multiport interface will be used  in most

cases, I  will elaborate  on that.   For modem  operation you  need at

least FBB v5.15 or better.

TFPCX should be loaded with the '-DR' and '-IFF' options. If you  need

to support more  than 10 channels,  the '-CH' option  is also required

(see  section  6.2.3.).  When  using  2  ports, the configuration file

PORT.SYS will look as follows:

# PORT.SYS for FBB 5.15

#

#Ports TNCs

1     2

#

#Com Interface Address (Hex0) Baud

7   4         0              4800

#

#TNC NbCh Com MultCh Pacln Maxfr NbFwd MxBloc M/P-Fwd Mode Freq

1   5    7   0      230   2     1     10     00/60   UDYW 144.650

2   5    7   1      230   2     1     10     00/60   UDYW 438.458

TFPCX simulates  2 TNCs  via a  virtual COM-Port  (COM7). Interface  4

means DRSI: the address and  baudrate will be ignored, but  these have

to appear in  the file. The  sum of the  number of ports  allocated to

the channels  (NbCh) shall  not be  greater than  the total  number of

available  TFPCX  channels  (given  with  option -CH or otherwise 10).

There  is  however  only  one  INITTNC1.SYS  or MAINT1.SYS file needed

which  will  set  all  the  needed  parameters  and port settings (see

Section 8.1) for all ports.   The 'P'-command in TFPCX cannot be  used

to set  all parameters  of a  port simultaneusly  like the DRSI-TNCTRS

driver can. For more information refer to the FBB-Documentation.

7.10. MS-Windows 3.1x, OS/2 and others.

When using a  modem TFPCX requires  a very low  interrupt latency (see

apendix 2) which  can not be  accomplished when running  under Windows

3.x or IBM OS/2  2.0 (although there are  a lot of people  who find it

hard not accept this fact). That  is why TFPCX will generate an  error

message if  an attempt  is made  to start  TFPCX for  modem use  while

running one of these systems.

Operating an SCC-Card using these systems is possible.   Actually,this

works  quite  well  when  using  1200  baud  under Windows. When using

higher baud-rates runnig SCC cards will also cause problems.   Running

OS/2 will  not work  as well  as running  under Windows,  but this was

only tested  during a  short test.  I have  no idea  how well a PAR96,

PICPAR or YAM96 modem will work under these systems.

KISS mode works in my setup on both systems up to at least 38400  baud

without any problem. The COM ports are fully supported by Windows  and

OS/2 drivers,  which is  unfortunatly not  the case  for the SCC cards

(which is of course reasonable).

When  using  other  multitasking  systems  you  may  find  a   similar

behaviour.

8. Operation

In  this  section  the  most  important  differences and extensions in

TFPCX compared to the TNC-Firmware TF 2.7b. will be explained.

8.1. Multiport extensions.

The most  significant extension  is without  any doubt  the option  of

using  more  than  one  port  simultaneously, which enables control of

multiple modems and  tranceivers.  If  you only use  one port you  can

skip this section and move ahead to next section.

TFPCX-Firmware can manage up to 8 ports. I think that using them all

will only occur in exceptional cases. The unused ports can be used

for internal connects. This will enable you to check your CTEXT

message without disturbing the Digi uplink (which seems to be a

popular passtime with some people)

A unique  number (0-7)  is assigned  to each  port. The  assignment of

those numbers follows the exact  same order as the '-P'  options which

are given at the start of  TFPCX. These port numbers are displayed  by

TFPCX in  conjunction with  link status  reports, in  the Monitor, and

when using certain commands ('C',  'L' and 'S'), so you  can associate

the  message  with   a  specific  port.   This  portnumber  is    only

displayed if one  of the options  '-DR', '-DX' or  '-DM' is used  when

starting TFPCX.

Example:

1:fm DG0FT to DB0BLN ctl SABM+

1:fm DB0BLN o DG0FT ctl UA-

CONNECTED to 1:DB0BLN

The exact format of the messages  can be found in appendix 4.2.  Where

the  portnumber  is  displayed  on  the  screen depends on the type of

message, and on the terminal program.

Some  firmware-commands  (see  section  8.2.  and  appendix  1.)   are

extended to accept a  port number so you  can set some parameters  for

one specific port or connect a station on one specific port.

Command Format:

<Command> <Port>:[Parameter]

<Port> is a  digit between 0  und 7, no  space is allowed  between the

digit and the ':'.

Example:

C 1:DB0BLN

T 2:15

If  the  port  number  is  not  specified for a command which normally

needs a port number,  either the port on  which a channel just  became

active is used, or the port  which is specified on monitor channel  as

unproto port is used.  There are, however, hostmode terminal  programs

which do not send  a command to the  same channel as the  channel from

which  the  command  was  originated,  and this will undermine correct

port  selection.  Therefore,  always  specify  the port number when in

doubt.

Most  of  the  times  the  parameter  settings  are  defined  in   the

configuration files  of the  terminal program  and have  to be defined

individually for each port. The  file TFPCX.INI is an example  of this

and can,  once the  personal data  has been  set, be  used directly to

initialise the ports (Option '- F', Section 6.2.3.).

Incoming  connects  are  normally  assigned  to the first free channel

which has an matching MYCALL setting, regardless of the port on  which

the connect  was recieved.   By using  the command  '@PO' (see Section

8.2.) a port can be assigned to use only specific channels.

8.2. Some special command characteristics.

There are a number of commands - which all start by using the  ESC-Key

and which  are executed  by pressing  <ENTER> (see  Appendix 1.).  The

TF27b commands  'A', 'H',  'K', 'Z',  '@F' and  '@K' do  not exist  in

TFPCX.

The Parameters  'B', 'O',  'P', 'T',  'W', 'X',  '@C', '@D', '@T2' and

'@TA' are used separately for each port.

The internal timer works  with an accuracy of  +/- 20 ms or  +/- 60 ms

if KISS or the OptoPcScc-Card  without timer port is used.  This could

be important  for some  parameters .  For TXTAIL  (Command '@TA') this

inaccuracy is automatically taken into account.

The individual commands:

C  Connect

When using  multiple connects  to the  same station  with the selected

SSID already in  use, the SSID  will be incremented  automatically (up

to a maximum  of 15). The  terminal program may  not be aware  of this

and may show the wrong SSID.

You  can  specify  a  port  number  when  using  the C command. If not

specified, the number  will be sought  in the Link-List  (see '@L') or

the  Unproto  port  is  used  if  the  search fails. If the channel is

allocated to a port by means of the '@PO' command, that port will  act

as default  port for  that channel.   On the  monitor channel  you can

select the  Unproto port  by specifying  a port  number, the UI frames

will be transmitted (Default port 0) on this port.

Examples:

C 1:DB0BLN                  Connect DB0BLN on Port 1

C 2:TEST \ Monitor-Channel  Set Unproto-Port to 2 and set Unproto-ID

C 2:     /                  Set only Unproto-Port to 2

F  Frack

The  FRACK-parameter  can  set  in  seconds  or  alternatively in 10ms

units. Values 0 to  15 are considered to  be given in seconds  and are

converted internally to the new 10 ms units.

O  MAXFRAME

MAXFRAME be  set individually  for each  Port and  each connection. At

connect  time,  the  value  specified  for  the port will used for the

connection and can be changed independently from other connections  on

the same port.

P  P-Persistence

Under  DAMA-operation  the  non-DAMA-Value  will  be  displayed,   but

actually P=255 will be used .

If the indicated  parameter has a  value between 0  and 7 (Portnumber)

the  P-Value  will  not  be  set  but  instead  some parameters of the

indicated port will be displayed using the following format:

<Port> R P W F O N @T2 @T3 T <Baud> @D

This had to be built-in to keep TFPCX compatible with the  DRSI-TNCTSR

driver when it is  used by SP.   <Baud> is the configured  baudrate in

readable text formal, not like TNCTSR which uses a designated number.

The DRSI-driver also  uses this command  to set the  P-parameter, this

will  not  in  work  TFPCX.  TFPCX  will  ignore the entire command if

additional values are given following the port number.

QRES  Reset

QRES simply resets  TFPCX back into  Terminalmode (like 'JHOST0')  and

does not RESET  the computer. This  command is needed  because SP uses

it when the DRSI-Command 'HB' is executed.

U  Unattended Mode (CTEXT)

This comamnd is  mainly important when  using TFPCX in  terminal mode.

With setting 'U0' link-status messages are sent out immediately,  even

if another channel is active  at that moment. With setting  'U1' these

messages kept and only displayed when the appropriate channel  becomes

active by means  of the 'S'  command. This setting  will also cause  a

readable text to be send back when someone connects from outside.

The default setting 'U2' behaves almost the same as setting 'U1',  but

additionally it allows disconnection  of a connected station  by means

of the remote command '//Q' (or '//q'). This command has to appear  at

the start of a frame (otherwise it will not be handled).

You can also switch unattended mode  on if no CTEXT has been  defined.

The only effect of this command in host mode this the handling of  the

connect text (CTEXT).

@C  DCD-Working

TFPCX has a Soft-DCD  (software squelch). When using  slow tranceivers

(slow meaning  a slow  squelch cirquit)  you can  leave the squelch of

the receiver fully  open. TFPCX will  determine for itself  whether or

not a packet signal is being received.

The  Soft-DCD  can  be  controlled  by  means of the '@C' command, the

given parameter controlls the noise-cancelling level.  This  parameter

can be a  number between 0  and 63. When  using '@C0' the  Soft-DCD is

switched off (default)- all other  values activate the Soft-DCD.   The

lower the  value the  more unstable  and faster  the Soft-DCD becomes,

the  higher  the  value,  the  more  stable and slower it becomes. The

ideal DCD should be both  stable and fast. Therefore the  best setting

will good compromise between the two extremes. To simplify the job  of

finding the  right value  you can  use the  DCD indicator  (see option

'-C').   When  using  a  small  value  you  will  see the unstable DCD

indication, when using a large value you will see slow and  inaccurate

recognition of  signals.   The best  way to  set this  correctly is to

slowly increase the parameter value while listening to the signals  on

the PR QRG,  until the indication  is correct. A  good start value  is

'@C25'.

When using  SCC, PAR96  or PICPAR  ports an  adjustment is  not really

needed, all  values above  0 will  activate the  Soft-DCD and they all

have the same meaning for  the operational software.  I  was, however,

annoyed by the constant flickering of the indicator, so you can  still

adjust the value  to reduce the  flicker. For the  YAM96 modem the  @C

command will not work at all since it always uses a hardware DCD.

IMPORTANT!

The soft-DCD  only recognises  PR-Signals at  the same  baud rate. You

can _not_ use the  soft-DCD on packet QRGs  which carry PR signals  at

different baud-rates.

When  using  KISS,  the  TNC  handles  the  DCD, and this command only

defines the time  (in 10ms-Units) after  which the RX  indication will

be removed from the  screen if no further  data was received from  the

TNC.

When you are connected to a  DAMA station the DCD will be  ignored for

the used  port. The  DAMA station  will regulate  the traffic and will

ensure a free QRG when it  sends a DAMA-poll to you, so  TFPCX doesn't

need to double-check this. The advantage is a very fast response  from

TFPCX, which makes the DAMA handling better. This was introduced  with

TheFirmware TF2.7b, and is also used in TF2.7b based TNCs.

@L  Link List

Using this command an internal  link-list is managed which is  used to

allocate up to  8 calls (with  SSID) to a  specific port. The  list is

used for the digipeating function ('R' command) to select the port  on

which the  recieved frame  should be  re-transmitted. If  there is  no

appropriate entry in the  link-list, the frame will  be re-transmitted

to the same port as where it was recieved from.

Syntax:

@L <Port>:<Call> <Port>:<Call> ...

Example:

@L 0:DB0BLN 1:DB0BLO

Using '@L-' will clear  the link list. The  contents of the list  will

be displayed using '@L' without any parameters.

For crossband-digipeating  both source  and destination  calls have to

be present in the link  list, otherwise the connection will  only work

correctly in one direction.

@PO

By using this  command you can  define for each  channel if it  can be

used for all ports, only for a  specific port or if it cannot be  used

by any port.   If a channel is  set-up to use one  specific port, this

port will also be used as the  default port if a 'C' command is  given

on this channel.

As  parameter  a  string  is  passed.   The first character belongs to

channel 1, the second to channel  2 and so on. When using  10 channels

the  string  will  normally  consist  of  10  characters. When a lower

number of characters  are given the  allocation of remaining  channels

are not changed. When more  characters are given the extra  characters

are ignored.   The following  character can  appear in  the  parameter

string:

'0' to '7' Connect only possible from one port (Port Number)

'*'        Connect possible from all ports

'-'        No connect from outside possible

Examples:

@PO 0000011111*****-----

Channels 1  to 5  can only  be accessed  from outside  though port  0,

channels  6  to  18  only  through  port  1.  Channels  11  to  15 are

accessible  by  every  port  when  someone  connects  from outside and

channels 16 to 20 are not accessible from outside at all and can  only

be used for outgoing connections.

@PO **********

All  channels  can  be  accesed  from  all  ports: this is the default

setting and is compatible with earlier TFPCX versions.

Incomming connects are  assigned to the  lowest free channel  to which

the port is allowed to  connect (the channel either has  been assigned

to this port or can be connected  by all ports ('*')) and on which  an

appropriate MYCALL  is present.   If there  is no  appropriate channel

available the connection will be refused (BUSY).

@ST  Statistics

By using 'ST  <Port>:' some statistical  values will be  displayed for

the given  port. These  values are  gathered for  all connects on this

port. Be aware that the counters will wrap to 0 after reaching 65535.

Example:

0 SCC0 TX 87 11 10 RX 547 201 201 ERR 1

^ ^       ^  ^  ^     ^   ^   ^       ^

1)2)      3) 4) 5)    6)  7)  8)      9)

1) portnumber

2) interface port ('NULL', for internal Port)

3) total number of transmitted frames

4) number of I-frames sent

5) number of acknowledged I-frames ( 4)-5) are lost )

6) total number of received frames

7) number of received I-frames

8) number of effectivly received I-frames ( 7)-8) REJects )

9) number of errors occurred (will be shown for SCC and KISS and

only if it isn't 0)

It  is  possible  to  do  some  statistical calculations and draw some

conclusions by using  these values .  The interface port  name 2) will

have  a  '+'  appended  to  the  name  if  KISS  is  used and SMACK or

RMNC-CRC-KISS is active.

When using SCC cards value #9 shows the number of over- and  underruns

of the  SCC controller,  which will  occur if  the interrupts  are not

serviced fast enough. When using KISS the counter will be  incremented

each time  a byte  is lost  or when  a KISS  frame or CRC error occurs

(with SMACK  or RMNC-CRC-KISS).  When you  notice a  rapid increase of

the number of errors your PC  might be too slow for the  used baudrate

or, when using KISS, the connection to the TNC is not okay.

You can clear  the statistics by  means of the  '@ST <Port>:-' command

(for the indicated port).

@TA  TXTAIL

You  can  specify  the  TXTAIL  value  in 10ms units (0-6000) but this

value  is  in  normal  cases  (for  the  selected  baudrate  and timer

inaccuracy)  already  set  to  the  most  optimal value (@TA=4 for 300

baud, @TA=1 otherwise). When using  KISS the correct value depends  on

the used TNC, therefor there it is not set automaticly in this case.

@U  Unproto-Poll

By using  this command  you can  specify if  unproto-frames should  be

sent with (Default) or without a set poll-bit.

8.3. KISS

When using KISS there are some special things you'll need to know.  We

will elaborate on this in this section .

Before you start TFPCX the TNC has to be switched on should have  been

set to KISS mode. TFPCX does  not offer the option of putting  the TNC

into KISS mode. The TFPCR command  @K does not exist in TFPCX.  To set

the TNC to KISS mode you  can use the program SETKISS (which  sould be

in the package from which included TFPCX).

TFPCX  supports   the  KISS-enhancements   SMACK  (Version   1.0)  and

RMNC-CRC-KISS,  which  both  improve  the  reliability  of   transfers

between the TNC and the  PC. SMACK or RMNC-CRC-KISS will  be activated

automatically if the connected TNC  supports it. By using the  command

@ST (see section 8.2)  you can check if  one of these enhancements  is

activated (a  '+' sign  is shown  if SMACK  or RMNC-CRC-KISS is active

e.g. 'COM1+'). SMACK or RMNC-CRC-KISS  is only activated when least  1

Frame has been sent and received.

With KISS, the Send-/Receive indicator does not show the  send/receive

status  on  the  radio  channel,  but  the  send/receive status on the

serial port to the TNC or to a connected other PC.

To couple two PCs together (e.g. one running a digipeater and  another

running a mailbox) you need a null-Modem-Cable. When using this  setup

TFPCX always  works in  duplex mode  (the command  @D is meaningless).

The parameters sould be setup accordingly. The following  descriptions

are  mainly  important  for  normal  use  with  a TNC (no direct cable

connection).

In KISS-Mode,  TFPCX has  no direct  control over  the radio  channel,

because the  TNC is  in between.  TFPCX has  no possibility  to detect

whether or not the  frequency is free and  when the frames to  be sent

are actually  transferred, which  may lead  to unwanted transmissions.

Two situations are especially interesting.

- Assume TFPCX has transferred  one frame to the TNC  for transmission

and waits for an acknoledgement  from the other station. It  at that

moment the radio link is  occupied for some time by  another station

(for instance  a Digi)  the TNC  will not  be able  to transmit  the

frame. After some time however TFPCX assumes the frame was lost  and

starts a retransmission, but in  reality the original frame was  not

transmitted at all at that time.  The result of this is an  unwanted

duplicate transmission of a frame.

The time beween sending a frame and reception of the  acknoledgement

is measured by TFPCX and it  will 'learn' how long on avarage  TFPCX

has to wait before receiving  a response. This mechanism will  adapt

itself to the  traffic on the  readio channel and  avoids the double

transmissions. In  TheFirmware 2.7b,  the parameter  @A3 (with which

you could manipulate the delay in earlier TF versions) is fixed  and

cannot be adjusted anymore.

- When  more  frames  are  received  in  a  row, all these frames  can

normally be acknoledged with  a single response frame.  The question

is, when can TFPCX assume no  further frames will be sent so  it can

generate the  single response  frame? When  using a  serial modem or

SCC Card, TFPCX can wait until the other station stops  transmitting

and the radio channel is free again.

This  method  is  not  usable  anymore  when using KISS. The correct

setting of  parameter @T2  is very  important ehen  using KISS.  @T2

specifies  the  time  the  receiver  of  the  frame is going to wait

before a response  is generated. If  a new frame  is received during

this time,  the wait-time  is starts  all over  again. The wait-time

should only expire if no further  frame is going to be received,  so

all received frames  can be acknowledged  using one response  frame.

If  the  wait-time  specified  with  @T2  is too short then unneeded

response frames will be send (wasting time and channel occupation).

Which value should @T2 have?  The wait-time should be just  a little

longer with respect to the transmit-time of one frame from the  peer

station. The following times are derived from the used baudrate  and

can be used a guideline:

Baud    @T2

1200    250

2400    150 (Default)

4800    100

9600     50

These times are valid when using  256 data-bytes in a frame and  are

a bit on the  high side. If you  want to experiment with  this value

you should watch the monitor to see if sequentially recieved  frames

are acknowledged with only  one RR-frame. It is  probably acceptable

if  more RR-frames are sent occasionally.

IMPORTANT

The default-value  @T2=150 is  too short  to use  for 1200 baud. You

should certainly use a larger delay setting for @T2 if you use  1200

baud.

The value @T4,  to be used  as @T2 with  DAMA, which was  present in

previous versions of TFPCX, does not exist anymore. For DAMA @T2  is

not  needed at  all since the response  can be generated as  soon as

the DAMA  station polls  for a  response (result  of the  new TF2.7b

firmware used in TFPCX).

8.4. DAMA

As soon as a connection  is established to a DAMA-master  (digipeater,

box or bbs), TFPCX will only transmit if it receives a frame from  the

master  with  the  request  to  respond.  TFPCX  will send all pending

transmit-frames for all channels which are using the same port as  the

port on which de DAMA  connection exists. The DAMA-slave mode  is only

activated on the port which carries the connection to the master.  The

connections using other ports will  run normally without DAMA. When  a

connection to another  DAMA master exists  using another port  it will

handle  the  channels  using  that  port  independently  from the DAMA

connection  on  the  other  port  (i.e.  for  DAMA  all  the ports are

independent of each other).

You don't  need to  change any  parameter setting  for DAMA. This will

enable you  to use  the same  port also  for non-DAMA  connections. By

looking at  the monitor  you can  see '[DAMA]'  appended to the frames

from the DAMA master. If the  frames are sent to you, TFPCX  will also

have  switched  to  DAMA.  The  'B'  command which existed in previous

versions of  TFPCX does  not exist  anymore (TF2.7b  uses this command

for another purpose).

This version  of TFPCX  changed to  the DAMA  handling of TF2.7b. This

means TFPCX now supports the latest version of the DAMA protocol.  All

previous versions of TFPCX did not. In fact this was one of the  major

reasons to upgrade TFPCX to the latest TheFirmware version. This  also

means the layer 1 DCD is ignored when connected to a DAMA master.  The

DAMA master ensures  a free QRG  so there is  no need to  double-check

it. This will ensure a fast response on a DAMA request. This  solution

may lead to  problems for KISS  however, since TFPCX  does not control

the DCD in  the KISS TNC.  If the KISS  TNC has a  good DCD it  should

work  anyway  because  the  QRG  is  clear  after  being polled by the

DAMA-master.

Appendices

1. Overview of Commands

Command        Parameter       Description

------        ---------       ------------

* C            Call ...       Connect-route

Connect on channel 0 sets port and name

and name for unproto messages

D                           Disconnect

E(1)          0             No echo of input characters

1             Echo input characters

F (250)      1-15           Start value for T1 (in seconds)

16-65535        Start value for SRTT (10ms)

G            [0]            Obtain information in hostmode

[1]            Obtain status in hostmode

I            Call           Set own call sign

JHOST (0)     0             Change to terminal-mode

1             Change to hostmode

K             0             No timestamp

1             Timestamp on status messages

2             Timestamp on status and monitor messages

hh:mm:ss         Set TNC time

dd.mm.yy         Set TNC date (european style)

mm/dd/yy         Set TND date (american style)

L           [0-10]          Status display of a channel, if no

channel given, displays all channels

M (N)       NIUSC+-         Monitor-Operation Type

N (20)       0-127          Number of Tries (0 = unlimited)

* O (2)        1-7            Maximum number of unacknowledged

packets allowed

* P (32)       0-7            Parameter request for a port

8-255          P-Persistence value for non-DAMA

QRES                        Reset into terminal-mode

R (1)         0             Digipeater function off

1             Digipeater function on

S (0)        0-10           Select channel (number, 0 = unproto)

* T (25)       0-127          Time between PTT on and frame start

(10ms)

U (2)        0              Suppress connect text

1 [Text]       Connect text active

2 [Text]       Connect text and remote-//Quit active

V                           Display TF version string

* W (10)       0-127          Time slot for P-Persistence (10ms)

Ignored for DAMA

* X (1)         0             PTT switching off, TX suppressed

1             PTT switching on, TX allowed

Y (10)       0-10           Maximum number of connections

@B                          Shows number of free buffers

* @C (0)        0             Software-DCD off

1-63           Threshold value for software-DCD

* @D (0)        0             Full duplex off

1             Full duplex on

@I (80)       0             IPOLL off

1-256          maximum Length of an IPOLL-Frame

@L           Port:Call ...  Linklist setup (max. 8 Entries)

'-'            Linklist clear

@PO ('*')    cccccccccc     Port assignment (each channel one

character)

c = '0'-'7'    Connect only possible from port c

'*'        Connect from all ports possible

'-'        No connect from external stations possible

@S                          Current link state

* @ST                         Status indicator per Port

'-'            Reset status counters

* @T2 (150)    0-65535        Timer T2 (10ms)

@T3 (30000)  0-65535        Timer T3 (10ms)

* @TA (1/4)    0-65535        Time between frame end until PTT off

(10ms)

@U  (1)       0             Unproto-frames without Poll

1             Unproto-frames with Poll

@V  (0)       0             Call sign check switched off

1             Call sign check switched on

*   Parameter <Port>: possible

[]  optional Parameter

()  Standard installation

... additional Parameters possible

A lot of these  commands can be used  without parameters to display  the

current setting for the command.

2. Error Remedies (Modem Operation)

When you  have a  problem you  should first  find out  what could have

caused it. Besides TFPCX, it could be caused by the Terminal  program,

the modem,  the SCC  Card, DRSI,  PAR96/PICPAR or  YAM96 modem, or the

tranceiver.

This section focuses mainly on use with a modem.

TFPCX demands more from your PC  compared to the use of a  normal TNC.

When your PC cannot live up to this demand you will be in trouble.  To

understand  this  I  will  explain  how  TFPCX  works when sending and

receiving.

When  using  packet  radio  the  information  is  transmitted  using a

synchronous serial link.  The PC's serial  port can, when  used in the

normal way, only transmit asynchronous serial information using  start

and stop-bits which  do not exist  with packet radio.  The serial port

can therefore not be used in  the normal way. This means TFPCX  has to

handle each and every  bit itself: the serial  port is only used  as a

simple latch which can store only one bit.

To enable  TFPCX to  handle the  data in  a predefined  speed of,  for

example, 1200 bits/s it needs a accurate clock. To transmit the  clock

has to delever 1200 ticks/s. The  method used to receive data needs  a

clock  which  can  deliver  3600  ticks/s  which  makes it possible to

constantly synchronise on the  recieved signal. TFPCX uses  a software

PLL. The modem RX line is sampled 3 times per bit to detect if the  RX

line changed from a logical 1 to  0 or 0 to 1. In the  ideal situation

such  a  change  should  only  occur  on  every  third  sample. Due to

inaccuracies the timing drifts away with respect to the sender.  Since

three samples  are taken  per bit  the direction  of the  drift can be

detected and compensated.

As accurate clock I used the  PC's internal timer which is present  in

any  PC  and  is  normally  used  for  the  date/time  in  DOS.  TFPCX

re-programs  the  timer  to  supply  an timer-interrupt 3600 times per

second.  The  interrupt  handler  (the  so  called  Interrupt  Service

Routine, ISR)  will take  control over  the CPU  and suspend what ever

was executing at  that moment and  will call the  functions which take

care of  sending and  recieving. It  is inevitable  this can only work

correctly if  all the  interrupts are  handled in  a steady repetition

and  without  too  much  delay.  This  will  be a problem when running

Windows or OS/2,  that's the reason  why it does  not work right  when

using these systems.

If you compare the load on the  PC caused by TFPCX compared to a  TNC,

TFPCX will cause a 30 times higher load when using the same  baudrate,

using 1200 baud it can be compared  with a 36000 baud TNC on your  PC,

which gives problems on a lot  of slow PCs. Luckily enough for  us,PCs

are  getting  faster  and  faster,  so  this  has  become  a less of a

problem  over  the  years.  The  timing  inaccuracies are causing most

problems nowadays  when TFPCX  doesn't run  under plain  DOS. This  is

still a big difference between using TFPCX or using a TNC.

2.1. Send- and Receive Problems

The used  PC should  in the  first place  be capable  of handling  the

large number of interrupts.  When your PC is  not able to handle  this

it  will  run  very  slowly,  or  your  PC will lock-up or crash. From

experience  the  following  table  may  used  as  a guideline (without

guarantee):

PC    XT   XT   286  386

MHz   5    8    12   20

Baud

300  *    *    *    *

1200  ?    ?    *    *

2400  -    ?    *    *

4800  -    -    ?    *

*  Operation possible

?  Operation probably possible (with Limitations)

-  Operation impossible

As you  see, a  modern PC  with at  least a  486 or  Pentium processor

should not cause any problems with processing speed any more.

There are,  however, also  problems with  PCs which  are in  principle

fast enough. Most of the  time the installation suffers from  unstable

reception, which leads to many  REJects, which in turn will  result in

many retransmissions.   This is in  most of the  cases caused by  some

residential program,  driver or  additional hardware  which delay  the

interrupts handling  too long.  When the  timer interrupt  handling is

delayed for more than 200�s during the reception of a frame the  whole

frame will be lost (even if it occurs only once). When you  experience

these kind of  problems you could  start TFPCX using  the '-D' option.

When you hear interruptions of the tone you hear from the PC  speaker,

you experience this problem.

Common sources of problems:

- Usage  of Extended  (XMS) or  Expanded   Memory (EMS)  as buffer for

the terminal program  (e.g. SP, GP)  and disk-caches (especially  on

a 286 PC)

To avoid this you should prevent usage of this RAM as long as  TFPCX

is used. When using SP  you should not use SPO.EXE  and XMS-Swapping

('SWP=1'  in  CONFIG.SP).  GP  should  be  started  with  the option

'/NOXMS'.

- A   driver, which  enables resident  programs to  be loaded  in high

memory (e.g. EMM386)

If the  previous solution  did not  work for  you, you could disable

the whole EMM386 driver while using TFPCX.

- Slow Keyboard-Driver (KEYB)

If  frames  are  always  lost  when  a  key is pressed you could try

another keyboard driver (like CKEYGR.COM from the SP-Disk).

- VGA-cards and HD-controllers

Many  VGA-Card  disable  interrupts  for  some  time when running in

graphic mode. I also heard  there are HD controllers which  give the

same  kind  of  problem.  When  the controller was removed everthing

worked again. It is a problem for me to give a general solution  for

this. You could try a program which does not run in graphic mode.

- Running under Windows

Although you can not start TFPCX in a DOS session under Windows  for

use with a serial modem, you can start TFPCX in DOS before  starting

windows. Some  people have  reported this  works for  them, but most

users will get  nothing but trouble  using this setup.  Windows will

delay the timer  interrupts to much  to make a  reliable connection.

The only solution is to quit using TFPCX this way.

Sometimes you have to live with a compromise to use TFPCX. If you  are

not willing to accept this then TFPCX  is not the right way to go.  If

you wonder  why TFPCX  stopped working  all of  a sudden  you have  to

question if you loaded  a new driver or  changed anything else in  the

setup of your PC. Something I  do NOT want is somebody using  TFPCX on

a Digi QRG with a setup which only receives a frame correctly after  3

retries each time.

2.2. Problems with other Programs

While TFPCX is  active you should  not run programs  which use the  PC

timer which is in use by TFPCX.  If you do your system may crash,  run

extremly slowly,  or the  DOS clock  will not  run correctly  anymore.

Among these programs are:

- MS-Word 5.0 and 5.5

- EDIT in MS-DOS 5.0

- MS-Windows

- many Mouse Drivers

Using the following  programs also gave  problems, the exact  cause is

unknown:

- Keyboard  driver  from  DR-DOS  6.0  (keyboard hangup), use  another

drivers (e.g. CKEYGR.COM, which was distributed with SP).

- Microsoft mouse driver (MOUSE.COM). Cure: use another driver

- IBM VCPI.SYS-Driver (used in notebooks), removal of the driver might

solve the problem

2.3. Hardware Problems

There are PCs (especially Laptops), which do not have 100%  compatible

serial ports. The demands on the ports are not so high as for  BayCom,

when the deviations are too big  it can also cause problem with  TFPCX

on these  computers. In  many cases  receiving works  buy sending does

not. Up till  now I received  some reports of  this for the  following

computers:

- Toshiba 1000XE

- NEC Multispeed

- Olivetti M24

TFPCX offers the posibility  to use the modem  on the LPT port,  which

might be an option to bypass the problem.

Most  laptops  and  notebooks  have  build-in Power Management to save

(battery) power. When the keyboard is not pressed for some time  Power

Management  may  reduce  de  processor  speed,  which  may  reduce the

processing power for TFPCX below a usable level. When using this  kind

of PCs (e.g.  Olivetti Quaderndo) it  is often required  to deactivate

the  Power  Management  (especially  the  reduction  of  the processor

speed). If your PC has enough power even with reduced processor  speed

you can try to leave it enabled.

3. Hardware Connections

3.1. Serial Modems

BayCom-compatible modems can  be used without  changes. In rare  cases

there could be problems which are caused by the more stable supply  of

power to the  modem in TFPCX  compared BayCom. In  TFPCX the TXD  line

remains steady at approximately +12V, the BayCom solution has a  pulse

signal on  this control  line. Therefore  the supplied  voltage to the

modem is a little higher, the voltage on pin 7 of the TCM3105 may  not

have the ideal  voltage anymore. In  this case a  re-adjustment of the

voltage on  pin 7  is required  (see modem  documentation). Tip:  this

voltage is  also very  important for  the correct  functioning of  the

software  DCD,  you  can  use  the  Soft  DCD  indicator to adjust the

voltage.

Additionally there  is a  option of  connecting the  modem (e.g.  from

DigiCom)  to  a  LPT-Port.  When  using  this, 6 data output lines are

switched to a stable 5V which  could be used as voltage supply  to the

modem (Use at your own risk)

Here are the connections for the Modem-Ports

COM-Port

Signal   25pol.  9pol.  Meaning

DTR      20      4      Transmit data +/- 12V

RTS      4       7      PTT, High active, -12V=RX, +12V=TX

CTS      5       8      Receive data

GND      7       5      Signal ground

TXD      2       3      +12V for BayCom-Modem

LPT-Port

Signal   25pol.         Meaning

DATA1-6  2-7            constant 5V approx. for Modem

DATA7    8              Transmit data, TTL-Level

DATA8    9              PTT, High active, 0V=RX, 5V=TX

BUSY     11             Receive data

GND      18-25          Signal ground

Modems using the  AM7911 can also  be used. You  may have to  increase

the TXTAIL-parameter (command @TA) a little for this. At this point  I

like to point out you need different modems for higher baudrates.

3.2. BayCom-USCC-Card

The  connections  needed  for  the  USCC  card  can  be  found  in its

documentation. Here  I will  only supply  the numbering  given to  the

ports  and  the  default  setting  of  the  modem  clock  supply   and

baudrate:

Port  SCC  Modem-Clock    Baud  Modem

SCC0  1A   Softclock      1200  AFSK (TCM3105)

SCC1  1B   Softclock      1200  AFSK (AM7911)

SCC2  2A   Disable        9600  External

SCC3  2B   DF9IC-Modem    9600  FSK  (DF9IC)

The second  SCC controller  (Z8530) does  not have  to be present when

the  appropriate  channels  are  not  used,  the  first  controller is

mandatory.  Therefore  you  can  also  use  the  9k6 USCC card (option

-PUSCC:<Base>:<IRQ>:31).

The following  table shows  the exact  clock source  for receive (RxC)

and  transmit  (TxC)  and  also  the  used  encoding  mode. The column

contains the number  to be supplied  with the -PUSCC  option, the last

column shows the equivalent  values for the BayCom  parameters CARRIER

and  HENNING.  Soft-DCD  and  Duplex-Operation  can  be switched on by

means of the commands @C and @D.

-P              RxC  TxC  Mode CARRIER HENNING

1 Softclock    DPLL BRG  NRZI   0/1      0

2 Hardclock    DPLL RTxC NRZI   2-4      0

3 DF9IC-Modem  TRxC RTxC NRZ    1-4      1

BRG   Baud rate generator \ embedded inside the

DPLL  Digital PLL         / SCC-Controller

RTxC                      \ Connections of the

TRxC                      / SCC-Controller

It looks like  some USCC cards  have timing problems  in accessing the

data port  of the  SCC-controller. In  TFPCX v2.00  sometimes nonsense

was  transmitted  because  of  this.  After lowering the bus-clock and

exchange of the  SCC contollers by  original ZILOG types  this problem

could be partly eliminated. TFPCX does not send the transmit data  via

the data port anymore, just like  BayCom does it. I hope this  problem

is now of something of the past.

3.3.  BayCom PAR96 and PICPAR modem

You  can  use  the  PAR96  and  PICPAR modem with TFPCX (since version

2.21). There are some things to note:

The PAR96  and PICPAR  modem are  connected to  one of  the LPT ports.

TFPCX assumes a normal  LPT port is present.  On modern PC's this  can

be an  EPP port  (Enhanced Printer  Port). TFPCX  assumes this port is

set as if it is a normal LPT port at startup. If not TFPCX might  fail

to communicate with the modem (I do not have information how to  setup

the EPP port as normal printer port so I can't do it).

There is  a particulair  problem with  the PICPAR  modem. The power to

the PICPAR  modem is  supplied by  the LPT  port. If  the modem is not

powered  at  the  start  of  TFPCX,  the  initialisation  by  TFPCX is

finished before the power on  the PICPAR is stable. The  communication

to the modem completely  fails in that case.  A way to solve  it seems

to be lowering  the value of  the capacitors on  the power line  or to

switch over to an external power supply.

You must specify the correct IRQ at startup: the default IRQ is 7.  If

you attempt to use the wrong IRQ, the communication to the modem  will

also fail.

3.4.  YAM96 modem

You can  use the  YAM96 modem  with TFPCX  (since version 2.71). There

are some things to note:

Before you can use the YAM96  modem you have to download the  software

into the FPGA by means of the YAMINIT program you got with the  modem.

If this  is not  done before  TFPCX starts,  TFPCX will  give an error

message.

The YAM96 modem is connected to one of the COM ports. TFPCX assumes  a

normal COM port is present. When no port address is given TFPCX  tries

to retrieve this address  from the BIOS area.  When using COM ports  3

or  4  this  port  address  maybe  invalid.  In  that case you have to

specify these values yourself.

You must specify  the correct IRQ  at startup:   the default IRQ  is 4

when using COM1 or COM3, 3 when using COM2 or COM4. If you attempt  to

use the wrong IRQ, the communication to the modem will fail.

Also be aware not to  use COM1 and COM3 or  COM2 and COM4 at the  same

time. If your mouse is connected  to COM1 you should not use  COM3 for

example.  You can use this combination however if COM3 is assigned  to

another  IRQ  (so  in  fact  there  should be only one active COM-port

using the IRQ at the same time).

4. Information for Software Developers

4.1. Program-Interface

The  communication  with  TFPCX  is  done  using a software-interrupt.

There are  several sub-functions  which are  selected by  the value in

register AH. Parameters are passed in the AL register when needed.  On

return register  AX will  hold the  result or  0xFFFF when  an unknown

sub-function  was  selected.  All  input  characters  have  to be read

before  the  output  can  be  send  again.  TFPCX supports 2 different

interfaces which only differ slightly.

4.1.1. TFPC-Interface

Sub functions:

AH = 1  Check, whether a character is available on the input

Returns :   AX = 0  No character available on the input

AX = 1  Character available on the input

AH = 2  Character input (call only if sub function 1 reported

the availability of a character on the input)

Returns:    AL      Character code

AH = 3  Output a character

Parameter:  AL      Character to output

Three  bytes  behind  the  entry  of  the  TFPCX  interrupt  routine a

recognition string 'N5NX' is available, which can be used to find  out

which interrupt is used by TFPCX.

4.1.2. DRSI-Interface

The implementation in TFPCX is derrived from SP because I didn't  have

a description or a TNCTSR  driver myself. Therefore I could  only find

out the  functions used  in conjunction  with SP.  It is possible that

this interface deviates from the original TNCTSR deiver.

Sub functions:

AH = 0  Input of a character

Returns :   AH = 0  No character available on input

AH = 1  Character available on input

AL      Character code (only if AH = 1)

AH = 1  Output a character

Parameter:  AL      Character to output

The Interrupt-Routine starts with the following Bytes:

0x53 0x1E 0xBB 0x?? 0x?? 0x8E 0xDB 0x84 0xE4 0x74 0x20

You can find  out which interrupt  is used by  TFPCX by comparing  the

start of the  interrupt routines for  vectors 0x40 till  0xff with the

byte sequence above. The byes noted as 0x?? can have value and  should

be  skipped  in  the  comparison.  This  method  also  works  with the

DRSI-TNCTSR driver and is also used by SP.

4.1.3. Special Functions

The following sub functions are extensions which only exist in  TFPCX.

They are available for both interface methods mentions above.

AH = 0xFB  Request number of Ports and Channels (from v2.00)

Returns:   AL       Number of used ports (0 to 8)

AH       Number of available Channels (4 to40)

The number of channel is defined by the option '-CH'

AH = 0xFC  Request the send-/receive status (from v2.00)

Returns:   AL       Receive status (Bit-Nr. = Port)

AH       Send status

By using  this functions  a send/recieve  indicator can  be

build in the terminal  program. The option '-C'  only works

in text-mode and  the location is  fixed (which may  not be

the  optimal  place  for  it  in the terminal program). For

every port uses one bit of  AL and AH. Bit 0 (LSB)  belongs

to  Port  0,  Bit  1  to  Port  1  and  so on. Something is

recieved (DCD) or  send on the  port if the  respective bit

is set.  During duplex-operation  both bits  can be  set at

the  same  time.  The  Function  0xFB returns the number of

ports, which should be indicated.

AH = 0xFD  Request TFPCX busy status (from v1.11b)

Returns:   AX = 0   Busy (free Buffer < 88)

AX = 1   not Busy

This  sub  function  enables  the  implementation of a send

handshake in the terminal mode.

AH = 0xFE  Request for the TFPCX-Version (from v1.01)

Returns:   AH = 2   Main Version Number

AL = 0   Lower Version Number (no BCD)

This sub function makes  it possible to make  a destinction

between  TFPCX  and  TFPCR   (delivers  AX  =  0xFFFF)   or

DRSI-TNCTSR (delivered in a test  AX = 0). You should  test

whether 1<=AH<=20 is  true and only  in that case  conclude

it is TFPCX. In addition  you can use this sub  function to

check  if  an  old  TFPCX  version  is  used which does not

support a certain function.

4.2. Format of Reports

In this  section the  messages of  TFPCX are  presented which  deviate

from the  TNC-Firmware. Most  deviations are  just the  addition of  a

port  number  to  the  message.  Another  deviation has to do with the

display of  FlexNet frames.  <Port> is  a digit  between 0  and 7. The

<Port>: part  is only  displayed if  TFPCX is  started with the option

'-DR',  '-DX'  or  '-DM'.  The  monitor  messages  with '-DR' are kept

compatible to the DRSI-TNCTSR driver.

- Link-Status

BUSY fm <Port>:<Call> via <Digis>

CONNECTED to <Port>:<Call> via <Digis>

LINK RESET fm <Port>:<Call> via <Digis>

LINK RESET to <Port>:<Call> via <Digis>

DISCONNECTED fm <Port>:<Call> via <Digis>

LINK FAILURE with <Port>:<Call> via <Digis>

FRAME REJECT fm <Port>:<Call> via <Digis> (x y z)

FRAME REJECT to <Port>:<Call> via <Digis> (x y z)

- Monitor (with options '-DX' and '-DM')

<Port>:CONNECT REQUEST fm <Call> via <Digis>

<Port>:fm <Call> to <Call> via <Digis> ctl <Name> pid <Hex>

- Monitor (with options '-DR' and TNCTSR)

<Port>:CONNECT REQUEST fm <Call> via <Digis>

<Port>: fm <Call> to <Call> via <Digis> ctl <Name> pid <Hex>

^

extra space character

- Response of the 'C' command if used without a parameter

<Port>:<Call> via <Digis>

- Monitor output when receiving FlexNet frames with header

compression

Format of the Monitor-header:

<Port>:fm #<Qso> to <Call> ctl ...

<Port>:fm <Call> to <Call> ctl SABM+ #<Qso>

<Port>:fm <Call> to <Call> ctl UA- #<Qso>

<Qso> = FlexNet QSO number

4.3. Extended Host-Mode

The extended hostmode (DG3DBI) is  a compatible extension to the  'G'-

command, which  it enables,  using a  global query-command,  to gather

information  about  all  channels  of  the  firmware  on which data is

pending to  be read  next. The  use of  the extended  host mode  is an

inprovement,  especially  if  you  are  using  many channels with very

different data traffic loads: the  exchange of data between TFPCX  and

the terminal program will be improved.

The global  query uses  the 'G'  command on  virtual channel 255 (byte

sequence: 0xFF 0x01 0x00  'G'). Parameters supplied with  this command

are  ignored.  TFPCX  will  respond  with  a  0 terminated list of all

channels,  which  have  buffered   data  (byte  sequence:  0xFF   0x01

channel+1 ... 0x00). Channel+1 ...  is a row of channel  numbers which

are increased by 1.

Example:

The channels  0 (monitor  channel), 1  and 5  have data pending. TFPCX

responds with:

0xFF 0x01 0x01 0x02 0x86 0x00

The terminal program  should use this  response to read  the data from

the indicated channels until all  data is read. When all  channels are

empty  the  response  0xFF  0x01  0x00  will  be given on the extended

hostmode command. On the first  global query you should check,  if the

error  message  "INVALID  CHANNEL  NUMBER"  is  generated,  which will

always be the case  if the extended hostmode  is not yet supported  by

the used firmware.

4.4. Previous Versions

Since v1.00 the following alterations have been implemented.

v1.01

- Pointer about unread information by means of the flashing

square if not in hostmode (can be switched off by -NB)

- Baseaddress of the modem-ports can set explicitly now

- TxD-line on the modem-port (power supply to modem) kept fixed

to +12V and switched off when unloading

- Default TFPCX interrupt now 0xFD (previously 0xFE)

- Version information via TFPCX interrupt (AH = 0xFE)

v1.10

- Transition to TheFirmware v2.3b DAMA (previously TF v2.1c)

- Soft-DCD (Configure with command @C)

- Send-/receive indicator in hostmode (can be switched off by -NC)

- Delay of disk-access during send/receive as intermediate

solution in case of problems (-ND) build in

- Automatic increase of the SSID with connect if the same station

is already connected

- Internal connects possible

- Setting of Frack in 1s- or 10ms-units (command F)

- Unattended mode can be switched on without CTEXT,

Error report 'NO MESSAGE AVAILABLE' removed (Command U)

- 600 Buffer (previously 400)

- Bug cleared, which makes unloading impossible with 486's

v1.11

- Command Z available again (XON/XOFF for TERM)

- LPT-Pins DATA1-6 switched to 5V for modem powersupply

v1.11b (unofficial)

- Function for Send-handshake in the terminal mode via TFPCX-

interrupt for TERM (AH = 0xFD)

v2.00

- Support for BayCom-USCC card and a maximum of 2 modems (maximum

6 ports, internal 8 ports)

- Options -PUSCC and -B extended for USCC-cards and more ports

- Option -NC removed, the DCD-Indicator can be switched on by

-C[xx] (optional screen attribute xx)

- Now 20 channels (previously 10)

- Emulation of the DRSI-TNCTSR-driver, additional software-

interface (-DR)

- With option -DM DRSI-compatible reports, but via old software-

interface

- Output of port numbers in reports

- Parameter B, O, P, T, W, X, @C, @D, @T2, @T4 and @TA are

separated for each port (specification <Port>:)

- Command QRES resets TFPCX to terminal mode

- Linklist for crossband-digipeating (command @L)

- Statistic-Framecounter (command @ST)

- Adjustable TXTAIL-parameter (Command @TA)

- Command P with parameter 0-7 compatible with DRSI parameter

request (no parameter setting)

- Command Z removed again

- 'dynamic' MAXFRAME removed (command O)

- Error report during modem-operation under Microsoft-Windows (386

Enhanced Mode)

- Functions for the retrieval of the numbero of ports/channels (AH =

0xFB) and of the send-/receive status (AH = 0xFC) via the

software-interrupt

- Bugs solved, which lead to buffer overflows during background

use and from time to time to reduntant transmissions.

v2.01

- USCC-Sendproblems solved, through output of the transmit data via

control-port (timing-problems on the data port on a few USCC-Cards)

- Option -BU[nnnn] for the setting of the buffer size (minimum 400,

default 600), -BU without parameters will allocate the maximum

number of buffers.

- The number of connect-channels selectable with option -CHnn (4-40

channels, default 10)

- Command E (Echo) back again (default 1, echo on)

- Command @M for #BIN#-Receive, @M=0 Conversion of control-characters

(Standard), @M=1 transport-mode (for TERM)

- Init-File (Option -F) may contain empty lines and comments

(by using '#' or ';'), ESC automatically created ('^' will be

ignored), conversion of TABs to a space character.

- Remote-Command '//Q' (when U=1 and JHOST=0)

- Frames will be monitored, if more than 256 Buffers are Free

(previously 64 Buffers)

- Error reports on an attempt to operate a modem under OS/2 2.0

- DRSI-Function 1 (character output) returns AH=0 (for compatibility

with the TNCTSR-Driver, which reports in AH, whether characters

are pending for input)

v2.10

- KISS-Mode (incl. SMACK) supported (option -PKISS, max. 4 ports)

- Support for PA0HZP-OptoPcScc-Card (option -POSCC), clock Type 4

(PA0HZP-Port with external Clock) and 5 (PA0HZP-Timer for 75 Hz

timing-reference)

- Baycom-9K6-USCC-card support (2nd SCC-Controller not activated,

if not equipped)

- IRQs for SCC and KISS: 2-5/7, for AT additionally: 9-12/14-15 (2=9)

- Reports (monitor and CONNECT REQUEST) changed for the

DRSI-Interface (Option -DR) (incompatibilities with the

TNCTSR-driver eliminated), new option -DX for previous modified

reports.

- Command @PO for selective port allocation (channel can be

connected only from the allocated port, default-port for outgoing

connects).

- Internal connects (loopback) can be switched off (option -NL)

- TXTAIL (@TA) for modem and SCC set to optimal value depending on

the baud rate and timer-accuracy (@TA=4 with 300 Baud, @TA=1

otherwise), max. value for @TA is 6000 (255 for KISS)

- 0 Ports, if -P not specified (No default-port anymore)

- Option -D is applicable to the previous supplied option -P

- With DAMA before polling also wait for expire of T1 (as TF 2.6)

- Extended Host-Mode (DG3DBI) supported

- Remote-Command '//Q' only with U=2 (Default)

- @ST also clears the ERR-Counter

- NET/ROM-Monitor removed

- Default-Parameters F, N, P, R, T, U, @A3, @I, @T2, @T3, @T4 and @TA

changed.

v2.21

- BayCom-PAR96 modem support added

- KISS now also supports RMNC-CRC-KISS

- BayCom-Digi-SCC card support added (only ports 0 to 3).

- The firmware-command K (Timestamp) can be used now. You have to

supply the -ST option at startup to use it

- The monitor will now show FlexNet frames with header compression.

Command M +/- can be used.

- When using KISS, BayCom-PAR96 and PA0HZP-OptoPcScc card the

internal realtime clock is used for internal timers, this will

result in a more exact time measurement (not under Windows)

- The option -ND is removed.

v2.70

- Other author of the software, taken over from Rene.

- Replacement of TF23b with TF27b. This will give:

- Improved DAMA handling: TFPCX can now be used on DAMA connections

again.

- Framesammler support.

- TNC commands in line with TF27b, some commands are not available

anymore like '@A3', 'B' and '@M' for example.

- When connected to a DAMA master the DCD is now ignored like in

TFX and TheFirmware 2.7b based TNCs (with the latest TF2.7b).

- When using the -ST option on the commandline the DOS-Time will be

used to intitaly set the clock in the TNC. This will give the real

time to the timestamps (K command)

- Everything I broke. This TFPCX just contains a small portion of the

original TFPCX, I had to put all the enhancements into the TF27b

code and may have overlooked something.

- Changed licence condition. Due to use of TF27b code from NORD><LINK

the ALAS is applicable again.

- Windows95 will no longer use 'compatibility mode' after TFPCX

has been in memory.

v2.71

- Added support for YAM96 modem, new -PYAMx command.

- Added -SA option.

- At link-setup time timer T1 (frame-acknoledge timer) did  not run,

this is fixed now.

- Before generating a RR response a check is done if DCD is active.

This allows the use of much  smaller @T2 values.