Welcome to 7pserv V4.0.

This program was created and designed by:

Tore Andersson (SM0NCS)

Mrtstigen 34

151 39 Sdertlje

Telefon: 08-550 97343

Data:    08-550 97325

Arbete:  08-28 76 31

Mobil:   010-667 80 11

=============================================================

###   ######             ####

##### ##########         ######

##################       #######

####################     ######

#####        ##################

####           ################

####             #############              #####

#####                    ######              #####

#####                  #######               #####

######               #######                 #####

######             ########                  #####

####            ########                    #####

########                      #####

#######              #########################

#######               #########################

#######                #########################

#######                           #####

######                            #####

#######                            #####

######                             #####

######                             #####

#######                             #####

#######                             #####

#######

#######

######

####         7PLUS v2.0 - (c) 1991,1992 by DG1BBQ

An intelligent ASCII to binary encoder with enhanced

correcting capabilities.

=============================================================

As you all propably know I came up with the idea to have

a server for encoded files several years ago. The choice of

algorithm came to 7plus and therefore the server where called

7pserv. The server was being used by very many people and

more servers basically on the same idea where created by other

software "designers?!". Since then 7plus has been whidely used

and in many ways become a problem when missused by people who

cannot understand how to use it.

This new version of 7pserv contains many new ideas and is aimed

to solve some of the general problems of today.

I wish to thank SM5NCZ, Bjrn Karlsson for the help of creating

this software.

This document describes the installation procedures for

7pserv V4.0 and how you use it. It is intended as for the Sysops

and not written for the general user. You as a Sysop is urged

to write down in your own language a user manual from the

contents of this manual.

With this program you can send and receive binaryfiles with

your BBS.

ACKNOWLEDGEMENT

This product uses the same algorithm as 7plus.exe a program

by Alex, DG1BBQ and it is built on his ideas and always made

compatible with the latest version of 7plus. The intention

is that users should use 7plus.exe to transmitt and receive

the files. I wish to thank Alex for a great job without his

super ideas this program would propably be based on some

other more or less silly idea.

GENERAL DISCLAMER

THIS PROGRAM 7PSERV.EXE IS HEARBY FREELY DONATED TO THE PUBLIC

DOMAIN, THERE IS NO FEE ATTACHED TO ITS USAGE THE AUTHORS

ONLY CONDITION IS THAT IT MAY BE DISTRIBUTED IN ITS ORIGINAL

UNMANIPULATED FORM ALONG WITH THIS DOCUMENTARY FILE. THE

AUTHOR ASSUMES NO RESPONSIBILITY WHAT SO EVER FOR THE EVENTUAL

DAMAGE THAT MAY BE CAUSED BY THE USAGE OF THIS PROGRAM.

Contributions is much aprechiated to:

SM0NCS, Tore Andersson

Mrtstigen 34

151 39 Sdertlje

Sweden

Inside Sweden to Pg 650905-1170.

73 de Tore SM0NCS@SM0NCS

Computer BBS: +46 (0)8 550 973 25

Contents.

A.......................Whats new 4.0

1.......................Installation

1.1.............7pserv.cfg

1.2.............Setting download paramters, DOWNLOAD.DEF

1.3.............Setting upload parameters, UPLOAD.DEF

2.......................How the server works

2.1.............Example of CRON.SYS

2.2.............Example of AT commands

2.4.............Special working fields

2.4.1...Redirecting the reply

2.4.2...Destination

2.4.3...Pass

2.5.............Statistic files (.DBT)

2.6.............Password establishing

3.......................File access

4.......................Receiving files

5.......................Automatic server to server

A               Whats new 4.0

* 7plus is now made as a subrutine from 7pserv, 7pserv could

just as well been called by spawnl or exec. Although this is

no possible because it takes to much memory which the general

FBB BBS do not have. Instead 7plus main routine is renamed

to start_7plus and called from whithin 7pserv. What this actually

means is that whenever a new 7plus is released we can implement

it in 7pserv just with one simple compilation.

* From the point above 7pserv now uses return codes from 7plus.

* You can now also limit the local user from a parameter in

7pserv.cfg.

* You can define for each intended user or for a NORMAL user.

* When 7pserv is open Monday through Sunday for download and

upload. You specify the time period in a setup file.

* You can specify the max allowed size allowed to be downloaded

whithin a specific period (messured in hours).

* 7Pserv will arrange a statistic file both for uploads and

download. This file is used by 7pserv but can be viewed and

edited by the Sysop as it is a regular textfile with entries

per call.

* You can now direct the 7plus files by entering

SP CALL @ BBS or SB CALL @ BBS in the message.

* The user can request statistics by entering /STATUS at the end

of the subject at any time. He is then given a detailed status

report.

* 7Pserv is now language independent. With the release of 7pserv

you are given 4 languages SWEDISH, ENGLISH, FRENCH, and GERMAN.

These four are represented by induvidual .7P files. When a

user request e.g /STATUS from 7pserv it will look at the callsign

and match this with the entries in the file 7PSERV.LNG. If

7Pserv finds a match it will use the specified language file.

1               Installation

This server was originally written for F6FBB BBS and every

new version is tested on the latest FBB BBS product. Since

Version 4.0 it is also tested on SPY BBS software. I

have tried to make the program independent of the FBB

environment and I know that some are using it on AA4RE

for example. If you are not sure if your BBS software

can use this product I'm happy to help you with any

questions concerning 7pserv installation to your BBS

software. Before aproaching me with your question please

read through the documentation concerning servers,

external program execution, import and export in your

BBS documentation so that you can put a good description of

the capabilities of your BBS.

The program uses the 7plus algorithm to convert the files to

text format. It splits the files according to 7plus standard

with a size that you as a Sysop determines. The size of the

sent file also depends on how far the request has travelled.

The size of the file is substracted with a fixed amount for

every R-row in the received request. Therefore you can also set

the minimum size of files sent out from the BBS. All of these

configuration items is set in the 7pserv.cfg file. An example

of this file is shown below.

1.1             7pserv.cfg

# 7PSERV.CFG for V 4.0

# Number of parts per mail.in for remote users

1

# Maximum size given to 7plus as sb parameter

9000

# Minimum size. The above size is subtracted down on each

# R:row in the message. This amount is a minimum value which

# the server go lower than.

4000

# The name and location of your mail in file.

c:\project\usagent\mail.in

# User directory, from where the server looks for new files

# and stores uploaded files

c:\project\usagent\

# The call of your BBS, used as return adress of 7pserv mail

sk5as

# The place where 7pserv stores uploaded 7plus parts not yet

# all parts received.

c:\project\usagent\7ptemp\

# The maximum queue on the server. mail.xxx files allowed.

20

# Directory where .inf files are being put.

c:\project\usagent\doc\

# Number of parts per mail in for local users

1

# The callsign of the sysop

sk5as

# Priviliged uses same as 7pserv V3.3

1.1     Setting download parameters (DOWNLOAD.DEF)

In the setup file DOWNLOAD.DEF you must enter some definitions

consering downloading files from the server. First you must

enter a field called "NORMAL" which states the parameters given

to a user which is not mentioned in this file. For each other

call you which to alter from the NORMAL entry you should enter

one line with his/her call and what parameters should be given.

Syntax:

<CALL> <Monday Open>-<Closed>...<Sunday Open>-<Closed> <Bytes> <per hour>

Example:

NORMAL 08-09 08-10 09-15 09-15 09-15 09-15 09-15 40000 24

SM0NCS 08-09 08-20 09-20 09-20 09-20 09-20 09-20 10000 200

SM5NCZ 08-09 08-20 09-21 09-20 09-20 09-20 09-20 80000 20

This example states that the server is open for all users not

mentioned in the list during:

Monday:    08-09

Thuesday:  08-10

Wednesday: 09-15

Thursday:  09-15

Friday:    09-15

Saturday:  09-15

Sunday:    09-15

And that they in this case are allowed to download 40000 bytes during

a period of 24 hours. (After 24 hours since last download they

are given 40000 more bytes).

SM0NCS on the other hand can use the server:

Monday:    08-09

Thuesday:  08-20

Wednesday: 09-20

Thursday:  09-20

Friday:    09-20

Saturday:  09-20

Sunday:    09-20

He is allowed to download 10000 bytes during a period of 200

hours.

--------------------------------------------------------------

NOTE:   Don't forget to enter a NORMAL field

--------------------------------------------------------------

1.2     Setting upload parameters (UPLOAD.DEF)

This setup file has the same syntax as UPLOAD.DEF but with

one small differens. Since it would be hard to prevent sizes

of the extracted file we instead set the amount in number of

parts.

Example:

NORMAL 08-09 08-10 09-15 09-15 09-15 09-15 09-15 4 24

SM0NCS 08-09 08-20 09-20 09-20 09-20 09-20 09-20 1 5

SM5NCZ 08-09 08-20 09-21 09-20 09-20 09-20 09-20 8 20

The server is open for upload for users not in the list

Monday:    08-09

Thuesday:  08-10

Wednesday: 09-15

Thursday:  09-15

Friday:    09-15

Saturday:  09-15

Sunday:    09-15

This users can upload 4 parts during a period of 24 hours.

Och att det i det normala fallet kan laddas upp 4 gnger under en

period av 24 timmar.

Och att SM0NCS kan anvnda servern:

Mndag: 08-09

Tisdag: 08-20

Onsdag: 09-20

Torsdag: 09-20

Fredag: 09-20

Lrdag: 09-20

Sndag: 09-20

Han kan dessutom ladda ner 1 gmg under en period av 5

timmar.

DOWNLOAD.DBT/UPLOAD.DBT

The server saves the statistics in a regular textfile. The

file contains how much the users have upload/downloaded.

These files are named DOWNLOAD.DBT and UPLOAD.DBT and have

the following syntax:

CALL   DAY:HOUR TOTALT MAX

One entry can look as follows

SM0NCS 2:10 1300 10000

Vilket innebr att SM0NCS har laddad senast den 2

klockan 10.00 och att han senast laddade 1300 byten och att

den hans total r 10000 byten sedan servern startades.

2               How the server works

The server does NOT read the init.srv file (FBB), this is

done because it should be possible to use the server at

other BBS software aswell. It also allows you to portion

the mail.in files in mail.xxx files with a utility called

mailren. This program is independet of 7pserv.exe so you

could use it for other purposess as well. Mailren should be set

to run in the cron.sys file (FBB). Your choice in how often the

program is set to be run affects the load you cause to the

network. For example if you set the number of outgoing mails to

2 in 7pserv.cfg and the Mailren to exec every second hour then

two files will be going out every second hour. This operation

is merely a renaming of files which is done very quickly.

2.1             Example of CRON.SYS

IF D 0-6

IF H 0,2,4,6,8,10,12,14,16,18,20,22

X MAILREN

ENDIF

ENDIF

2.2             Example of AT commands

If you are using SPY BBS, there is no CRON.SYS instead you should

set up AT commands in the AUTOEXEC.SPY file. Example of AT command

to run MAILREN follows:

AT 00 RUN MAILREN

AT 30 RUN MAILREN

This will run mailren.exe every half hour, or:

AT 11:00 RUN MAILREN

AT 22:00 RUN MAILREN

This will run mailren.exe two times every 24 hour once 11 and once 22.

2.2             Special fields

Due to the fact that the server has some automatic functions so

that two servers of the same type can exchange files without

any human involvement, some communication field must be used.

These are from field, destination, type and pass. They can also be

used by the users to manipulate the servers behavior. A field

consist of the keyword listed above, followed by a semicolon

and a space and the fieldinformation. The keyword is not case

sensitiv.

Keyword: Fieldinformation

From version 4.0 an additional way of manipulating the reply

and type has been added this is done by typing a regular

SB or SP command in the letter. For example to redirect the

7plus mail to SM5NCZ @ SK5AS type following would do the job.

SP SM5NCZ @ SK5AS

To send bulletin the following would be OK.

SB 7PSERV @ SM

----------------------------------------------------------------

NOTE:   When redirecting the new callsign is taken into acount

considering the permissions. This is also valid for

bulletines.

----------------------------------------------------------------

2.2.1   SP Field

This field specifies from which station the message is sent

from. This is because we want the servers to be able to adress

to themself without specifying an illegal from adress. An

example of such a field is SP 7pserv@sm0ncs. Because of

this there could be many ways for the server to determine to

which BBS some reply should be sent. 7Pserv determines this in

the following order. Ther server always adds the SP field in

the letter to allow automatic reply from distant servers.

Priority .           Source.

1                 SP field if it exists

2                 the route in the subject if exists

3                 Scanning of R:rows

2.2.2   Destination

This function is a copy function to get a file and put it in

another directory and or with another filename. The subject of

the message determines where the file will be put. The

destination  field is therefore switched with the subject in

the different serverlocations. An example of use is shown in

the chapter about fileaccess.

2.2.3   Pass

This is a field for the users to notify the user and to access

their special permissions given. If pass is not given or if it

is not correctly given the users rootdirectory is defaulted to

the user library stated in init.srv. The server also has a

function which allows two servers to copy messages from one

special permission directory at one server to another special

permission directory at the second server. This function is

however not much tested yet and I would very much apprichiate

any possible missfunctionreports. To use this function you

should specify a field lookin like.

Pass: """"" Newpass: """""

The newpass is then set to a Pass: field in the reply to allow

the file to be put in special permissions directory.

2.2.4   Type

The Type field is a backward compatible way of entering the

type of the message the syntax is Type: X where X is the type

of the reply. An other way of doing this is by entering the

SZ XXXX @ YYYY field  where Z is the type. (Se above).

2.3             Password establishing

As stated above if a user wants to access his special

permissions given by the Sysop at the BBS he should specify the

pass field in the message. The password is then checked with a

special algorithm. This algorithm is somewhat similar to the

way FBB uses its Sysop login. The differens is that it ofcourse

is not possible to randomise any numbers for the user to make

as lookup numbers in the passwordstring, because it is not an

interacting connection. Instead I choose to generate this five

numbers from the filename that the user wishes to get or upload

respectively.

The filename, not the path, are scanned through for letters in

the alphabet (a - z) not case sensitive. All other characters

such that underscores are ignored and the next characters are

taken instead. The first up to five, maybe fewer, is taken to

be based as the basis for the index to the password string. The

integer index is set to be the order it has in the intervall

(a - z), a = 1 b = 2 etc. If the case is that this number is

bigger then the length of the password the number is

substracted by passwordlength. E.g if the passwordlength i 10

and an index is found with the number 12 the new number is 2.

This is similar to begin counting from the begining of the

password again when reaching the end of it.

Example.

Imagine that sm0ncs wants to get a file from the BBS and that

we are using the 7pserv.pri file given as an example in this

documentation. That is the password we want to use is TestAr.

If the filename we want to fetch is autoexec.bat the password

that should be entered in pass field is:

Character         Order        Char in password

a               1               T

u               21              s

t               20              e

o               15              s

e               5               A

The get is made by:

sp 7pserv@bbscall

\autoexec.bat

Pass: TsesA

Because of the complexity of this password counting I have

given a utility program called 7ppass that does this counting

for you. You could give the users a copy of 7pserv.pri edited

so that only their permission is entered in it. And the 7ppass

program. Then they could just call 7ppass with the filename and

their callsign as an argument then the program will tell them

what string to enter in the message.

2.4     User priviliges levels

Because of the fascility of using passwords there is also a

way of knowing the users priviliges level that determines

what kind of manipulation the user is allowed to do. This

version of 7pserv suports three levels of priviliges, one

where the user is setup in the list of priviliged users as

a user with full permissions that is with no restrictions,

the second level is a user with in the same list but with

restrictions such as permissions on one directory level,

the third and last level is a user that is not mentioned in

the list as priviliged.

3               Fileaccess

The user access your files by sending a letter to the bbs

adressed like:

sp 7pserv@bbscall

\public\fv.com@bbs_route

As stated erlier in this text the server has different methods

for determining what route the reply should be sent to. One of

these methods are the @field in the subject noted above. If

none of the 7pfrom field an the @field above is found the

server scannes through the R:Lines in the received request to

find a reply BBS.

When a mail arraives to the server the path stated is checked

to see if it is a valid path. What root directory is used is

depending on if the user uses special permissions or not. If

the path does not exist a reply message stating it is sent to

the user.

The file FV.COM is splitted into pieces and sent to the user.

If the user when unpacking receives an error from his 7plus he

can request a correction file from the server by:

sp 7pserv@bbs_call

\public\fv.err

The correction file is then sent to him as a .cor file that

then could be used directly in his 7plus.exe program.

4               Receiving files

In addition you can also receive files automatically from users

that whishes to send you them. This is done by:

sp 7pserv@bbs_call

\upload\test.p01

The user should do this for every part he has splitted the file

into. Because of this the server must keep track when all files

has been received. If some file is missing nothing happens.

Whan all files has been received it tries to unpack it. If then

an error is detected it automatically sends a correction

request to the user in the form of a standard .err file that

could be entered in his 7plus.exe program. The user is then

supposed to send a correction file to the user in the following

format:

sp 7pserv@bbs_call

\upload\fv.cor@bbs_call

The message should contain the error file created by 7plus

program.

5               Automatic exchange

The server provide functions for sending files between two

servers of this type without human interaction.

Ex.

SM6XXX has installed this server and so has SK0XXX. SM6XXX

finds an interesting file in SK0XXX system and wants to get it

from him. He therefore enters the following.

sp 7pserv@sk0xxx

\BBSER\PRG\RLI.ARJ

Destination: \BBS\W0RLI.ARJ

7PFrom: 7pserv@sk0xxx

This is the only human intervention needed for transfering this

file to him. The servers is automatically sending .cor and .err

to each other switching names to inform the other which file is

intended.