FBB Forward Protocole.

----------------------

FBB software includes  two forward protocoles.  The first one

is standard with MBL/RLI protocole.  The second one was developped

to allow  a better  efficiency, particularly  on long  links where

propagation time  of data  are long.  The exchange  of commands is

reduced to a  minimum, and not  acknoledged to get  time. The data

transfer direction is changed every block of data, a block of data

holding up to  five messages.  This uses the  "pipeline" effect of

long links (Nodes and digipeaters),  and gain some time over short

links (HF...).

FBB protocole is very simple in its principle. It is based on

MID/BID usage. The identification  is made by the  F letter in the

SID (system  type identifier  contained  in square  brackets). All

command lines must start in  first collumn with the 'F' character.

All command lines are ended by a return (CR) character.

Suppose I  call  another BBS  to  forward some  mail.  When I

connect another BBS  using FBB  protocole, I will  receive the SID

followed by a text and the prompt (">"). If the SID contains the F

flag, I will send immediately my SID and the first proposal.

Proposals looks like :

FB P F6FBB FC1GHV FC1MVP 24657_F6FBB 1345

F>

FB          : Identifies the type of the command (proposal)

P           : Type of message (P = Private, B = Bulletin).

F6FBB       : Sender (from field).

FC1GHV      : BBS of recipient (@field).

FC1MVP      : Recipient (to field).

24657_F6FBB : BID ou MID.

1345        : Size of message in bytes.

F>          :  End of proposal.

ALL the fields are necessary.  This kind of command must hold

seven fields.  If  a field  is  missing upon  receiving,  an error

message will be send immediately followed by a disconnection.

A proposal can  handle up  to five  FB command  lines. If the

total size of messages seems to be too important, the proposal can

handle less  lines. In  FBB  software, a  parameter is  defined in

INIT.SRV file to tell the maximum size of the message block. It is

set by default to 10KB  for VHF use. It can be  adjusted according

to the quality of the link.

Exemple of proposal :

FB P F6FBB FC1GHV.FFPC.FRA.EU FC1MVP 24657_F6FBB 1345

FB P FC1CDC F6ABJ F6AXV 24643_F6FBB 5346

FB B F6FBB FRA FBB 22_456_F6FBB 8548

F>

This proposal is limited to three  FB lines, as the amount of

messages overran the 10KB limit defined for this link.

When receiving  the  proposal,  the  other  BBS  will reject,

accept or defer the message. This command is made by a FS line :

FS -+=

This means :

- I don't want the first message (-).

- I need the second message (+).

- I defer the third message, as I'm still receiving it.

It should  interesting to  defer a  message if  you are still

receiving it on a other channel, or  if you think that the size is

to big,  or for  another  reason. The  message should  be proposed

again at the next connection.

FS line  MUST  have  as  many +,-,=  signs  as  lines  in the

proposal.

When  receiving  the  FS  lines,  I  can  send  the block  of

messages. Each message is  made with the title  on the first line,

the text, and  a Ctrl  Z in  the last line.  The is  no blank line

between the messages.

Title of 2nd message

Text of 2nd message

.....

^Z

When the other  BBS has  received all the  asked messages, it

acknoledges by sending its proposal, and the system is reversed.

If it has no message to send, it only sends a line :

FF

This line must not to be followed by a F>.

If the other hand has no message, it sends a line :

FQ

and asks for the disconnection.

Example :

---------

F6FBB                    FC1GHV

----------------------------------------------------------------

Connects FC1GHV

Connected

[FBB-5.11-FHM$]

Bienvenue a Poitiers, Jean-Paul.

>

[FBB-5.11-FHM$]   (F6FBB has the F flag in the SID)

FB P F6FBB FC1GHV.FFPC.FRA.EU FC1MVP 24657_F6FBB 1345

FB P FC1CDC F6ABJ F6AXV 24643_F6FBB 5346

FB B F6FBB FRA FBB 22_456_F6FBB 8548

F>

FS +-+  (accepts le 1st et le 3rd).

Title 1st message

Text 1st message

......

^Z

Title 3rd message

Text 3rd message

......

^Z

FB P FC1GHV F6FBB F6FBB 2734_FC1GHV 234

FB B FC1GHV F6FBB FC1CDC 2745_FC1GHV 3524

F>

FS --  (Don't need them, and send immediately the proposal).

FB P FC1CDC F6ABJ F6AXV 24754_F6FBB 345

F>

FS +  (Accepts the message)

Title message

Text message

......

^Z

FF  (no more message)

FB B F6FBB TEST FRA 24654_F6FBB 145

F>

FS +  (Accepts the message)

Title message

Text message

......

^Z

FF  (still no message)

FQ  (No more message)

Disconnection of the link.

In this example,  FBB protocole is  used as the  two BBS were

identified by the  F flag in  the SID.  If F6FBB had  sent the SID

[FBB-5.10-MH$] when answering FC1GHV,  the protocole should be the

standard MBL/RLI.

All callsigns are only examples !

Extension to the protocole. Compressed forward FBB.

---------------------------------------------------

The protocole utilized for the  transfer of ascii files compressed

is an extension to the  existing protocole. The compressed forward

is  validated  by  the  presence  of  the  letter  B  in  the  SID

[FBB-5.12-BFHM$]. The transfer  of compressed files  can only take

place under FBB protocole. The presence of the letter B in the SID

without the F letter will remain without effect.

The only difference as regard to the standard protocol is the

submit line.  It can  specify the  type of  data contained  in the

compressed message. FA  means that  the transfer will  be an ascii

compressed message.  FB means  that the  message will  be a binary

compressed file (this  last possibility is  not yet implemented).

The submission of an ascii message will be in the form :

FA P FC1CDC F6ABJ F6AXV 24754_F6FBB 345

The submission of a binary file will be in the form :

FB P FC1CDC F6ABJ F6AXV 24754_F6FBB 345

The transfered data are of a specific format. The transfer will be

done in binary mode. This last one is derived of the YAPP protocol

which is very reliable. All transfer  is made of a header, a block

of data,  an  end of  message  and  a checksum.  Each  transfer is

equivalent to the transfer of one message of the standard protocol

and shall  not  be  followed  by  a control  Z,  the  end  of file

specifier is defined in another way.  Unlike YAPP transfers, there

is  no acknowledgement  during the  transmission of  messages, the

protocole is then more simple and efficient.

Format of header for an ascii compressed message (submission FA) :

<SOH>                    1 byte = 01 hex

Length of the header     1 byte = Length from the title,

including the two <NUL> characters.

Title of the message     1 to 80 bytes

<NUL>                    1 byte = 00 hex

Offset                   1 to 6 bytes

<NUL>                    1 byte = 00 hex

Format of header for a binary compressed file (submission FB) :

<SOH>                    1 byte = 01 hex

Length of the header     1 byte = Length from the filename,

including the two <NUL> characters.

Name of the file         1 to 80 bytes

<NUL>                    1 byte = 00 hex

Offset                   1 to 6 bytes

<NUL>                    1 byte = 00 hex

As to follow  the french regulation,  the title of  the message or

the file name are transmitted in readable ascii, not compressed.

The offset is also  transmitted in ascii  and specifies the offset

at which the  data should be  inserted in  the file (in  case of a

fragmented file).  In  the  version 5.12,  this  parameter  is not

utilized and is always equal to zero.

A data  block contains  from one  to 256  bytes. It begins  by two

bytes which specify the format.

Data block format :

<STX>                    1 byte = 02 hex

Number of data           1 byte = 00 to ff hex.

if length is 256 bytes, the value is 00.

Data bytes               1 to 256 bytes

The last data block  is followed by the  end of file specifier and

the checksum.

End of file specifier format :

<EOT>                    1 byte = 04 hex

Checksum                 1 byte = 00 to ff hex

The checksum is  equal to  the sum  of all  the data  bytes of the

transmitted file, modulo 256 (8 bits) and then two's complemented.

The checking of the checksum is very simple :

The sum  of the  datas  from the  file  and the  checksum received

modulo 256 shall be equal to zero.

In case of a checksum error, the  message or the file is not taken

to account and the system issues a disconnect request after having

sent the comment :

*** Erreur checksum

Ascii values of the characters (1 byte) used in the protocole :

<NUL> = 00 hex

<SOH> = 01 hex

<STX> = 02 hex

<EOT> = 04 hex

Most of  ideas for this  binary transmission  are issued from YAPP

protocole. Thanks to WA7MBL.

Comments will be welcome.

F6FBB @ F6FBB.FMLR.FRA.EU