KAM 2.85 YAPP DOCUMENT

Wayne Cornick N0FUL

This document is to inform you the prospective user of YAPP of as

many pitfalls and problems as I have found in the setup of the Kantronics

KAM using version 2.85 firmware. Not all the commands will be present on

lower versions and all I can recomend is that you change only the one's

that are common to your firmware.

In the YAPP.CNF file you MUST tell the program what COM PORT and the speed

of that COM PORT!  I use 9600 baud on COM1 here.

If the program tries to do a download and either times out or aborts the

download you may have to change the TNC TYPE from 1 to 2 and try again!!!

The KAM must have CONMODE set to CONVERS and PERM'ed!

While useing the YAPP program for file transfers you MUST turn

MONITOR OFF!!!!!! If you are useing it as a terminal program for regular

packet use you may run monitor in the on position.

********************************************************************************

!!!!!***** IMPORTANT *****!!!!! DO NOT have MCOM or MCON on during file

transfers or the first time your system "HEARS" any other packet signal it

WILL include whatever it heard in the file you are trying to download and corrupt it!!!!! It will do this whether or not the packet was addressed to you or not!!!!

********************************************************************************

You MUST have XON XOFF START & STOP all set to $00 before using YAPP for

transfering files! I suggest you enter those commands into the YAPP.CNF file

and change them back via the YAPP.CNF file also. These changes are allowed in the .CNF file after the basic system configuration.

you will see a line that reads as follows:

*** EOF of commands set to tnc at startup

Place all commands that need to be changed from your normal operating parameters in this area before the ***EOF.

just after that area there is annotherone:

*** EOF of commeands sent to tnc at shutdown

place all changes of paramentes that will return your tnc to it's normal

operating parameters in this are before the ***EOF marker.

PLACE NOTHING AFTER THE SECOND *** EOF MARKER.

One more thing!            GOOD LUCK AND HAVE FUN!!!!!

see you yapp'n      --wayne--