AX.25 LAYER 2 PROTOCOL SPECIFICATION

This   protocol   conforms  with  the  ISO  Recommendations  3309,  4335

(including DAD 1&2) and 6256 high-level data  link  control  (HDLC)  and

uses some terminology found in these documents.

This  protocol also conforms with ANSI X3.66, describing ADCCP, balanced

mode.

This protocol is written  to  work  equally  well  in  either  half-  or

full-duplex amateur radio enviroments.

This  protocol  has  been  written  to  work  equally  well  for  either

point-to-point connections, or connections made thru  a  larger  device,

such as a metropolitan network controller (MNC).

This  protocol  does  allow  the  establishment of more than one layer 2

(link layer) connection per device, if the device is so capable.

This protocol also follows in principle the CCITT  X.25  recommendation,

with  the exception of an extended address field and the addition of the

Unnumbered Information (UI) frame.

Most layer 2 protocols assume that one large device (generally  called a

DCE,  or  data  circuit-terminating  equipment)  is connected to several

smaller devices (usually called a DTE, or data  terminating  equipment).

AX.25  assumes  that  both  ends  of  the  link  are  balanced,  thereby

eliminating the two different classes of device.

Frame_Structure

Level 2 packet-radio transmissions are  sent  in  small  blocks,  called

frames. These frames are made up of smaller parts, called fields. Fig. 1

shows how the three types of frames are made up. Fig. 1 shows the frames

in   the   same   bit   order  that  most  packet  articles  show  them.

Unfortunately,  this  method  has  led  to  some  confusion,  since  the

least-significant  bit (LSB) is to the left rather than to the right, as

most people would ordinarily assume.  I am pointing  this  out  early in

this  paper  to  prevent mass confusion as I progress.  Later on, I will

switch to a hopefully more understandable way of showing the  frame  ans

its components.

Field_Definitions

The  frame  is  made  up of several parts, called fields.  Each of these

fields is made up of an integral number of octets (or bytes), and serves

a specific function.

Flag_Field

Since  amateur  packet radio is a bit-oriented protocol, the only way to

tell when one frame is over and another  is  starting  for  sure  is  to

delimit each frame with a certain bit sequence both at the beginning and

the end.  This is the job of the flag field.  A flag consists  of a zero

followed by six ones followed by another zero, or 01111110 (7E hex). Due

to the bit stuffing mentioned above, the  only  time  this  sequence  is

allowed is at the beginning and end of a legitimate frame.

Address_Field

The address field is used to identify both where the frame came from and

what the destination of it is.  In the CCITT  recommendation  X.25, this

field is only one octet long. This permits at most 256 users per level 2

channel, and since some bits of this field were used for other purposes,

the real number of users were about thirty per level 2 channel. Both the

HDLC and ADCCP recommendations allowed the address field to be extended,

so  we  decided to extend the address field per their recommendations in

the amateur version of  X.25  to  include  the  callsigns  of  both  the

destination and source amateur radio stations. The method used to extend

the address field will be described shortly.

Control_Field

The control field is used to identify the  type  of  frame  and  control

several attributes of the level 2 connection. It is one octet in length,

and its encoding will be discussed in a following section.

PID-Field

The Protocol Identifier (PID) field is used only in information  frames,

and  identifies  what  kind of layer 3 protocol, if any, is in use.  Its

encoding is as follows:

M      L

S      S

B      B

xx00xxxx  Reserved at the moment.

xx01yyyy  AX.25 layer 3 implemented.

xx10yyyy  AX.25 layer 3 implemented.

11110000  No layer 3 implemented.

11111111  Escape character.  Next byte contains more PID information.

Where:

1.   An x indicates a "don't care" bit.

2.   A y indicates all combinations used.

Information_Field

The information field is used to convey the actual user  data  from  one

end of the link to the other. I fields are allowed in only three tyes of

frames, the I frame, the UI frame, and the FRMR frame.  The I  field can

be up to 256 octets long, and should be an even multiple of octets long.

Any information in the I field should be passed along the  link  totally

transparently,  except  for  any zero-bit insertion necessary to prevent

flags from accidentally appearing in the I field.

Frame_Check_Sequence

The frame-check sequence is a sixteen-bit number calculated by both  the

sender  and receiver of a frame.  It is used to make sure that the frame

was not corrupted by the medium used to get the frame from the sender to

the  receiver.    It  is  calculated  in accordance with ISO 3309 (HDLC)

recommendations.

Bit_Stuffing

In order to assure  that  the  flag  sequence  mentioned  above  doesn't

accidentially  appear  anywhere  else  in a frame, as the frame is being

sent it should be monitored, and if more than five contiguous  ones  are

detected,  a  zero bit should be added between the fifth and sixth ones,

eliminating the possibility of a flag appearing in the frame other  than

where  it  belongs.    The  receiver of five ones, a zero, and more ones

should automatically eliminate the inserted zero before passing the data

on.

Bit_Order_of_Transmission

With  the exception of the FCS field, all other fields in an AX.25 frame

should be sent starting with the least-significant bit.    In accordance

with HDLC practices, the FCS should be sent most-significant bit first.

Frame_Abort

If a frame must be prematurely aborted, at least fifteen contiguous ones

should be sent with no bit stuffing added.

Invalid_Frames

Any frame consisting of less than 136 bits, or not  bounded  by  opening

and  closing  flags, or not octet aligned (an integral number of octets)

should be considered an invalid frame by the link layer.

Address_Field_Encoding

The address field  of  all  frames  should  be  encoded  with  both  the

destination  and  source  amateur  callsigns of the frame.  If a level 2

amateur "repeater" is to be used, its callsign should  also  be  in  the

address  field.   AX.25 follows the HDLC recommended method of extending

the address field in order to fit all this information into the  address

field.

Basically,  the  way the HDLC address field is extended beyond one octet

is to reserve the least-significant bit of each octet for what is called

an  "extender  bit".  This bit is set to zero if the next octet contains

more address field information, and is set to one if this  is  the  last

octet.   To make room for this extender bit, the amateur radio call sign

information is shifted  one  bit  to  the  left.    The  actual encoding

techniques for both non-repeater and repeater operation follows.

Non-Repeater_Address-Field_Encoding

If a level 2 repeater is not being used, the address field is encoded as

shown in Fig. 2. The destination address is the call sign of the amateur

radio  station  that the frame is addressed to, while the source address

contains the amateur call sign who sent the frame.  These call signs are

the  call signs of the two ends of a level 2 AX.25 link only, not of any

other station, such as  the  destination  of  a  packet  going  thru  an

intermediary  link.    Those  addresses should be in a higher layer, not

layer 2.

A1 thru A14 are the  fourteen  octets  that  make  up  the  two  address

sub-fields  of  the address field.  The destination sub-address is seven

octets long (A1 thru A7), and  is  sent  first.    This  will  allow the

receivers  of  the frame time to check the destination address sub-field

and see if the frame is for them while the rest of the  frame  is  being

received.    The source address sub-field is then sent in octets A8 thru

A14. Both of these sub-fields are encoded in the same manner, except for

the last octet having the HDLC address extender bit set.  Since they are

basically the same, only the destination sub-address will be outlined.

There is an extra octet at the end of each address sub-field that allows

room for a Secondary-Station Identifier (SSID) and three additional bits

for future expansion. The SSID field allows an Amateur Radio operator to

have more than one packet radio station.  This is useful when an amateur

wants to put up a repeater  in  addition  to  his  regular  station  for

example.

Appendix A shows a typical AX.25 frame in the non-repeater mode.

Destination Sub-Field Encoding

Fig.    3  shows  how  an amateur call sign is placed in the destination

address sub-field, occupying octets A1 thru A7.

| Octet | ASCII |Bin.Data|Hex Data|

-----------------------------------

|  A1   |   W   |10101110|   AE   |

|  A2   |   B   |10000100|   84   |

|  A3   |   4   |01101000|   68   |

|  A4   |   J   |10010100|   94   |

|  A5   |   F   |10001100|   8C   |

|  A6   |   I   |10010010|   92   |

|  A7   | SSID  |0RRSSID0|        |

-----------------------------------

Bit Position-->  76543210

Fig. 3.  Destination Field Encoding

Where:

1. The top octet (A1)  is  the  first  octet  sent

(sort  of  like  popping  it  off  the  top of the

stack), with bit 0 of each octet being  the  first

bit sent, and bit 7 being the last bit sent.

2.  The  first  (low  order  or bit 0) bit of each

octet is the HDLC extender bit, which  is  set  to

zero  on  all  but  the  last octet in the address

field, where it is set to one.

3. The bits marked "R" are reserved bits. They may

be  used  in  an  agreed upon manner in individual

networks.  If they aren't implemented, they should

be set to one.

4.  The  characters  of  the  callsign  should  be

standard seven-bit ASCII (upper case only)  before

being  shifted  left to make room for the extender

bit.  If the callsign is less than  six characters

long, it should be padded at the trailing end with

ASCII spaces between the end of the  callsign  and

the SSID octet.

5.  The  SSID  portion  of the last octet has been

intentionally left vague at  this  point,  and  is

left  up to the individual station to assign.  The

only recommended restriction  is  to  reserve  the

all-one  condition  (1111) for an all-call SSID in

case one wants to reach  an  amateur  but  doesn't

know what SSID that amateur operates under.

Level_2_Repeater_Address-Field_Encoding

If a frame is to go thru a level 2 amateur packet repeater, there is  an

additional address sub-field added to the end of the address field. This

additional sub-field contains the call sign of the repeater to be  used.

This  will  allow  more  than one repeater to share the same rf channel,

which has been a problem with the older protocols. If this field exists,

the last octet of the source sub-field has its extender bit set to zero,

indicating that more address-field data follows.   The  repeater address

sub-field  is  encoded  in the same manner as the destination and source

address subfields, except for one bit in the last octet, called the  "H"

bit.  The H bit is used to indicate whether a frame has been repeated or

not. This is necessary to prevent someone from potentially receiving two

identical frames, the one going to the repeater, and the one coming back

from the repeater.  Fig.  4 shows how the repeater  address sub-field is

encoded.    Appendix B is an example of a complete frame on its way back

from a repeater.

-----------------------------------

| Octet | ASCII |Bin.Data|Hex Data|

|---------------------------------|

|  A15  |   W   |10101110|   AE   |

|  A16  |   B   |10000100|   84   |

|  A17  |   4   |01101000|   68   |

|  A18  |   J   |10010100|   94   |

|  A19  |   F   |10001100|   8C   |

|  A20  |   I   |10010010|   92   |

|  A21  | SSID  |HRRSSID1|        |

-----------------------------------

Bit Order -->    76543210

Fig 4.  Repeater Address Encoding

Where:

1. The top octet is the first octet sent, with bit

0 being sent first, bit 7 sent last of each octet.

2. As with  the  source  and  destination  address

sub-fields  discussed above bit 0 of each octet is

the HDLC address extender bit,  which  is  set  to

zero on all but the last address octet (A21) where

it is set to one.

3. The "R" bits are  reserved  just  like  in  the

source and destination sub-fields.

4. The "H" bit is the has-been-repeated bit. It is

set to zero on a non-repeated frame,  and  set  to

one  by  the  repeater  when  the  frame  has been

repeated.

It should be noted that some of the advantages of this addressing scheme

are mentioned in Appendix C.

Control_Field_Formats

The  control  field is responsible for identifying what type of frame is

being sent, and is also used to convey commands and responses  from  one

end of the link to the other to maintain proper control over the link.

The  control  fields used in AX.25 use the CCITT X.25 control fields for

balanced operation, with an additional control field taken from ADCCP to

allow connectionless and round-table operation.

There are three general types of AX.25 frames.  They are the Information

frame (I frame), the Supervisory frame (S  frame),  and  the  Unnumbered

frame  (U  frame).   Fig.  5 shows the basic format of the control field

associated with these types of frames.

----------------------------------------

|Control Field |    Control Field Bits |

|    Type      | 7 6 5 | 4 | 3 2  1  0 |

----------------------------------------

|    I Frame   | N(R)  |P/F| N(S)  | 0 |

----------------------------------------

|    S Frame   | N(R)  |P/F| S S| 0| 1 |

----------------------------------------

|    U Frame   | M M M |P/F| M M| 1| 1 |

----------------------------------------

Fig. 5.  Control Field Formats

Where:

1. Bit 0 is the first bit sent, bit 7 is the  last

bit sent of the control field.

2.  N(S) is the send sequence number (bit 2 is the

LSB).

3. N(R) is the receive sequence number (bit  6  is

the LSB).

4. The "S" bits are the supervisory function bits,

and their encoding is discussed below.

5. The "M" bits are the unnumbered frame  modifier

bits and their encoding is discussed below.

6. The P/F bit is the Poll/Final bit. Its function

is described in more detail shortly.

Control_Field_Definitions

Information Frame Control Field

All I frames have bit 0 of the control field set to zero.   N(S)  is the

sender's  send sequence number (the send sequence number of this frame).

N(R) is the sender's receive sequence number (the sequence number of the

next expected received frame. These numbers are described in the section

regarding flow control.

Supervisory Frame Control Field

Supervisory frames are denoted by having bit 0 of the control field  set

to  one,  and  bit 1 of the control field set to zero.  S frames provide

supervisory  link  control   such   as   acknowledging   or   requesting

retransmission  of  I  frames,  and  link level window control.  Since S

frames don't have an information field, the sender's send  variable  and

the receiver's receive variable are not incremented for S frames.

Unnumbered Frame Control Field

Unnumbered  frames  are distinguished by having both bits 0 and 1 set to

one.  U frames are responsible for  maintaining  control  over  the link

beyond what is accomplished with S frames. They are also responsible for

the establishment and tearing down of the link.  U frames also allow for

the transmission and reception of information outside of the normal flow

control. Some U frames may contain information fields.

Control_Field_Parameters

Sequence_Numbers_and_Variables

Every AX.25 I frame shall be assigned a sequential number from 0  to  7.

This  will allow up to seven outstanding I frames per level 2 connection

at a time.

Send_State_Variable_V(S)

The send state variable is an internal variable that is never sent.   It

contains  the  next  sequential  number  to  be  assigned  to  the  next

transmitted I frame.  This variable is updated upon the  transmission of

each I frame.

Send_Sequence_Number_N(S)

The  send sequence number is found in the control field of all I frames.

It contains the sequence number of the I frame being sent. Just prior to

the transmission of the I frame, N(S) is updated to equal the send state

state variable.

Receive_State_Variable_V(R)

The receive state variable is an internal  variable  that  contains  the

sequence number of the next expected received I frame.  This variable is

updated upon the reception of an error-free I frame whose send  sequence

number equals the present received state variable value.

Received_Sequence_Number_N(R)

The received sequence number is in both I and S frames. Prior to sending

an I or S frame, this variable is updated to equal that of the  received

state variable, thus implictly acknowledging the proper reception of all

I frames up to and including N(R)-1.

Poll/Final_(P/F)_Bit

The P/F bit may be used in all types of frames.  It is used in a command

(poll) mode to request an immediate reply to a frame.  The reply to this

poll is indicated by setting the response (final) bit in the appropriate

frame. Only one outstanding poll condition per direction is allowed at a

time.

Control_Field_Encoding

Information_Frame_Control_Field

The information frame control field is encoded as shown in Fig. 6. These

frames  are  sequentially  numbered to maintain control of their passage

over the link level connection.

Control Field Bits

-------------------------

| 7 6 5 | 4 | 3 2 1 | 0 |

-------------------------

| N(R)  |P/F|  N(S) | 0 |

-------------------------

Fig. 6.  I Frame Control Field

Supervisory_Frame_Control_Field

The supervisory frame control fields are encoded as shown in Fig.  7. In

AX.25, S frames are used only as responses to other frames.

------------------------------------------------

|  Control Field Bits  | 7 6 5 | 4 | 3 2 | 1 0 |

|----------------------------------------------|

|   Receive Ready  RR  | N(R)  |P/F| 0 0 | 0 1 |

|Receive Not Ready RNR | N(R)  |P/F| 0 1 | 0 1 |

|  Reject          REJ | N(R)  |P/F| 1 0 | 0 1 |

------------------------------------------------

Fig. 7.  S frame control Fields

Receive_Ready_(RR)_Response

Receive Ready is used to do the following:

1. To indicate that the sender of the  RR  is  now

able to recieve more I frames.

2.  To  acknowledge  properly received I frames up

to, and including N(R)-1.

3.  To  clear  a  previously  set  busy  condition

created by an RNR command having been sent.

It  should be noted that the status of the other side of the link can be

requested by setting the poll bit.

Receive_Not_Ready_(RNR)_Response

Receive not ready is used to indicate to the sender  of  I  frames  that

receiver is temporarily busy and cannot accept any more I frames. Frames

up to N(R)-1 are acknowledged.  Any I frames  numbered  N(R)  and higher

that might have been caught in between and not acknowledged when the RNR

command was sent are NOT acknowledged.

The RNR condition can be cleared by the sending of a  UA,  RR,  REJ,  or

SABM frame.  The P/F bit can be used within the RNR frame to interrogate

the status of the other side of the link.

Reject_(REJ)_Response

The reject frame is used to request retransmission of I frames  starting

with N(R). Any frames that were sent with a sequence number of N(R)-1 or

less are acknowledged.  Additional  I  frames  may  be  appended  to the

retransmission of the N(R) frame if there are any.

Only  one reject frame condition is allowed in each direction at a time.

The reject condition is cleared by the proper reception of I  frames  up

to the I frame that caused the reject condition to be initiated.

As  with the other supervisory responses, the P/F bit may be used in the

REJ frame.

Unnumbered_Type_Frames

Unnumbered frame control fields are either commands or responses.   This

standard follows X.25 as much as possible.  The only deviation from X.25

is in the addition of the Unnumbered Information (UI) frame from  ADCCP.

X.25  is designed to work with in full-duplex systems with only one main

device (DCE) and potentially many users (DTEs).

Amateur Radio packet systems differ greatly on both of  these  respects.

Not  only  is  Amateur  Radio packet networking done in a half-duplex rf

environment, but many DCE/DTE links many be sharing  the  same  channel.

Many  amateurs  have  rejected  the  use  of  X.25  as a result of these

problems.  X.25 can easily be enhanced so that it will  perform properly

over amateur radio.

Fig. 8 shows the layout of U frames implemented within this standard.

------------------------------------------------

|   Control Field |Type|  Control Field Bits   |

|                  |   | 7 6 5   4   3 2   1 0 |

|----------------------------------------------|

|Set Asynchronous  |Cmd| 0 0 1 | P | 1 1 | 1 1 |

|Balanced Mode-SABM|   |       |   |     |     |

|----------------------------------------------|

| Disconnect-DISC  |Cmd| 0 1 0 | P | 0 0 | 1 1 |

|----------------------------------------------|

| Disconnected Mode|Res| 0 0 0 |P/F| 1 1 | 1 1 |

|       DM         |   |       |   |     |     |

|----------------------------------------------|

|   Unnumbered     |Res| 0 1 1 | F | 0 0 | 1 1 |

|  Acknowledge-UA  |   |       |   |     |     |

|----------------------------------------------|

| Frame Reject-FRMR|Res| 1 0 0 | F | 0 1 | 1 1 |

|----------------------------------------------|

|   Unnumbered     |Eit| 0 0 0 |P/F| 0 0 | 1 1 |

|  Information-UI  |her|       |   |     |     |

------------------------------------------------

Fig. 8.  U Frame Control Fields

Set_Asynchronous_Balanced_Mode_(SABM)_Command

The  SABM  command  is  used  to  place  2  stations in the asynchronous

balanced mode. This is a balanced mode of operation known as LAP B where

DCEs and DTEs are treated as equals.

Information  fields  aren't allowed in SABM commands.  Any outstanding I

frames left when the SABM command is issued will remain unacknowledged.

Disconnect_(DISC)_Command

The DISC command is  used  to  terminate  a  link  session  between  two

stations. No information field is permitted in a DISC command frame. Any

outstanding I frames will remain outstanding.

Disconnected_Mode_(DM)_Response

The disconnected mode response is sent whenever the DTE or DCE  receives

a  frame  other than a SABM while in a disconnected mode.  It is sent to

request a set mode command, or to indicate it cannot accept a connection

at the moment. The DM response cannot have an information field.

A  DCE or DTE in the disconnected mode will respond to any command other

than a SABM with a DM response with the P/F bit set to 1.

Unnumbered_Acknowledge_(UA)_Response

The  UA  response  frame  is  sent  to  acknowledge  the  reception  and

acceptance  of  a  U  frame command.  A received command is not actually

processed until the UA response frame is sent.  An information  field is

not permitted in a UA frame.

Frame_Reject_(FRMR)_Response

The  FRMR  response  frame  is  sent  to report that for some reason the

receiver of a command or information frame cannot  successfully  process

that  frame  and  that the error condition is not correctable by sending

the offending frame again.  Typically this condition will appear  when a

frame  without  an FCS error has been received with one of the following

conditions:

1.  The reception of an invalid or not implemented

command or response frame.

2. The reception of an I frame  whose  information

field exceeds the agreed upon length.

3. The reception of an improper N(R). This usually

happens when the N(R) frame has already been  sent

and  acknowldeged, or when N(R) is out of sequence

with what was expected.

4. The reception of a frame  with  an  information

field  where  one is not allowed, or the reception

of an U or S frame whose length is incorrect.

When a CMDR or FRMR frame is sent, an information field is added to  the

frame that helps to explain where the problem occurred. This information

field is three octets long and its contents is shown if Fig. 9 below.

--------------------------------------------------

|             Information Field Bits             |

| 2 2 2 2 1 1 1 1 1 1 1 1 1 1                    |

| 3 2 1 0 9 8 7 6 5 4 3 2 1 0 9 8 7 6 5 4 3 2 1 0|

|------------------------------------------------|

| 0 0 0 0|Z|Y|X|W| V(R)|C| V(S)|0| Rejected Frame|

|        | | | | |     | |     | | Control Field |

--------------------------------------------------

Fig. 9.  FRMR Frame Information Field

Where:

1. The rejected frame control  field  carries  the

control  field of the frame that caused the reject

condition.  It is in bits 1-8  of  the information

field.

2.  V(S) is the current send state variable of the

device reporting the rejection (bit 10 is the  low

bit).

3.  V(R)  is the current receive state variable of

the device reporting rejection (bit 14 is the  low

bit).

4.  If  W  is set to 1, the control field received

was invalid or not implemented.

5. If X is set to 1, the  frame  that  caused  the

reject condition was considered invalid because it

was a U or S frame that had an  information  field

that  is  not  allowed.  Bit W must be set to 1 in

addition to the X bit.

6. If Y is set to 1, the  information  field  of a

received  frame  exceeded  the maximum capacity of

the device reporting the condition.

7. If Z is set to 1, the  control  field  received

and  returned  in bits 1 to 8 contained an invalid

N(R).

8. Bits 8, and 20 to 23 are set to 0.   Bit  12 is

set to 0 if the rejected frame was a command, or 1

if if it was a response.

Unnumbered_Information_(UI)_Frame

The  unnumbered  information frame is used to pass information along the

link outside the normal information controls.   This  allows information

fields  to  go back and forth on the link bypassing flow control.  Since

these frames are NOT acknowledgeable, if one gets wiped out, there is no

way to recover it.

The UI frame is not defined in X.25.  It has been  taken  from  ADCCP to

allow uncontrolled information to flow thru the link without interfering

with a next higher layer.

Link_Error_Recovery

There are several link-level errors that are recoverable without tearing

down  the  connection.   These error situations may occur as a result of

malfunctions within the DTE or DCE, or if transmission errors occur.

Invalid_Frame_or_FCS_Error

If an invalid frame is received, or a frame  is  received  with  an  FCS

error, that frame will be discarded with no action taken.

Device_Busy_Condition

When a DTE or DCE becomes temporarily busy, such as when receive buffers

are full, it will send a receive not ready (RNR) frame.  This  tells the

other  side  of the link that the device cannot handle any more I frames

at the moment. This condition is usually cleared by the sending of a UA,

RR, REJ, or SABM command frame.

Send_Sequence_Number_Error

If  the send sequence number , N(S), of an otherwise error-free received

I frame does not match the receive state variable, V(R), a send sequence

error  has  occured,  and  the information field will be discarded.  The

receiver will not acknowledge this frame, or any other  I  frames  until

N(S) matches V(R).

The  control  field of the erroneous I frame(s) will be accepted so that

link supervisory functions can still be performed, such as checking  the

P/F bit. Because of this updating, the retransmitted I frame may have an

updated P bit and N(R).

Reject_(REJ)_Error_Recovery

REJ is used to request  a  retransmission  of  I  frames  following  the

detection of a sequence error.  Only one outstanding reject condition is

allowed at a time.  This condition is cleared when the requested I frame

has been received.

A  device receiving the REJ command will clear the error by sending over

the I frame indicated in N(R) of of the REJ command frame.

Time-out_Error_Recovery

When a transmission abnormality wipes out a single I frame, or the  last

I  frame  of a group, there is no way of telling this immediately, since

the receiver does not necessarily know something was sent until  another

frame  is sent resulting in an out-of-sequence error.  To cope with this

situation better some form of time-out delay will be incorporated by the

sender after it sends out a frame. This time-out timer is started at the

time a frame is sent, and stopped by the reception of an acknowledgement

for the sent frame.  If the timer times out before an acknowledgement is

received, any unacknowledged frames are retransmitted.  The delay  is an

agreed-upon  amount  that  will  vary  with  the  type  of rf medium and

signaling speed used.

Rejection_Error

A rejection error condition occurs when an error-free received frame has

one of the following problems:

1. An invalid command or response control field.

2. An invalid frame format.

3. An Invalid N(R).

4. An information field that exceeds  the  maximum

the device can accept.

Once  a  rejection error occurs, no more I frames are accepted (with the

exception of the P/F bit still usable) until the error is resolved.  The

error  condition  is reported to the other side of the link by sending a

FRMR response frame.

Primary/Secondary_versus_Balanced_Operation

There are two basic classes of link-level connections.  The first, known

as  Link Access Procedure (or LAP) is often called an unbalanced service

where the DCE is considered the primary (or master) devices and the DTEs

are considered secondary (or slave) devices. The second class of service

is known as LAPB, Link Access Procedure Balanced.  In this  service both

devices  are  treated  as equals as far as connection requests and other

types of commands.  There is still only  one  DCE  and  potentially many

DTEs, but both ends can command the link equally.

Primary/Secondary_(LAP)_Operation

LAP  is  the older style of link control, where most of the intelligence

was assumed to be in a large mainframe (the DCE) and the end users  were

just using smart terminals (the DTEs). Since network software can have a

lot of overhead, it made sense at the time to put most of  the  overhead

in the big computer, and just enough smarts to make the link work in the

terminals.

Balanced_(LAPB)_Operation

LAPB is a slightly modified version of LAP. It has been changed to allow

the  two  sides  of a link to operate in a more balanced manner.  In the

official version of X.25 there is still only one DCE to potentially many

DTEs, but the two can operate more as equals than master and slave.

LAPB  is  what this document describes for use over Amateur Radio packet

networks. Even when there is a network controller overseeing the network

operation, the balanced link procedure will enhance operation.

Connection_Operation

In  amateur  radio  network  operations, it would be very helpful if one

level 2 protocol would work with the various  rf  systems  in  use.   An

example  of  this  is  the  difference  in  operation  between  a simple

two-station  link,  and  multiple  stations  operating  thru  a  network

controller.    Obviously, when a network controller exists, it should be

considered the DCE, while the other stations connecting to it  would  be

the  DTEs.   A simple two-station connection is another matter.  To this

type of connection the station requesting a connection should always  be

considered  the  DTE,  while the device that is receiving the connection

request should operate as the DCE. This simple rule should eliminate any

ambiguity that might otherwise occur under these conditions.

NOTE:  There  are a couple minor changes from the official X.25 standard

in the protocol recommended  here.    These  changes  are  done  only as

absolutely  necessary  to work over the shared rf media.  Since X.25 was

written to work so that one DCE talked with  many  DTEs  over  a  closed

network,  it cannot properly cope with a channel where there may be many

DCEs linked to many DTEs.  Some amateurs have thrown X.25 out because of

this  problem.    It  seems  to  take just a couple minor changes in the

initial link set-up procedure to make X.25 work  properly  over  amateur

radio.    Where these changes are made, both the original X.25 procedure

and the recommended amateur procedure will be noted.

LAPB_Procedures

The  following  describes  the  procedures  used  to  set-up,  use,  and

disconnect a balanced link between a DTE and DCE.  These procedures have

been taken from X.25 and conform very closely to that  standard,  except

where it was necessary to change due to the radio environment.

Address_Field_Operation

All  transmitted  frames  shall  have address fields conforming to above

mentioned rules.  All frames should have both the destination device and

the  source  device addresses in the address field, with the destination

address coming first.  This will allow many links to share  the  same rf

channel. The destination address is always the address of the station(s)

to receive the frame, while the source address contains the  address  of

the  device that sent the frame.  The destination address can be a group

name or club call however, if point to multi-point operation is allowed.

This will be discussed further under link operations.

LAPB_Connection_Establishment

When a device (either a DCE or DTE) wishes to connect to another device,

it will send a SABM command frame to that device and  start  a  time-out

timer  (T1).   If the other device is there and able to connect, it will

answer with a UA response frame and at the same time reset both of  it's

internal  state  variables  (V(S)  and  V(R)).   The reception of the UA

response frame at the other end will cause  the  device  requesting  the

Nconnection  to  abort the T1 timer and set its internal state variables

to 0 also.

If the other device doesn't respond before  T1  times  out,  the  device

requesting  the  connection  will  re-send  the SABM frame, and start T1

running again. This trying to establish a connection will continue until

the  requesting device has tried unsuccessfully a number of times.  That

number (N1) is variable depending on the frequency of operation, type of

transmission (eg.  terrestial vs.  satellite), and the signaling speed n

use. N1 will be discussed in another section.

Information_Transfer

Once a connection has been established as outlined above,  both  devices

are able to accept I, S, and U frames.

Sending_of_I_Frames

Whenever  a station has an I frame to transmit, it will send the I frame

with N(S) of  Nth  control  field  equal  to  it's  current  send  state

variable  V(S).    Once  the I frame is sent, the send state variable is

incremented by one.

The station should not transmit any more I frames  if  it's  send  state

variable  equals  the  ast received N(R) from the other side of the link

plus seven.  If it were to send more I frames, the  flow  control window

would be exceeded and errors could result.

If  a  device is in a busy condition, it may still send I frames as long

as the other device is not also busy.

If a device is in the  frame-rejection  mode,  it  will  stop  sending I

frames.

Receiving_I_Frames

If  a  device receives a valid I frame (one with a correct FCS and whose

send sequence number equals the receiver's receive state  variable)  and

is  not  in  the  busy  condition,  it will accept the received I frame,

increment it's receive state variable, and act in one of  the  following

manner:

1.  If it has an I frame to send, that I frame may

be sent with the transmitted N(R)  equal  to  it's

receive  state  variable  V(R) (thus acknowledging

the received frame.  Alternately,  the  device may

send an RR frame with N(R) equal to V(R), and then

send the I frame.

2.  If  there  are  no  outstanding  I  frames,the

receiving  device  will send an RR frame with N(R)

equal to V(R).

If  the  device  is  in  a  busy condition, it may ignore any received I

frames  without  reporting  this  condition  other  than  repeating  the

indication of the busy condition.

If  a  busy  condition  exists, the station receiving the busy condition

indication should poll the sender of the  busy  indication  periodically

until the busy condition disappears.

The  reception  of  I frames that contain zero length information fields

shall be reported to the next level but no  information  field  will  be

transferred.

When  an  I frame is received with a correct FCS, but it's send sequence

number does not match the current receiver's receive state variable, the

frame  should be discarded and a REJ frame should be sent with a receive

sequence number equal to one higher (modulo 8) than the  last  correctly

received  I  frame  .    Any out-of-sequence received I frames should be

handled in this manner. The received state variable and poll bit in such

a  discarded  frame  should be checked before throwing it away, and take

any action needed depending on the condition of them.

Receiving_Acknowledgement

Whenever an I  or  S  frame  is  correctly  received,  even  in  a  busy

condition, the N(R) of the received frame should be checked to see if it

includes an acknowldegment of outstanding sent I frames.   The  T1 timer

should  be  reset if the received frame actually acknowledges previously

unacknowledged frames.  If the T1 timer is reset,  and  there  are still

some  frames that have been sent that are not acknowledged, T1 should be

started again.  If the T1 timer runs out  before  an  acknowledgement is

received, the device should proceed to the retransmission procedure.

Receiving_Reject

Upon  receiving  a REJ frame, the transmitting station will set its send

state variable to the same value are the REJ  frames  received  sequence

number  in  the  control  field.   The device will then retransmit any I

frame(s) outstanding at the next available opportunity conforming to the

following:

1. If the device is not transmitting at the  time,

and  the  channel is open, the device may commence

to retransmit the I frame(s) immediately.

2. If the device is operating  on  a  full  duplex

channel  transmittiong  a  U  or  S  frame when it

receives a REJ frame, it may finish finding  the U

or S frame and then retransmit the I frame(s).

3.  If  the  device  is operating in a full duplex

channel  transmitting  another  I  frame  when  it

receives  a REJ frame, it may abort the I frame it

was  sending  and  start  retransmission  of   the

requested I frames immediately.

4.  The  device  may  send  just  the  one I frame

outstanding, or it may send more than one  if  any

more   I   frames   followed  the  first  one  not

acknowledged, provided the total to be  sent  does

not exceed the flow control window (7 frames).

If  the  device  receives  a  REJ frame with the poll bit set, it should

respond with either an RR or RNR frame with the  final  bit  set  before

retransmitting the outstanding I frame(s).

Receiving_an_RNR_Frame

Whenever  a  device receives an RNR frame, it may transmit or retransmit

the I frame whose send sequence  number  equals  that  of  the  received

sequence number indicated in the RNR control field. If timer T1 runs out

after the RNR was received, the waiting acknowledgement procedure listed

below should be performed.  The poll bit may be used in conjunction with

S frames to test for a  change  in  the  condition  of  the  busied  out

station.  No I frames other than the one mentioned above may be sent out

before the busy condition is cleared.

Sending_a_Busy_Indication

Whenever a device enters a busy condition,  it  will  indicate  this  by

sending an RNR response at the next opportunity.  While the device is in

the busy condition, it may  receive  and  process  S  frames,  and  if a

received  S frame has the P bit set to one, the device should send a RNR

frame with the F bit set to one at the next  possible  opportunity.   To

clear  the  busy  condition,  the  device should send either a RR or REJ

frame with the received sequence number equal  to  the  current  receive

state  variable,  depending  on  whether  the  last received I frame was

properly received or not.

Waiting_Acknowledgement

The device should maintain an  internal  retransmission  count  variable

which  is  set  to zero whenever another I frame is acknowledged (either

thru the reception of a UA or RNR frame, or when a received I or S frame

has   an   N(R)   higher  than  the  last  received  N(R),  showing  the

acknowledgement of additional I frames).

Any time the timer T1 runs out,  the  device  will  re-enter  the  timer

recovery   condition,   the   retransmission   count  variable  will  be

incremented by one, and another internal variable (X) will be set to the

current send state variable value.

The  device  will  then  restart  the  T1  timer,  set its receive state

variable to  the  last  receive  sequence  number,  and  retransmit  the

corresponding I or S frame with the P bit set to one.

The timer recovery condition is cleared when the device receives a valid

S frame with the F bit set to one.

If the device receives an S frame with the F bit set  to  one  and  N(R)

within  the  range  from  the current send state variable to X mentioned

above inclusive while in the timer recovery  condition,  this  condition

will  be  cleared,  and  the send state variable will be set to the N(R)

received.

If the device receives an S frame  with  the  F  bit  set  to  zero  but

otherwise  the  same condition as the last paragraph, the timer recovery

condition will NOT be cleared.  The received N(R) may be used however to

update  the  send  state variable.  The device may keep the last I frame

transmitted (even if it was acknowledged) to be retransmitted with the P

bit set to one if timer T1 expires at a later time.

Once  the  retransmission  count  variable reaches N2, the device should

proceed to the resetting procedures outlined below.

Link_Disconnection

When in the information-transfer phase,  either  device  may  initiate a

link disconnection by sending a DISC frame.  It should then start its T1

timer, and wait for a response.  If  the  proper  response  doesn't come

before T1 times out, it should send the DISC frame again and restart T1.

If this happens N2 times,  the  device  should  enter  the  disconnected

state.

When  a DISC frame is received, the receiver should return a UA response

frame, and enter the disconnected state.

Disconnected_State

After having sent a DISC frame and received a UA, or  receiving  a  DISC

and having sent a UA, the device will enter the disconnected state.

In  the  disconnected  state,  the  device may initiate a link set-up as

outlined in connection establishment above.  It may also respond  to the

reception  of  a  SABM  and establish a connection, or it may ignore the

SABM and send a DM instead.

Any station receiving a DISC command while  in  the  disconnected  state

should send back a DM response frame.

Any  device receiving a command frame other than a SABM or UI frame with

the P bit set to one should respond with a DM frame with the F  bit  set

to one. The offending frame should also be ignored.

When  the  device enters the disconnected state after an error condition

or if it has recovered from an internal error condition by coming up  in

the disconnected state, it should indicate this by sending a DM response

rather than a DISC frame.  It should start the T1 timer when  the  DM is

sent,  and  if T1 times out before getting a SABM or DISC frame back, it

should send another DM frame, and restart T1.  After  retransmitting the

DM frame N2 times, the device will remain in the disconnected state, and

no other action will be taken.

Resetting_Procedure

The resetting procedure is used to initialize both  directions  of  flow

after  a  nonecoverable  error has occured.  This resetting procedure is

only used when in the information transfer phase of an AX.25 link.

A device shall request a reset by sending an SABM frame.  Upon receiving

an SABM frame from a station previously connected to, the receiver of an

SABM frame should send a UA frame back at the earliest opportunity. Both

devices  should then set their send and receive state variables to zero.

Any busy condition that previously existed will also be cleared.

It is possible to initiate a disconnect procedure instead  of  resetting

the link.

One  device may ask the other to reset the link by sending a DM response

frame.  After the DM frame is sent, the sending device  will  then enter

the disconnected state.

One  device may ask the other to initiate a link reset by transmitting a

FRMR response frame.

After sending the FRMR frame, the sending device will  enter  the  frame

reject  state.   This condition is cleared when the device that sent the

FRMR frame receives an SABM or DISC command, or a DM response frame. Any

other  command  received  while  the device is in the frame reject state

will cause another FRMR to be sent out with the same  information  field

as originally sent.

The  device  that sent the FRMR frame should start the T1 timer when the

FRMR is sent.  If above mentioned frames  are  not  received  before the

timer runs out, the FRMR frame should be retransmitted, and the T1 timer

restarted as described in the waiting acknowledgement section above.  If

the FRMR is sent N2 times without success, the link should be reset.

Rejection_Conditions

A  device  should  initiate  the  link-reset  procedure  when a frame is

received with the correct FCS and address field during  the  information

transfer phase with one or more of the following conditions:

1. The frame is not known as a command or response

to the device.

2. The information field is invalid (as an example

is longer than 256 octets)

A  device  will  initiate a reset procedure whenever it receives a DM or

FRMR response frame during the information transfer phase.

A device may initiate a reset procedure also whenever it receives  a  UA

response  frame or if it receives an unsolicited response frame with the

F bit set to one.

Collision_Recovery

Collisions_in_a_Half-Duplex_Enviroment

Collisions of frames  of  any  type  in  a  half-duplex  enviroment  are

essentially  taken  care  of  by  the  retry  nature of the T1 timer and

retransmission count variable.  No  other  special  action  needs  to be

taken.

Collisions_in_a_Full-Duplex_Enviroment

Collisions  in a full-duplex enviroment are not really frame collisions,

but have more to do with the  devices  being  pulled  in  two  different

directions at the same time.

Collisions_of_Unnumbered_Commands

If  the  sent  and  received U command frames are the same, both devices

should send a UA response at the earliest opportunity, and both  devices

should enter the indicates state.

If  the  sent and received U commands are different, both devices should

enter the disconnected state, and transmit a DM frame  at  the  earliest

opportunity.

Collision_of_a_DM_with_a_SABM_or_DISC

When  an  unsolicited  DM response frame is sent, a collision between it

and a SABM or DISC may occur.  In order to prevent  this  DM  from being

misinterpreted, all unsolicited DM frames should be transmitted with the

F bit set to zero.  All SABM and DISC frames should be sent  with  the P

bit  set  to  one,  so  there  isn't  any  confusion  when a DM frame is

received.

Connectionless_Operation

In Amateur Radio circles, there is a type of operation that isn't really

feasable  using  level 2 connections.  This operation is the roundtable,

where several amateurs may be engaged in one conversation.  The only way

to  accomplish  this  in  a  connected  mode  would  be to have everyone

crossconnected with each other.  This would require a seperate  frame to

be  sent  to  each  member  of  the  roundtable  every time someone says

something.  Obviously, this mode is not practical.  The way most amateur

packet  radio  enthusiasts  have  ended  up  implementing the roundtable

operation is outside the AX.25 connection, but  still  using  the  AX.25

frame  structure.   AX.25 does allow a special frame for this operation,

called the Unnumbered Information (UI) frame.   It  is  recommended that

when  this  type  of operation is in use, the destination address have a

code word installed in it  to  prevent  the  users  of  that  particular

roundtable  from  seeing all frames going thru the shared RF medium.  An

example of this is if a group of amateurs are in a roundtable discussion

about packet radio, they could put PACKET in the destination address, so

they would only receive frames from others in the same  discussion.   An

added advantage of the use of AX.25 in this manner is that the source of

each frame is in the source address sub-field, so re could be written to

automatically display who is making what comments.

Admittedly, this is a kludge to the level 2 AX.25 protocol. This type of

operation really belongs at the next layer (layer 3,  packet  level)  of

operation  but  until  layer  3  is  implemented,  this appears to be an

acceptable substitute.

Keep in mind that this mode is connectionless, so all transmitted frames

should   be   of  good  quality,  as  there  will  be  no  requests  for

retransmissions of bad frames.  Collisions  will  also  occur,  with the

potential of losing the frames that collided.

List_of_System_Defined_Parameters

Timers

It  is  recommended  that  there  are  two  timers  used to maintain the

integrity of the AX.25 layer 2 connection.

The first timer, T1, is used to make sure a device doesn't wait  forever

for  a  response to a frame it sends.  This timer cannot be expressed in

absolute time, since the time required to  send  frames  varies  greatly

with the baud rate used at level 1. T1 should be at least twice the time

it would take to send a maximum length frame to the  other  end  of  the

link,  and  get the proper response frame back from the other end of the

link.  This would allow time for the other end of the  link  to  do some

processing before responding.

The  second  timer,  T2,  is used whenever T1 isn't running to make sure

that  a  supervisory  frame  is  sent  periodically  to  maintain   link

integrity.   It  also  will  vary  dramatically  depending  on  layer 1

constraints, and is subject for further study.

Maximum_Number_of_Retrys_(N2)

The maximum number of retrys is used in conjunction with the  T1  timer.

It  will  vary  depending  on  the layer 1 in use, but will generally be

sixteen.

Maximum_Number_of_Octets_in_an_I_Field_(N1)

The maximum number of octets allowed in the I field will be 256.   There

should also be an even multiple number of octets.

Maximum_Number_of_I_Frames_Outstanding_(k)

The maximum number of outstanding I frames at a time is seven. A smaller

number may be used at any time, provided it  is  agreed  upon  ahead  of

time.

First

Bit Sent

-----------------------------------------------------------

|   Flag   |  Address   | Control |    FCS     |  Flag    |

|---------------------------------------------------------|

| 01111110 |112/168 Bits| 8  Bits |  16  Bits  | 01111110 |

-----------------------------------------------------------

Figure 1A.  U and S Frame Construction

First

Bit Sent

------------------------------------------------------------------------

|   Flag   |  Address   | Control|  PID  | Info.   |  FCS   |  Flag    |

------------------------------------------------------------------------

| 01111110 |112/168 Bits| 8 Bits | 8 Bits| N*8 Bits| 16 Bits| 01111110 |

------------------------------------------------------------------------

Figure 1B.  Information Frame Construction

First

Octet Sent

--------------------------------------------------

|             Address Field of Frame             |

|------------------------------------------------|

| Destination Address |     Source Address       |

|------------------------------------------------|

|A1 A2 A3 A4 A5 A6 A7 | A8 A9 A10 A11 A12 A13 A14|

--------------------------------------------------

Figure 2A.  Non-Repeater Address Field Encoding

First

Octet Sent

----------------------------------------------------------------------------

|                       Address Field of Frame                             |

|---------------------------------------------------------------------------

| Destination Address|     Source Address      |    Repeater  Address      |

|--------------------------------------------------------------------------|

|A1 A2 A3 A4 A5 A6 A7|A8 A9 A10 A11 A12 A13 A14|A15 A16 A17 A18 A19 A20 A21|

----------------------------------------------------------------------------

Figure 2B.  Repeater Address Field Encoding

Appendix A.  Non-Repeater Frame Example

-----------------------------------

| Octet | ASCII |Bin.Data|Hex Data|

-----------------------------------

| Flag  |       |01111110|   7E   |

|  A1   |   K   |10010110|   96   |

|  A2   |   8   |01110000|   70   |

|  A3   |   M   |10011010|   9A   |

|  A4   |   M   |10011010|   9A   |

|  A5   |   O   |10011110|   9E   |

|  A6   | space |01000000|   40   |

|  A7   | SSID  |01100000|   60   |

|  A8   |   W   |10101110|   AE   |

|  A9   |   B   |10000100|   84   |

|  A10  |   4   |01101000|   68   |

|  A11  |   J   |10010100|   94   |

|  A12  |   F   |10001100|   8C   |

|  A13  |   I   |10010010|   92   |

|  A14  | SSID  |01100001|   61   |

|Control| SABM  |00111111|   3F   |

|  PID  | none  |11110000|   F0   |

|  FCS  | part 1|XXXXXXXX|   HH   |

|  FCS  | part 2|XXXXXXXX|   HH   |

| Flag  |       |01111110|   7E   |

-----------------------------------

The  frame  shown  is  an SABM frame, not going thru a level 2 repeater,

from WB4JFI (SSID=0) to K8MMO (SSID=0), with no level 3 protocol.

Appendix B.  Repeater Type Operation

| Octet | ASCII |Bin.Data|Hex Data|

|---------------------------------|

| Flag  |       |01111110|   7E   |

|  A1   |   K   |10010110|   96   |

|  A2   |   8   |01110000|   70   |

|  A3   |   M   |10011010|   9A   |

|  A4   |   M   |10011010|   9A   |

|  A5   |   O   |10011110|   9E   |

|  A6   | space |01000000|   40   |

|  A7   | SSID  |01100000|   60   |

|  A8   |   W   |10101110|   AE   |

|  A9   |   B   |10000100|   84   |

|  A10  |   4   |01101000|   68   |

|  A11  |   J   |10010100|   94   |

|  A12  |   F   |10001100|   8C   |

|  A13  |   I   |10010010|   92   |

|  A14  | SSID  |01100000|   60   |

|  A15  |   W   |10101110|   AE   |

|  A16  |   B   |10000100|   84   |

|  A17  |   4   |01101000|   68   |

|  A18  |   J   |10010100|   94   |

|  A19  |   F   |10001100|   8C   |

|  A20  |   I   |10010010|   92   |

|  A21  | SSID  |11100011|   E3   |

|Control| SABM  |00111111|   3F   |

|  PID  | none  |11110000|   F0   |

|  FCS  | part 1|XXXXXXXX|   HH   |

|  FCS  | part 2|XXXXXXXX|   HH   |

| Flag  |       |01111110|   7E   |

-----------------------------------

The above frame is the same as in Appendix A, but with the addition of a

repeater address subfield (WB4JFI, SSID=1). The H bit is set, indicating

this is from the output of the repeater.

Appendix C.

Advantages of the WB4JFI Addressing Scheme

Some of the advantages to using this addressing system are:

1.  Every  packet station will have a unique fixed

address that  doesn't  change  every  time  a  new

network is logged into.

2.  Relocating to a new area won't cause major (or

minor) problems.

3. Allows for more than 62 or 31 users at a time.

4. No  local  packet  guru  is  needed  to  assign

addresses  with  attendant  concerns of backup and

transfer during failure.

5. Direct or network operation requires no  change

of address.

6.     All     the     problems    with    dynamic

allocation/de-allocation are eliminated.

7. Reduces local co-network  interference  due  to

users in overlapping local network rf domains with

the same address fields.

8. With every frame having  both  the  destination

and  source  addresses  in  them, it will be a lot

easier to set-up and run multiple  connections  on

the  same  data  channel  without  having problems

arise as to who is sending what frames to whom.

9. In round-table operation, every frame sent will

have  the  source address imbedded in it, allowing

automatic printing of the source of the frame.

Appendix D.  Layer 2 AX.25 State Table

-----------------------------------------------------------------------------------------------------------------------------------

|    State          |I with|I with-|RR with|RR with-|REJ with|REJ with-|RNR with|RNR with-| SABM | DISC |  UA   |   DM   |  FRMR  |

|                   | Poll |out  P | Final |out  F  | Final  |out  F   | Final  |out  F   |either|either| either| either | either |

|---------------------------------------------------------------------------------------------------------------------------------|

|S1 Disconnected    |  DM  |       |  DM   |        |   DM   |         |   DM   |         |UA,S5 | DM   |       |SABM,S2 |        |

|---------------------------------------------------------------------------------------------------------------------------------|

|S2 Link Setup      |      |       |       |        |        |         |        |         |  UA  |DM,S1 |   S5  |   S1   |        |

|---------------------------------------------------------------------------------------------------------------------------------|

|S3 Frame Reject    | FRMR |       | FRMR  |        |  FRMR  |         |  FRMR  |         |UA,S5 |UA,S1 |       |SABM,S2 | SABM,S2|

|---------------------------------------------------------------------------------------------------------------------------------|

|S4 Disconnect Rqst |      |       |       |        |        |         |        |         |  DM  |  UA  |   S1  |   S1   |        |

|---------------------------------------------------------------------------------------------------------------------------------|

|S5 Information Xfr |  RR  |   I   |   I   |   I    |   I    |    I    |   S9   |   S9    |  UA  |UA,S1 |SABM,S2|SABM,S2 | SABM,S2|

|---------------------------------------------------------------------------------------------------------------------------------|

|S6 REJ Frame Sent  | RR,S5|  I,S5 |   I   |   I    |   I    |    I    |   S15  |   S15   |UA,S5 |UA,S1 |SABM,S2|SABM,S2 | SABM,S2|

|---------------------------------------------------------------------------------------------------------------------------------|

|S7 Waiting Acknow. |  RR  |   I   |  I,S5 |   I    |  I,S5  |    I    |   S9   |   S12   |UA,S5 |UA,S1 |SABM,S2|SABM,S2 | SABM,S2|

|---------------------------------------------------------------------------------------------------------------------------------|

|S8 Device Busy     | RNR  |  RNR  |   I   |   I    |   I    |    I    |   S10  |   S10   |  UA  |UA,S1 |SABM,S2|SABM,S2 | SABM,S2|

|---------------------------------------------------------------------------------------------------------------------------------|

|S9 Remote Device   |  RR  |  RR   |  I,S5 |  I,S5  |  I,S5  |  I,S5   |        |         |UA,S5 |UA,S1 |SABM,S2|SABM,S2 | SABM,S2|

|     Busy          |      |       |       |        |        |         |        |         |      |      |       |        |        |

|---------------------------------------------------------------------------------------------------------------------------------|

|S10 Both Devices   | RNR  |  RNR  |  I,S8 |  I,S8  |  I,S8  |  I,S8   |        |         |UA,S8 |UA,S1 |SABM,S2|SABM,S2 | SABM,S2|

|     Busy          |      |       |       |        |        |         |        |         |      |      |       |        |        |

|---------------------------------------------------------------------------------------------------------------------------------|

|S11 Waiting Acknow.| RNR  |  RNR  |  I,S8 |   I    |  I,S8  |    I    |   S10  |   S13   |UA,S8 |UA,S1 |SABM,S2|SABM,S2 | SABM,S2|

| and Device Busy   |      |       |       |        |        |         |        |         |      |      |       |        |        |

|---------------------------------------------------------------------------------------------------------------------------------|

|S12 Waiting Acknow.|  RR  |   RR  |  I,S5 |  I,S7  |  I,S5  |   I,S7  |        |         |UA,S5 |UA,S1 |SABM,S2|SABM,S2 | SABM,S2|

| and Remote Busy   |      |       |       |        |        |         |        |         |      |      |       |        |        |

|---------------------------------------------------------------------------------------------------------------------------------|

|S13 Waiting Acknow.| RNR  |  RNR  |  I,S8 |  I,S11 |  I,S8  |  I,S11  |        |         |UA,S8 |UA,S1 |SABM,S2|SABM,S2 | SABM,S2|

| Both Devices Busy |      |       |       |        |        |         |        |         |      |      |       |        |        |

|---------------------------------------------------------------------------------------------------------------------------------|

|S14 REJ Sent and   | RNR  |  RNR  |   I   |   I    |   I    |    I    |   S16  |   S16   |UA,S8 |UA,S1 |SABM,S2|SABM,S2 | SABM,S2|

| and Device Busy   |      |       |       |        |        |         |        |         |      |      |       |        |        |

|---------------------------------------------------------------------------------------------------------------------------------|

|S15 REJ Sent and   |RR,S9 | RR,S9 |  I,S6 |  I,S6  |   I,S6 |   I,S6  |        |         |UA,S5 |UA,S1 |SABM,S2|SABM,S2 | SABM,S2|

| Remote Busy       |      |       |       |        |        |         |        |         |      |      |       |        |        |

|---------------------------------------------------------------------------------------------------------------------------------|

|S16 REJ Sent and   | RNR  |  RNR  | I,S14 |  I,S14 |  I,S14 |  I,S14  |        |         |UA,S5 |UA,S1 |SABM,S2|SABM,S2 | SABM,S2|

| Both Devices Busy |      |       |       |        |        |         |        |         |      |      |       |        |        |

-----------------------------------------------------------------------------------------------------------------------------------

######################