��������������������������ͻ

�  THAPN v1.01 desription  �

�           HA5DI          �

�      March 5, 1994.      �

��������������������������ͼ

1) General, Comments:

=====================

TFHAPN  is a WA8DED host mode compatible TSR driver, written for the old

HAPN1 PC board, equipped with INTEL 8273 HDCL controller and. The program

can be run on any PC or clones with 80286 or higher CPU.

This program is based on TFKISS v1.02, which program is handling

different type of KISS TNC's. In this document the differences are

described. For more information, read TFKISS.ENG, TF26.ENG and

HAPN-API.DOC files.

TFHAPN has been tested with SP 5.0 under DOS and WINDOWS 3.1 as well as

with FBB 5.15c version. GP or other TFPCR/TFPCX/TFKISS compatible

terminal or BBS programs can be used, too.

2) Installation:

================

At first, install HAPN1.SYS device driver, adding the following line to

your CONFIG.SYS line:

DEVICE=HAPN1.SYS

If you have the standard HAPN1 board, the default parameters can be used,

to add optional driver parameters is not necessary.

Next, install TFHAPN. In most cases, you do not need any optional

parameters, driver will work fine with it's default.

Finally, start your application program. Don't forget to make the

required changes in the configuration files, according to the relevant

documentation.

3) The Command Line:

====================

The Command Line Syntax has been created the same as the Parameter

convention of the MS-DOS Version. All Parameters must be preceded by a

hyphen - or a Backslash / . Numerical Values are separated from the

Command by a Colon : .

After the Start, the Screen output appears as follows:

������������������������������������������������������ͻ

� TFHAPN v1.01.  10 Channels Int FE  Pars at 08B1:51BF �

� Based on TF 2.6a by NORD><LINK.     Rev: Mar 05 1994 �

� (c) OE1HHC & OE3GMW.    Free for non-commercial use. �

������������������������������������������������������ĺ

� (c) HAPN adaptation by HA5DI                         �

������������������������������������������������������ͼ

TFHAPN -? shows a small Help Text:

Usage:

TFHAPN [[-?][-i:xx][-l:nn][-m:nn][-u]]

?  or  h  this text

i  TFKISS int vect xx (FE)

l  max channel number (10)

m  memory usage nn kB (61)

u  unload TFKISS

In Brackets issued numbers are those pre-installed Values with which

TFHAPN starts, chiefly when no Command Line Parameter is given. They

are interpreted as Decimal Numbers with the exception of -i.

The i-Parameter

determines the Software Interrupt for communication with the

Hostmode Terminal Program. It should be altered only in

exceptional cases and must be given as Hexadecimal.

Standard Value is 0FEh.

The l-Parameter

determines the maximum number of selectable Channels. All

TFPCR Versions prior to 3.30 were compiled for a fixed Quantity

of available Channels, now it is installable.

The RAM demand by TFHAPN alters itself accordingly.

Permissable Values are 1 to 40.

Standard Value is 10.

The m-Parameter

limits the RAM consumption (in Kilobytes) and makes possible

with it, a controlling influence on the RAM management.

If -m:n is not issued, TFHAPN searches for 800 Frame Buffer

places. Instead  approximately 54 kB of main RAM will be needed

(see also TF-2.6 DOC - @B Command).

Minimum Space demand is (dependent upon L-Parameter) 36..40 kB,

then there are certainly only 300 more Frame Buffers for

disposal.

The Maximum Value can amount to 64.

Admissible Values for nn are therefore 36 to 64.

The u-Parameter

removes TFHAPN completely out of the RAM and sets all System

Vectors that were altered by the Program back to their Original

Value.

4) Hostmode-Program:

====================

4.1 SP

------

CFG=PORTx:5

or

CFG=PORTx:5h  if TFHAPN is to be load High (LH TFHAPN; highly recommended

when the minimum RAM available is 35 kB UMB.

4.2 GP

------

GP recognises TFHAPN itself, here no additional declarations are

necessary.

4.3 FBB

-------

An example for PORT.SYS, version 5.15 or later:

#

#Com Interface Address (Hex) Baud

8    5         0             0

#

#Tnc NbCh Com MultCh Paclen Maxfr NbFwd MxBloc M/P-Fwd Mode Freq

1    10   8   0      230    2     3     10     00/15   UD   70cm

#

The keys are the interface type 5 and mode D parameters.

5) Anatomy. About the Inner Life of TFHAPN:

===========================================

This chapter is dedicated to the Internal workings.

5.1) The default Parameter Block

In the Greeting Test is the Advice: "Pars at xxxx:xxxx" to read. It

behaves thereby as a Pointer (Segment:Offset) which shows on the "default

Parameter Block". In this Structure will be saved Declarations for "The

Firmware" and the so called Level-1 Parameters.

In the case where you can genuinely bypass the corresponding Help Programs

(Diskedit, Debug, ...), you have the possibility, to patch your personal

Default Values.

With a Disk-Monitor, find the corresponding place in the COM-File, in

which can be deduced from the displayed Offset Value 256 (100h). Also will

be issued e.g. "Pars at 0B73:571E", the Parameter-block begins in

TFHAPN.COM at File position 561Eh.

WARNING: Make the alteration with great care.

For this purpose only use a Backup Copy.

Never work on the Original !

The Parameter block has a size of 124 Bytes and is constructed as follows

(Words are laid down in INTEL-Format Low/High-Byte):

Offset   Size     Default Value       Explanation

0     BYTE         0FEh         TFKISS-Int-Vector (-i)

1     WORD           0          not used, must be zero

3     BYTE           0          not used, must be zero

4     WORD           0          not used, must be zero

6     WORD         101h         TFHAPN-Version Number (1.01)

8     BYTE          10          maximum Channel number (-l)

9     BYTE          20h         MYCALL    (TF:  I)

10     BYTE          20h           .

11     BYTE          20h           .

12     BYTE          20h           .

13     BYTE          20h           .

14     BYTE          20h           .

15     BYTE          60h           .

16     BYTE          'C'         UNPROTO   (TF:  C on Monitor Channel)

17     BYTE          'Q'           .

18     BYTE          20h           .

19     BYTE          20h           .

20     BYTE          20h           .

21     BYTE          20h           .

22     BYTE          60h           .

23     BYTE           0           Valid Characters in CTEXT (max. 72!)

24     BYTE          20h         CTEXT      (TF:  U)

25     BYTE          20h           .

.       .            .            .

.       .            .            .

94     BYTE          20h           .

95     BYTE          20h           .

96     BYTE           0          DUPLEX     (TF: @D)

97     WORD         250          FRACK      (TF:  F)

99     WORD           7          SRTT A1    (TF: @A1)

101     WORD          15          SRTT A2    (TF: @A2)

103     WORD           3          SRTT A3    (TF: @A3)

105     BYTE           0          MONITOR    (TF:  M)     00h...0Fh:

Bit 0: I, Bit 1: U, Bit 2: S, Bit 3: C

106     BYTE          10          RETRY      (TF:  N)

107     WORD          60          IPOLL      (TF: @I)

109     BYTE           2          MAXFRAME   (TF:  O)

110     BYTE          32          PERSIST    (TF:  P)

111     BYTE           0          DIGIPEAT   (TF:  R)

112     BYTE           0          Time Stamp  (TF:  K)     0...2:

0 = Stamp off

1 = for Status Reports

2 = for Status- and Monitor Reports

113     BYTE          25          TXDELAY    (TF:  T)

114     BYTE           0          CTEXT      (TF:  U)

115     BYTE          10          SLOTTIME   (TF:  W)

116     BYTE           4          MAXUSERS   (TF:  Y)

117     BYTE           0          CALLCHECK  (TF: @V)

118     BYTE           0          UIPOLL     (TF: @U)

119     WORD         150          TIMER2     (TF: @T2)

121     WORD       18000          TIMER3     (TF: @T3)

123     WORD         120          DAMATIMER  (TF: B)

5.2) The Soft-Interface

TFHAPN communicates with the Terminal Programs via a Software Interrupt.

The following sub-Functions are implemented:

Function: STATUS request

checks whether a Character is ready to be fetched.

Register AH: 1

Return     : If   AL = 0  nothing there,

AL = 1  Character ready to be collected.

Function: GET

Fetch Character. Only called after Function has reported

that 1 Character is ready!

Register AH: 2

Return          : AL = Character

Function: PUT

Character sent to TFHAPN.

Register AH: 3

Register AL: Character

Return     : none

Function: GET POINTERS

Ascertain the Pointer on the "default Parameter Block" and

on the "Debug/Error Counter" of an already resident TFHAPN.

Register AH: 5

Return     : When AX = 0  Result valid.

Other cases Result Invalid.

ES:BX : Far-Pointer on the default Parameter Block.

ES:DX : Far-Pointer on the Debug/Error Array.

Expanded Functions compatible with TFPCX 2.0:

Function: Request Port and Channel Number

Register AH: FB

Return     :

AL: Number of the Hardware-Ports (always 1)

AH: Number of installed Channels (Y-Command)

Function: Send- and Receive Status

Register AH: FC

Return     :

AL: Receive Status, 0..inactive  1..Data arrives

AH: Send Status, 0..inactive  1..it will send Data

Rem.: This function is not implemented in TFHAPN yet.

Function: TX-Busy Flag

Register AH: FD

Return     :

AX:  0..TX-Busy (insufficient Frame Buffers available)  1..ok

Function: get Driver Type

Register AH: FE

Return     :

AX: Bits: 15 14 13 12    11 10 09 08    07 06 05 04 03 02 01 00

Driver Class  Main Version   Subversion (0..99 dec)

TFKISS has the Driver Class  E

(TFPCR (DL1MEN Original) -"-  F

TFPCX Version 2.0       -"-  0

TFHAPN                  -"-  D

5.3) The Code

TFHAPN has been programmed 90 % in C. The remaining 10 % has been coded in

Assembler. The Source Code has an overall quantity of approximately. 13.100

Lines (no, that is not a Typing Error!). Has been Compiled with Borland

C++ 3.1.

6) Copyright:

=============

TFHAPN is based on TFKISS, replacing the Level1 functions by HA5DI. Other

parts of the original code are untouched.

TFKISS is a developed and basic updated New version of TFPCR, implemented

by OE1HHC and OE3GMW.

Author of TFPCR Version 2.xx was DL1MEN. For that purpose it employed

Program Parts from THENET by DF2AU, DC4OX and others namely unknown

Authors.

At this point I have to thank Harald OE1HHC, for the source code of

THKISS.

TFHAPN is public domain. It may be freely copied and distributed for non-

commercial Usage.

The Program has been tested carefully and all known Errors removed.

The Authors however are in no case responsible for malfunctions or damage

of any type, which could be the result of the employment of TFHAPN

directly or indirectly.

7) Final words:

===============

This is the first distributed version of TFHAPN. Any notes, reports are

welcome. Please send messages to:

AX.25:      ha5di@ha5di.hun.euro

or

EMAIL:      ha5di@ha5kfu.sch.bme.hu, [44.156.1.13]

The full distribution package and the possible new releases can be

downloaded from ha5kfu.sch.bme.hu with FTP.

And now, let me allow to copy a part of the TFKISS.ENG file.

"Now what: TFKISS is a Product of our Hobby, please remember,

before the sending of bitter, angry  Mails to ALL @ WWW. Serious Bug

Reports and Improvement suggestions are very welcome here!"

Budapest, March 5, 1994.

B�la, HA5DI