TNCEmu v1.0                         (May 7, 1996)

-----------

by Jose Carlos EB7CJO

Index

-----

1. Introduction

2. Getting started

3. Parameters

4. Program handling

5. Problems with Winpack 5.4

6. History

7. To do

8. Greetings

9. About the author

1. Introduction

---------------

Before you start reading, please have in mind that english is not my native

languaje, so if you find a spelling or syntactic mistake, help me to correct

it!

TNCEmu is a MS-DOS program which can, as its name suggests, emulate a TNC.

That is, having a second PC and a Baycom modem, you can use it as if it were

a TNC, with all advantages it has (to use Windows software, etc).

It can controls not only a Baycom modem, but any other device, provided you

have the suitable driver.  So TNCEmu can also be used as a "device adaptor",

controlling a non-standard device with programs that can only handle

serial-port TNCs.

This program's main utility is to save to the user the cost of a TNC,

provided you have a second PC.  This PC, which makes the emulation, only

needs following hardware:

- Motherboard, 640 Kb

- 2 RS-232 serial ports controller

- Disk unit (or Hard disk)

- Any video board

- Null-modem cable

It is not needed any more perypheral: monitor, keyboard, hard disk, etc.

The video board is useful for the PC to be able to boot.  If you don't attach

a keyboard, you will have to disable a Setup option which says "Wait for F1

when error" or something like that, because not to have a keyboard will cause

a boot error, and if you don't disable it, the PC will remain waiting for you

to press a non-existent F1 key.

The PC speed hasn't to be very high, but if you use BPQ and high baud rates

(9600 bps) it may cause characters to be missed, which can be solved

decreasing the baud rate.  Processor type can be any of 80x86 family

(including XT).

TNCEmu can handle 3 driver types for controlling the modem device, and also

it can do 3 emulation types (depending of the driver):

Driver        Firmware

------        --------

TFPCX         WA8DED (TF)

BPQ           TAPR

BPQ External  KISS

2. Getting started

------------------

For the impatient, this section will help to quickly configure TNCEmu,

so you can see how it works and begin to use it immediately.

When you execute "TNCEmu /?", you will get the following help text:

TNCEmu 1.0

(c) Jose Carlos EB7CJO

Format: TNCEMU [COMNum Speed] [/bpq:n] [/kiss] [/int14] [/noflow]

COMNum    = Serial port number for null modem       (default 1)

Speed     = Serial port baud rate                   (default 9600)

/bpq[:n]  = Use BPQ driver (COMn) instead of TFPCX  (default TFPCX)

/kiss     = Use BPQ external driver on int. 0x60    (like BPQAX25.COM)

/int14    = Use serial port driver (COMBIOS/MBBIOS)

/noflow   = Without flow control (RTS/CTS handshaking)

In the following section of this document you will find a detailed

explanation of every parameter.  As you can see, COMNum and Speed are

referred to the serial port which is connected to the other PC through

a null modem cable.  /int14 and /noflow parameters also have effect over

this serial port.  /bpq and /kiss select the driver type (and so the

emulation type) that will be used to control the Baycom modem.

The more versatile manner to get TNCEmu started is to save it in a disk,

which will be used to boot the PC that will act as a TNC.  For that you will

have to format an autoboot disk (format a: /s), copy all needed files and

create CONFIG.SYS and AUTOEXEC.BAT files in the root directory.

A CONFIG.SYS example might be:

SWITCHES=/F

BUFFERS=20,0

SHELL=A:\COMMAND.COM A:\ /P

DOS=HIGH

DEVICE=A:\HIMEM.SYS /TESTMEM:OFF

FILES=20

STACKS=9,256

Some things like SWITCHES, DOS=HIGH, HIMEM.SYS aren't strictly needed,

but will help to have a half-decent MS-DOS and, in case of SWITCHES, a

quick boot.

The AUTOEXEC.BAT file will vary depending of the wanted emulation type.

You can make a generic file so it will be able to change easily the emulation

type.  Here you have mine:

@echo off

prompt $p$g

path A:\DOS;A:\

rem *** Put in following line (goto): TAPR, DED, KISS, WINGT or TPK

goto TAPR

rem --------------------------- BPQ Node + TAPR ---------------------------

:TAPR

cd bpq

bpqax25 -I3 -B2f8 -T40 -cd

bpqcode

bpqhtnc2

cd ..

tncemu 1 4800 /bpq:5

goto FIN

rem -------------------------- WA8DED Host Mode ---------------------------

:DED

tfpcx286 -pCOM2 -b1200

tncemu 1 4800

tfpcx286 -u

goto FIN

rem ------------------ WA8DED Host Mode without RTS/CTS -------------------

:WINGT

tfpcx286 -pCOM2 -b1200

tncemu 1 4800 /noflow

tfpcx286 -u

goto FIN

rem -------------------------------- KISS ---------------------------------

:KISS

bpq\bpqax25 -I3 -B2f8 -T40 -cd

tncemu 1 4800 /kiss /noflow

goto FIN

rem ------------------------------- TNCDED --------------------------------

:TPK

tfpcx286 -pCOM2 -b1200

tncded 5

tncemu 1 4800 /bpq:5

tncded -u

tfpcx286 -u

goto FIN

:FIN

In my case I have a Baycom modem (1200 baud) on COM2, and a null modem

cable on COM1 at 4800 bits/sec.  The PC is a 33MHz 386SX, with a standard

serial port controller (16450/8250).  BPQ is located in a separate directory.

With this hardware it seems that when I use BPQ with 9600 bits/sec. some

characters are missed in bursts of 4-5 chars.  But with TFPCX or AX25.COM

(KISS) there are no problems with speed.

I suggest to change some of the BPQCFG.TXT parameters (don't forget to

run BPQCFG.EXE after making the changes):

DESQVIEW=0

NODECALL=EB7CJO-1

NODEALIAS=CADIZ

BBSCALL=EB7CJO

BBSALIAS=CJO

UNPROTO=EA7RKC

NODESINTERVAL=0

IDINTERVAL=10

BBS=1

NODE=1

TNCPORT

COM=5

ENDPORT

PORT

ID=Baycom_Modem

PORTNUM=1

TYPE=EXTERNAL

PROTOCOL=KISS

IOADDR=3F8H

INTLEVEL=96

SPEED=9600

CHANNEL=A

QUALITY=0

MAXFRAME=3

TXDELAY=250

SLOTTIME=100

PERSIST=50

FULLDUP=0

FRACK=6000

RESPTIME=2000

RETRIES=20

PACLEN=250

CWID=EB7CJO

DIGIFLAG=1

DIGIPORT=0

USERS=0

UNPROTO=EA7RKC

MHEARD=Y

ENDPORT

This file most important parameters are the relative to external driver

control (PORT section) and the virtual COM port number that BPQHTNC2 will

create (TNCPORT section).  You must also have in mind that in normal cases

there will not be a BBS in the other PC, so you will have to put your call

without SSID (-0) in the BBSCALL parameter.  For the BPQ node not to cause

havoc in the local NET/ROM network, it is better to put NODESINTERVAL to 0.

Finally, if you want to use a program which will handle FBB unproto lists

(like TSTHost, AGWWin, etc.) you will have to put the BBS call in both

UNPROTO= (in my case it is EA7RKC), because BPQ's UNPROTO command doesn't

set this parameter (maybe it's a BPQ bug).

Please note that if you plan to use Winpack with TNCEmu, go to section

"5. Problems with Winpack 5.4".

3. Parameters

-------------

In the previous section you found the parameters list which TNCEmu accepts.

In this section you will find a detailed explanation of these, and also the

possible applications in practice.

Format: TNCEMU [COMNum Speed] [/bpq:n] [/kiss] [/int14] [/noflow]

- "COMNum" is the serial port number where it is connected the null modem

cable and, so, tne PC that is going to use the "TNC".  In this version

you can specify values from 1 to 4, which corresponds to COM1-COM4. The

IRQ numbers and I/O port address can't be changed (by now), using the

following:

COM1  COM2  COM3  COM4

----  ----  ----  ----

Port   3f8   2f8   3e8   2e8

IRQ     4     3     4     3

It is not recommended to use serial devices which use the same IRQ.  As

the PC that runs TNCEmu can only handle a Baycom modem and one serial

port, this should not be a problem...

If you use the /int14 option, it is possible to specify a serial port

number greater than 4 (provided that the INT14 driver can handle it).

- "Speed" indicates the baud rate, in bits per second, that TNCEmu will

use to communicate with the other PC through the serial port.  In the

actual version possible speeds are between 1200 and 38400 if you do not

use /int14, and between 1200 and 9600 if you use this option.  The limit

could be increased in the future if there is needings for this.

- "/bpq" is an option which says that a driver like BPQ will be used for

controlling the modem device.  This driver can be handled through a

virtual COM port.  You can set the virtual port number adding ":number"

in the tail of this option and without a leading space.  If it is not

set, it will be used COM5 by default (be warned, it doesn't search for

it!).  If you don't specify /bpq nor /kiss, a TFPCX-like driver will be

used.  The BPQ-like drivers are which creates a virtual COM port to

communicate with them, or also they locates "over" an existent serial

ports (like TNCDED).

- "/kiss" is used for the KISS emulation.  In this case you must have

loaded an "external" BPQ driver, like BPQAX25.COM, and it must be in

interruption line 0x60 (hex, or 96 dec).

WARNING: it is not possible to check for the driver to be present in

memory, so it is very important you check that it is in the correct

interruption line before you use it.  BPQAX25.COM locates itself in

0x60 by default.

BE CAREFUL: until actual version, it is not possible to use BPQHKISS

with the /bpq option, because it can receive but not transmit. (?)

If you don't specify /bpq nor /kiss, it will be used a TFPCX driver

by default.

- "/int14" serves for the use of a serial port driver of COMBIOS or MBBIOS

types (also known as INT14). An example of these drivers is ESS by F6FBB.

I included this option for you to be able to use non-standard serial

ports which needs a special driver.  It can also be used when it is

necessary to use another flow control or "handshaking" method not being

RTS/CTS.  In the case of ESS, it uses DTR/RTS handshaking, which is

compatible with RTS/CTS.

WARNING: if the other PC uses a UART 16550A chip for the serial port,

you can notice problems like characters being missed when you use ESS.

It is because of the handshaking method and the use of a FIFO.

- "/noflow" lets you deactivate the flow control or RTS/CTS handshaking.

In normal cases you should use handshaking, but you must disable it when

the terminal program can't use flow control.  At this moment, XON/XOFF

is not supported.

Programs that must use this option are: WinGT (Host), PR4Win (KISS),

AGW Win (KISS) and in general any program that uses KISS for controlling

the TNC.

4. Program handling

-------------------

In this version, TNCEmu only uses two keys while the program is running:

- TAB: switch echo on screen.

- ESC: exit from program.

Echo consists on a representation of characters that are being sent and

received through TNCEmu.  Below code 32 these are represented with the

format <no.>.  As the text printing consumes a significant CPU time, it is

not recommended to use it on slow computers.

5. Problems with Winpack 5.4

----------------------------

Until version 5.4 of Winpack program, by Roger Barker G4IDE, there is a

compatibility problem with TNCEmu.  Winpack uses the K command instead of

CONV to pass to Converse mode when it is going to send an unproto resyn-

chronization to the BBS.  In TAPR standard the K command is a synonym of

CONV, but BPQ interprets it in another form: as a synonym of NODE, which

connects to the internal BPQ node.

So, by now it is not possible to use the Unproto list with Winpack 5.4

(nor also the 5.43 version), but it can be solved using KISS mode instead

of TAPR.  Winpack 5.4 can control a KISS TNC if you use BPQ running under

Windows.

Fortunately, Roger Barker said to me that he is going to correct this

problem in a next Winpack version.

6. History

----------

1.0 (07-May-96) : First public version.

7. To do

--------

- Support for SMACK (KISS with CRC)... when I understand the german

specs.  Thanks Bernd!  (Bernd OE8DJK, PR4Win's author).

- To be able to choose the port address and IRQ of serial port.

- Handle better the TNCDED driver (one Ctrl-C does not work, you must

send 3 Ctrl-C to exit from the Transparent mode).

- ... and all that you want to tell to me!

8. Greetings

------------

- To Mike Dumdei, for his MCOMM5 library for handling the serial port.

- To the Baycom-team, for his low-cost 1200 baud modem.

- To John Wiseman G8BPQ, for his excellent node software.

- To Jon Welch G7JJF, for his adaptation of the AX25.COM packet driver

from Pawel Jalocha, to the BPQ external driver BPQAX25.COM, simplifying

considerably my work.

- To Rene Stange DG0FT, for the popular driver TFPCX.

- To the Tucson Amateur Packet Radio for his TNC firmware.

- To Ronald E. Raikes WA8DED, to DG3DBI and to the NORD><LINK for the TF

firmware.

- To Roger Barker G4IDE, Winpack's author, for his kindly cooperation for

TNCEmu not to have problems with Winpack.

- To Luis Ramon EA7CVR, for his recopilation of TFPC programs interrupts.

- And to many more that aren't here, but that should be...

9. About the Author

-------------------

The TNCEmu's author am I, Jose Carlos Ramirez Perez, EB-licensed number

F10-N44867-L8 (Spain), operating the EB7CJO station.  I'm student of

third course of Computer Sciences.  I made TNCEmu for saving the money

for a TNC, because I had another computer and knew it can make its work.

Also, I found a big part of the job yet done because of Baycom drivers and

firmware interpreters these includes.  When I started to program TNCEmu I

didn't know that there exists a similar program, PC-TNC, and I got advice

of that when I finished it.  The true is that I had a nice feeling when

I knew TNCEmu has more features than PC-TNC in the 1.10 version.  However

I give my thanks to DG3VT for his work and my congratulations for being

the first person who made a program like that.

TNCEmu is freeware for radio-amateurs only, if there are commercials

that want to use the program, they should contact with me.

Please note that there is a spanish version available, contact with me

or search for "tncemu" with archie in a spanish site.

For any subject about TNCEmu, you can get in contact with me at the

following addresses.  I hope you will not have problems for getting it to

work, but if this is not the case, please ask to me.

73's from Jose Carlos - EB7CJO

Packet: eb7cjo@ea7rkc.eaca.esp.eu

Internet: cramirez@hercules.uca.es

cramirez@arrakis.es      (until 22-Jul-96)

eb7cjo@digigrup.es       (Internet-Packet gateway)

InfoVia:  cramirez@axis.inf        (only Spain and SouthAmerica)

SnailMail: P.O. Box 2684, Postal Code 11080 Cadiz (Spain)