UI-Path32 V1.0 G7OCW (apritch@lineone.net) 1999-2000

Features

- Calculates actual distance of signal travelled from digi to digi to your station. Note this is not the distance as the 'crow flies'

- Save current list to disk will load into excel

- Stations are added to list when heard even if no co-ordinate information is given

- Gives the known shortest digi path to a station

- Realtime operation links to ui-view

- Starts-up and shutdown with ui-view

- Access routes which shows you what stations can contact the destination station

- DX Announcment facility capable of paging you if required

- Monitors and displayes Traffic levels

- Graphically shows the path stations take

- Graphically shows the entire UI-Network mesh

- Compatible with 16 and 32 bit versions of UI-View

Installation

Place all files in your \ui-view directory and restart ui-view

Should you wish ui-path to not start automatically edit uipath.xtr and set the last two lines to false

UIPath.ini

This is a configuation file which you can alter at will

DX=80 (equal or greater DIRECT distance to trigger app below, 0 disables)

DXAPP=explorer c:\ui-view\wav\message.wav (app to run example wav file)

DXDIGI=150 (equal or greater DIGIED distance to trigger app below, 0 disables)

DXDIGIAPP=explorer c:\ui-view\wav\message.wav (app to run example wav file)

This DX function also writes the latest DX to the following files

\ui-view\uipath.dx (direct)

\ui-view\uipath.dxd (digied)

IGNORE_INET_TRAFFIC=TRUE (This causes UI-Path to ignore everything it hears via I-GATES)

AUTO_LOAD_SAVE=TRUE (UI-Path will automatically on exit <not close button> automatically load the contest of uipath.out and the save the complete list to uipath.out)

Using It

Front Screen

This is the place where all the currently known routes are displayed it is layed out in the following manner:

<Callsign><Total Distance Travelled><Shortest Path Taken><Latitude><Longitude><Distance as crow flies><Number of times the station has been heard><Most recently heard status text>

MAIN MENU Functions

<Click on a callsign>

- If you ask to see the map then displays a graphical route to the destination station. Start and finish stations are displayed in red. Digipeaters on route are displayed in yellow.

- Otherwise you see the details for that station in an easier 	to read format including the digi path in reverse which can be cut/paste into ui-view. There is also a <Alias> button which when pressed will ask you for an alias which this station uses for example TELFRD (I.E. THERE sub is set to this) when you then do load alias from the menu it will add this alias to this list and ui-path will then use it in it's calculations

<CLOSE Button> Closes down UI-Path without autosaving to uipath.out

<SAVE Button> Saves all the paths currently know to uipath.out

<ACCESS Button> Take you to the access section

<SORT Button> Sort the list by Total distance travelled

<LOAD Button> Loads from uipath.out all callsign not current in the list

You can find additional functions on the menu:

File:

- Load dB (load uipath.out)

- Save dB (save to uipath.out)

- Load Alias (load the aliases defined by the above <Alias> button

- Exit (Exit uipath, same as close)

Action:

- Access routes (see below)

- Sort List (sorts station list)

- Further Information (see below)

- Test DX App (tests 3rd party application see uipath.ini)

- Test DXDIGI App (tests 3rd party application see uipath.ini)

Options:

- Toggle on and off DX annoucments

About:

- Details about version and author

At the bottom of the screen there is a total number of stations currently being listed by UI-Path.

Traffic monitoring on the main screen

The two colour boxes indicate the traffic level. The one on the left is the traffic for the last 10 minutes and the one on the right is the traffic level for the last minute. Traffic is based on UI frames only so it won't work in BBS frequencies!

The colour of the boxes is indicated in the following way

- Green - Quiet

- Yellow - Moderate

- Red - Busy

This function is further enhanced in the Action -> Further Information section

Access Routes

This function analyses the digi paths to work out what stations can be accessed by which stations

<Click on a callsign> Displays graphically all stations which can be accessed by the specific station

<SAVE> Saves the routes to uipath.acc for later viewing

<MAP ALL> Display graphically all the routes available. This gives you the option of seeing all stations or ust those that are accessed by other people

Further Information

This screen has a lot of information on one screen it is divided into several different areas

<Heard Direct> Simply list all the stations you can hear direct

<DX Broadcasts>

This is a list of all the dx broadcasts which are under 2 hours old

<Graph on the right>This is the current propagation based on the dx broadcasts. On entry it displays the maximum but you can change this using the buttons at the bottom. The graph is time shifted in -hours from your current time so if it's 23:00 currently -1 will show you the propagation at 22:00

The UI Network section tries to summarise this along with a list of all the propagation since UI-Path was started.

<Traffic Level> This shows the current level of traffic based on UI transmissions per minute. The zones are divided by the following

Green zone <= 7 UI frames per minute

Yellow zone <= 14 UI frames per minute

Red zone above the 14 frames per minute

This is summarised in the last 10 minutes box with also graphical representaion in the graph to the left.

+++++++++== Techi Notes ===

*Station      - Callsign of station

*Dist            - Calculated distance travelled via digipeaters. This may be the same as the distance from your station if the digi station has not yet been recorded in the list

*Digi Path - The shortest known path to the particular station. Remeber you need to reverse the path to get back to the station e.g.

g7ocw x*,y*,z* ping g7ocw digi path z,y,x

*Latitude    - Self explanatory

*Lonitude    - Self explanatory

*Home            - Distance in straight line from your station. Should be the same as ui-view!

*X	      - Number of time the station has been heard

*end col      - Status text as broadcast from the specified station

You may notice that the distance values decrease over time. This is because UI-Path is learning of new distances as more stations are added    to it's table also if a shorter path is found it also updates the table. The longer it runs the more intelligent it gets

The map display is an aproximation on the latitude and longitude of a specific station in fact it is just a clever graph!

Both map facilities rely heavily on having the co-ords of other stations to know where to plot the stations. I use the save and load facilities to compile quite a list currently 160+ stations in the file

== Got Problems? ==

P. UI-Path want start up and just gives a GPF

S. Check that grid.vbx is in the \ui-view directory

P. File not found on startup

S. Check that both uiview.ini and uipath.ini are present

P. Distances are incorrect

S. Doh!

P. How do I save the maps UI-Path generates

S. Click on the MAP then do ALT + Print Screen then for example using

paint do edit paste. Then of course save it in paint

File List:

Uipath.exe (the executable)

Uipath.xtr (the extra file requierd for UI-View to start UI-Path automatically)

Uipath.ini (the config file)

Uipath.out (the file containing all known routes)

Uipath.ali (the file containing the alises for stations)

Uipath.err (oh! An error occured. If this file has more than a couple of entries please send to me)

Uipath.dx (The last direct dx)

Uipath.dxd (The last digied dx)

Uipath.acc (The routes as calulated by the access secion)

== Acknowledgment ==

Thanks go to Roger - G4IDE for the help given with the longitude latitude calculation, I have learnt alot! also using DDE example code supplied with winpack. Thanks also to Karl G6ODT for supplying me with the MIC-e decoding routines

73 - Andy, G7OCW