WA7MBL MailBox Sysop Documentation - Preliminary

Version 3.13 - 06/11/87

This document briefly describes version 3.13 of the WA7MBL

Packet Radio BBS.  This is a preliminary manual. A complete

Sysop Manual is under development.

ACKNOWLEDGEMENTS

Wes, K7PYK, has made major contributions of equipment, time and

money.  By handling distribution, he has given me extra time to

work on updates.  Roy, AA4RE, furnished the original COMBIOS.

Norm, W2JUP, has contributed in many ways.  Bob, WA7MXZ, has

helped with local testing and had provided BITNET/ARPANET access

for communications with others.  AEA has kindly furnished TNCs

for testing.  Many others have helped in a number of ways,

including: K0GUZ, W9ZRX, AD8I, W3IWI, W3VS, TAPR, NEPRA, WB7BNI,

KA2BQE, WA4ZLW, and N0AN.  Thanks to many members of the 14,109

HF BBS net for handling messages.  A large number of others have

made suggestions or helped with funding.  Although this BBS code

is no longer a clone of the original W0RLI BBS, much of it is

based on his original efforts.

REQUIREMENTS

TNC-2 or equivalent is REQUIRED.

This release requires a TNC which will go into command mode

upon receipt of a "break".  The TNC must support the

"NOMODE ON" command, and must indicate connect status on

pin 8.  I have had success with all true TNC-2 "clones".

AEAs PK-87 and PK-232 also work if they have the current

firmware.  I have not tried Kantronics TNCs and have had

mixed reports of success.  At this time, TNC-1s will not

work.

IBM PC or equivalent.

This software has been running on IBM PCs and many clone

systems.  I am not aware of any problems running on a true

compatible.  This software writes directly to the screen,

and may not work on some MS-DOS computers that are not IBM

compatible, such as the SANYO 555.

A HARD DISK is strongly recommended.

WA7MBL Sysop Documentation                                     1

(C) 1986 Jeff Jacobsen

CHANGES from version 2.0x

1 -  Resident I/O drivers.

The I/O drivers have been separated from the BBS code and are

loaded into memory before starting the BBS.  This allows the

development of drivers for non-standard I/O addresses, and for

the use of multiple I/O ports.  The BBS code is current written

to support up to 6 ports.

2 -  Resident Timer.

The timeout timer has also been separated from the BBS code.

One benefit from this change is that if the BBS should "crash",

there are no interrupts (I/O or timer) left pointing into the

area of memory where the BBS was loaded.

3 -  BTRIEVE File Manager.

A separate File Manager is also used.  Btrieve is copyrighted

by SoftCraft Inc., and furnished under a license agreement with

them.

4 -  Direct Screen I/O.

The current code writes directly to the screen.  It checks for

the presence of DoubleDOS or DesqView and reportedly will run

under the current version of either.  It is not TopView

compatible at this time.

5 -  Mail "Beacons"

The mail beacons are no longer sent using the TNC BT command,

but are sent as Unproto text by the BBS, thus each port can have

different text and can be any length.

WA7MBL Sysop Documentation                                     2

(C) 1986 Jeff Jacobsen

GETTING STARTED

The file CHANGES.313 gives information on how to update your

files if you are currently running version 3.12.

If you are setting up the BBS for the first time, there are a

number of files that must be created.  There are 3 database

files created under BTRIEVE that contain message headers, the

user list, and a list of other BBSs that handle bulletins.

To create these new blank files, do the following:

BTRIEVE /C

BBSMKFIL        [create all 3 files]

You also need to edit the sample BBS313.CNF file that has been

included, and will need to set up a forwarding file.  If you can

find a local BBS already running this code, you will find it a

lot faster to get started if you can obtain help from a local

Sysop.

You should also edit some of the help files (such as GHELP.HLP

and INFO.HLP) to describe your particular situation.

To keep the mail, old mail, help files, upload/download text and

binary files organized, I recommend that they be placed in

separate subdirectories.  The path names for these are specified

in the BBS313.CNF file.  The sub-directories must exist, BBS

will not create them.

WA7MBL Sysop Documentation                                     3

(C) 1986 Jeff Jacobsen

GETTING STARTED

I use the following setup (with the main program in a

subdirectory called \BBS) on my hard disk:

\BBS\

BBS.COM

BBS.000

BBS.001

BBS313.CNF

CALLS.BBS

FWD.BBS

JLIST.BBS

LOG.BBS

RVSFWD.BBS

BBSUSER.BTR

BBSMAIL.BTR

BBSBULL.BTR

YAPPB.COM

YAPPB.CNF

\BBS\HELP\

B.HLP

D.HLP

H.HLP

(etc)

HELP.HLP

GHELP.HLP

LHELP.HLP

INFO.HLP

\BBS\MAIL\

MSG00111.MAI

(etc)

\BBS\OLDMAIL\

MSG00001.MAI

(etc)

\BBS\FILES\

BBS.DOC

TNC2.MOD

(other user uploaded/downloadable files)

\BBS\YFILES\

CRC.COM

(other user uploaded/downloadable binary files)

If you want the users to be able to download LOG.BBS, FWD.BBS,

etc, place them in the FILES\ subdirectory and set the file

names in BBS313.CNF accordingly.

WA7MBL Sysop Documentation                                     4

(C) 1986 Jeff Jacobsen

GETTING STARTED

STARTING THE SYSTEM

To start the system, you must load the appropriate I/O drivers,

load MBTIMER, load Btrieve, then load the BBS program.

The I/O drivers supplied are:

COMBIOS  (supports COM1 and COM2)

COM1BIOS (COM1 only)

COM2BIOS (COM2 only)

QUADBIOS (COM3 - COM7 on Quadram Quadport AT board)

An alternate I/O driver called MBBIOS is a multi-purpose driver

under development by AA4RE.  I do not have a copy and have not

tested it.  When complete, it should be able to replace all of

the above drivers, and will support other multi-port I/O cards.

The following series of commands will start the BBS:

COMBIOS       (or alternate)

MBTIMER

BTRIEVE /C

BBS

As with version 2.04, you may request that the BBS go directly

to the waiting for connect mode by specifying /A on the BBS

command line.  An AUTOEXEC.BAT file containing the following

will start the BBS and place it on line upon power up or after a

power failure.

CD \BBS

COMBIOS

MBTIMER

BTRIEVE /C

BBS /A

Btrieve has an option to set the amount of memory in K for cache

buffers.  Valid sizes are from 9K to 64K.  To allocate

additional memory to Btrieve use the /M:xx option:

BTRIEVE /C/M:64

WA7MBL Sysop Documentation                                     5

(C) 1986 Jeff Jacobsen

TNC SETTINGS

I have tested the BBS code with v 1.1.3 and 1.1.4 of the TNC-2

code, and with AEA's PK-232 and PK-87.  The major settings are

as follows.  You will want to set DWAIT and other timing

parameters as appropriate for BBS operation in your area.

TNC-2                    PK-232/PK-87

AUTOLF OFF               ALFDISP OFF

AWLEN 8                  AWLEN 8

DELETE OFF               DELETE OFF

ECHO OFF                 ECHO OFF

FLOW OFF                 FLOW OFF

PARITY 0                 PARITY 0

SCREENL 0                ACRDISP 0

START 0                  START 0

STOP 0                   STOP 0

STREAMSW 0               CHSWITCH 0

TXFLOW OFF               TXFLOW OFF

XFLOW OFF                XFLOW OFF

XOFF 0                   XOFF 0

XON 0                    XON 0

RELINK OFF

DCDCONN ON    (PK-232 only)

Please note that the AWLEN and PARITY parameter changes are not

made effective until the TNC is turned off then back on, or

until the RESTART command is given.  It is often wise to give

the RESTART command to the TNC before starting BBS operation to

insure the TNC is in a known state.

Hardware handshaking is used.  Also, the BBS code determines

when a station has connected by monitoring pin 8 of the TNC,

rather than scanning for a "*** CONNECTED" message.  Once a

station has connected the BBS then turns CONOK OFF and MON OFF

on any other ports and sends a "C" command to the connected port

to determine the callsign of the connecting station.  [Note:

3.13 does recognize the "*** LINKED to" method of indicating a

gatewayed connection from another type BBS.  However, if this

string arrives too early (while the BBS is doing the above

tasks) it may be missed.  The BBS is not expecting its first

command until it sends a menu prompt.]

If your TNC holds pin 8 high, the BBS will think someone has

connected, start the cycle, determine that no one is connected,

and continue to loop.

The cable between your TNC and serial port must connect, at

minimum, pins 1-8 and pin 20 on your TNC to your serial port.

WA7MBL Sysop Documentation                                     6

(C) 1986 Jeff Jacobsen

COMMANDS

User Command Summary

A - ABORT a listing

This will abort while reading a message or downloading a

file.  The TNC may have buffered a large part of the

message or file, so this command may not be received in

time.  It is recommended that before downloading a file the

User use the download option to read the first few lines

before requesting a large file.

B - BYE

Logs off and disconnects.  The User may simply disconnect

rather than use the B command.

D - DOWNLOAD a file

A number of options have been added to this command so the

user may take a look at a file without downloading the

entire file.  Examples:

D JUNK.TXT        10        [show first 10 lines]

D TECH/TNC1.MOD  -20        [skip first 20 lines]

D TNC2.SET       5 15       [show lines 5 to 15]

H - HELP

Sends the User Help File (normally HELP.HLP)

I - INFORMATION about this BBS

Sends the Information File (normally INFO.HLP)

J - List of calls heard by this BBS

JK - List most recent Connected stations

JA - List calls monitored on TNC-A

JB - monitored on TNC-B

JC - monitored on TNC-C

JD - monitored on TNC-D

JE - monitored on TNC-E

JF - monitored on TNC-F

K - KILL a message

K x     - Kill message # x

K x y z - Kill messages # x y z (up to 6)

KM      - Kill Mine (addressed to User that have been read)

KT x    - Kill message # x (type S or T message)

Users can only kill mail addressed To or From them.  (Any

User can kill a type S or T (traffic) message with the KT

command.  KT can only kill one message at a time.)

WA7MBL Sysop Documentation                                     7

(C) 1986 Jeff Jacobsen

COMMANDS

L - LIST message headers

L - New messages since last using the L command

LM - Only messages to you (List Mine)

LN - Only unread messages to you (List New)

LT - List NTS Traffic (NTS Traffic Handlers)

LL 10 - List last 10 messages

L< callsign - Messages from 'callsign'

L> callsign - Messages to 'callsign'

L@ callsign - Messages sent care of 'callsign'

L x - Messages above a given number x

L x y - Group of messages from x to y

Personal messages (type P) are only shown to the sender,

addressee, or Sysop.  Hidden messages (stat H) are only

shown to the Sysop.

N - NAME

N name  - Enter Users first name into the User file

R - READ messages without forwarding headers

R x     - Read message # x

R x y z - Read messages # x y z (up to 6)

RM      - Read Mine (addressed to User)

RN      - Read New (addressed to User, status = N)

Users may read all messages except those marked as private

(type = P) or those that are Hidden (status = H).

S - SEND messages

S callsign - Send message to 'callsign'

SP callsign - Send private message to 'callsign'

S callsign @ pbbs - Send message to a station

at another PBBS

ST NTSxx - Send NTS traffic for 'xx' state

SB ALL - Send a general Bulletin addressed to ALL

T - TALK to the SysOp

If the System Operator is available the User may request to

chat directly with the SysOp

U - UPLOAD a file TO the BBS

V - VERSION

The V command with no parameters shows the BBS version #,

release date, number of active messages, and next message

number

WA7MBL Sysop Documentation                                     8

(C) 1986 Jeff Jacobsen

COMMANDS

V - VERBOSE READ (Same as R but includes forwarding headers)

V x     - Read message # x

V x y z - Read messages # x y z (up to 6)

VM      - Read Mine (addressed to user)

VN      - Read New (addressed to user, status = N)

W - WHAT's available

W    - List files available for downloading

WN   - List new files only

X - EXPERT Status

The X command changes User to Expert or Non-Expert in the

user file.  BBS stations or Remote Sysop stations will

always get the short menu anyway, so changing Expert mode

will make no difference for them.

Y - YAPP Binary File Transfer

Y or ?Y or YH - Print YAPP help file

YW - List Binary file directory (YAPP What)

YN - List New Binary files (YAPP New)

YU - YAPP Upload

YD - YAPP Download

YAPP Upload and Download commands require that the User

have a copy of a program supporting the YAPP binary

transfer protocol.

YAPP is a trademark of Frontier Computing Inc.

? - Help commands

?A - Give help with A command

?B - Give help with B command

etc

?A through ?Z sends the user help files A.HLP - Z.HLP.

(These reside in the Help subdirectory specified in

BBS313.CNF).

? alone sends H.HLP

WA7MBL Sysop Documentation                                     9

(C) 1986 Jeff Jacobsen

COMMANDS

Remote Sysop & Local Commands

A Remote Sysop has access to the User commands.  In addition,

he may use the following commands in Remote Sysop mode (entered

by using the @ command from the standard User menu).

The Local Sysop may also use these commands from the console.

DU - Display USERS in User file

DB - Display BBS stations in User file

DS - Display remote SYSOPS in User file

DX - Display eXpert users in User file

DE - Display EXCLUDED users in User file

DP - Display File in paged mode

D$ - Display BBS File (BBSBULL.BTR)

E  - Edit Message Header

EU - Edit User File

E$ - Edit BBS File

YZ - Kill Yapp file

Z  - Kill File

LH - List Hidden Messages

HH - Release all Hidden

H x - Release hidden # x

H x y z - Release hidden # x y & z  (up to 6 per command)

Remote sysops can edit user file, but cannot change Sysop status

of a user (can change BBS, Expert or Excluded stuff).

Remote sysops can only Access files in the standard file areas.

WA7MBL Sysop Documentation                                    10

(C) 1986 Jeff Jacobsen

LOCAL & REMOTE SYSOP COMMANDS

Commands only Available from Local Menu

The Local Sysop has access to most of the User commands, except

those that are not appropriate for local use:

A  -  Abort

B  -  Bye

YU -  YAPP Upload

YD -  YAPP Download

LOCAL SYSOP COMMANDS:

A  - Place BBS on-line awaiting connects

B  - Do forwarding (same as X command) then go on-line

O  - Gateway to Operating System (DOS)

OA to OF - call YAPPB and pass the port number.

GR - Change reverse fwd filename

GF - Change forward filename

H  -  List LHELP.HLP (local 'menu')

WO - Show directory of BBS\OLDMAIL\*.*

RO - Read old message:  [also may use DO]

RO 113            [Display BBS\OLDMAIL\MSG00113.MAI]

RO 113   5        [Show first 5 lines]

RO 113  10 20     [Show lines 10-20]

RO 113  -15       [Skip first 15 lines]

KO - Kill old message:

KO 113            [Delete BBS\OLDMAIL\MSG00113.MAI]

KO 113 172        [may delete up to 6 per line]

$  - Display bulletin header.  (This will display a list of all

BBS stations that have received the bulletin.)

$  147            [Display bulletin header for Msg # 147]

Alt-F10 will dump user immediately in most cases.

Alt-F10 will also abort during forwarding.

F6 will request chat with user.

F1 will toggle Sysop Available status.

Status line shows # Active messages, Last message #, fwd minute.

Sysop AVAIL for chat or MAIL waiting will appear on status line.

WA7MBL Sysop Documentation                                    11

(C) 1986 Jeff Jacobsen

LOCAL & REMOTE SYSOP COMMANDS

ACCESSING FILES from Local menu:

The W, U, and D command by default go into the FILES\

subdirectory IF you use a standard type name  (TNC2.UPD,

ARRL\BUL304.TXT, etc.)

You CAN access any other area if you either

1) Precede it by the full path name starting with a \

(ie, \BBS\FWD.BBS)

2) Use a Drive identifier (A:BORK.TXT)

So:

W    will do a directory of \BBS\FILES\

W \  will show your main directory

W A: will do a directory of drive A

WA7MBL Sysop Documentation                                    12

(C) 1986 Jeff Jacobsen

GUEST USERS

GUEST Users on BBS-only ports and HIDDEN messages:

About the Guest feature...  A non-BBS station may connect to a

BBS only port. If he does so, he will get the guest hello which

tells him his options.  A guest is limited to only a few

commands:

B - Bye

H - Help (Can only be done once, second time will log him off)

S - Send message

T - Talk to SYSOP

RM - Read Mine

RN - Read my New

KM - Kill Mine

V - Print version

V - Verbose Read

Any messages that are left by a guest with the S command are

flagged with a status of "H" (Hidden).  Only the Sysop or remote

sysops can look at Hidden messages.  The LH command may be used

to look at hidden messages.  They are made visible by setting

them to a status of "N" with the H command.  HH will make all

hidden messages visible.  H 14 will make message #14 visible,

etc.  If the hidden messages are for the sysop, doing an RM will

change the status to "Y" as part of reading them.  If there are

hidden messages in the system, the prompt "HIDDEN" will appear

on the console status line.

This feature will allow non-BBS stations to access the system

for the limited purpose of reading or leaving mail and to

request normal access.  It also will allow another station not

set as a BBS to forward mail, but will hold the message until

the sysop releases it.  This may help solve the problem of

world-wide 3rd party traffic on HF.  If non-BBS stations connect

to leave 3rd party mail, the sysop will have the opportunity to

review it before it can be read or forwarded to anyone else.

WA7MBL Sysop Documentation                                    13

(C) 1986 Jeff Jacobsen

MAIL, BULLETINS & REQUESTS

The Mail headers have been changed to allow a text file to be

appended to the normal message text.  At present, this is only

available from Local mode.  When in local mode, the S command

will prompt for an append file.  If you don't want one, just hit

return when prompted.

If you enter an invalid file name or the name of a non-existing

file, you will receive a warning, but the name will be saved.

You may either edit the mail header with the E command or

create/rename the file you wanted by exiting to DOS via the O

command.  To edit out an AFILE name, enter one space.

If the append text file is deleted or missing, only the regular

message header & text (if any) will be sent.  When a message

with an append file is deleted, the append file will remain.

This feature is nice if you want to send a message to more than

one person without making multiple copies of the text:

Local> sp k7pyk

Enter Subject for Msg # 337:

3.13 mods

Enter Text, ^Z (CTL-Z) or /EX to end:

/EX

Enter Append filename:

BBS313.CHG

Local> sp w3iwi

Enter Subject for Msg # 338:

3.13 mods

Enter Text, ^Z (CTL-Z) or /EX to end:

/EX

Enter Append filename:

BBS313.CHG

It text is entered, the text will be followed immediately by the

appended file.

Note that the Append file name appears under the message line

when doing an "L" list.  This only appears on the Local screen

and does not appear if done by a user.

Local> l 337 338

Msg# TS  Size TO     @ BBS  From   Date    Subject

338 PN  1340 W3IWI         WA7MBL 03-Jan  3.13 mods

AFILE: BBS313.CHG

337 PN  1340 K7PYK         WA7MBL 03-Jan  3.13 mods

AFILE: BBS313.CHG

WA7MBL Sysop Documentation                                    14

(C) 1986 Jeff Jacobsen

MAIL, BULLETINS & REQUESTS

"FLOOD" BULLETINS

I decided there was a real need for a better way to distribute

bulletins to a large number of BBS stations.  I felt that the

"F" type message was not the best way to implement bulletins.

The most effective way I could think of was to "flood" the

system.  Rather than passing a bulletin to one BBS, I would pass

it to all BBSs that I can reasonably distribute it to.  They in

turn pass it to all that they talk to.  The trick, how to keep

everyone from receiving multiple copies...

1)  Every bulletin must have a unique identifier (The BID or

bulletin identifier.  I have allowed 12 spaces.  I would

recommend that individual stations use their callsign then a

sequential number to identify any bulletins originating from

their station.  Also, it would be nice if someone would propose

a scheme for bulletins from "Well known sources" (ARRL, AMSAT,

TAPR, etc) to keep them identified the same.  If a number of

stations start sending ARRL bulletins and identify them with

their own callsigns we still haven't eliminated the duplicate

message problem.  Note that the BID is forced to upper case.

2)  Next, we need a way to identify which BBSs to forward which

bulletins to.  I propose using the @BBS field to describe the

appropriate group.  Blank for a general message, perhaps HF for

all members of an HF group, etc.  The contents of the TO field

would be ignored for forwarding purposes.  To stay compatible

with the RLI world, any message type could be used.  The status

would be automatically set to "$".  They would never be marked

as read or forwarded.

To mark which BBSs get which group of bulletins, we would add

the group identifier in with the list of calls to be forwarded

to in the current forwarding file.

To identify the bulletin groups from a regular user, they would

be preceded with a "$".  A single "$" would mean to send all

bulletins with no callsign (group identifier) in the @BBS field.

WA7MBL Sysop Documentation                                    15

(C) 1986 Jeff Jacobsen

MAIL, BULLETINS & REQUESTS

A forwarding file with Bulletin groups may look like the

following:

GA K7PYK

00-23

@K7PYK

K7PYK

WB7BNI

etc

$       [all blank @BBS bulletins]

$HF     [all @HF]

$AZ     [all @AZ  (Arizona)]

etc

*** EOF

GA W3IWI

00-23

@W3IWI

W3IWI

$

$HF

*** EOF

So, a message looking like this:

Local> ll 2

Msg# TS  Size TO     @ BBS  From   Date    Subject

343 P$   804 SYSOPS@HF     WA7MBL 03-Jan  New L.HLP File

BID: WA7MBL00343   AFILE: \BBS\HELP\L.HLP

would be forwarded to W3IWI and K7PYK.

If K7PYK received the message first and had W3IWI in his

forwarding file as $HF, he too would be trying to forward the

same message to W3IWI...

Which brings us to

3)  If I am forwarding the same message to more than one

station, I can't just mark it with a status of F when I have

forwarded it.  So... this is where the BBSBULL file comes in.

In it I can include up to 99 BBSs that I forward bulletins to.

(These are stations that I forward directly to remember.  If

this number needs to be increased, it could be done.  But

remember the idea is to send it to the stations we forward to

easiest and let them move it on.  Sorta "chain letter" style.)

Anyway, in the BBSBULL file, each BBS may be given a unique BBS

# from 1 to 99.  (The file may have other uses eventually, so it

is not mandatory to assign a number to everyone in it.  Only

those stations you wish to forward bulletins to need a number.)

WA7MBL Sysop Documentation                                    16

(C) 1986 Jeff Jacobsen

MAIL, BULLETINS & REQUESTS

Stations are added to the BBSBULL file with the E$ command and

may be displayed with the D$ command.

Local> d$

Call     Last-Heard   No

K7PYK    861230/1735    3

W1AW     800000/0000

W3IWI    800000/0000    4

WA7MXZ   861230/1241    1

WB7BNI   861223/0342

The Last-Heard time is included for possible future use.  The

idea would be to skip forwarding to a station that has not been

heard within the last x hours.  I notice a lot of connect

requests at times when the other station is off or at times when

the path is terrible.  Even if an occasional opportunity was

missed, keeping the channel cleaner may allow others to get more

real forwarding done.  Also, with reverse forwarding, when a

good connection is made, much more can be accomplished.

The mail header record has been extended to include an

additional 99 bits (rounded up to 13 bytes).  (Aha, I see a

gleam in your eyes.)  Once a bulletin has been sent to one of

the stations in the BBSBULL file, the matching bit is set in the

mail header.  If a station is set to receive bulletins in the

forward file, but doesn't exist or have a number assigned in the

BBSBULL file, that station doesn't get the bulletin.  Or, if the

station's bit is set in the mail header, he has already been

sent the bulletin and also doesn't get it.

4)  The biggest trick of all.  If I had my way, I'd like to

separate all bulletins from private mail.  However, we are sorta

stuck with remaining compatible with the existing BBS network.

We've solved the problem of not sending the bulletin to the same

station twice, but how do we keep someone else from sending it

to someone we have sent it to, etc, etc.

I've modified my code to recognize another BBS running 3.12 code

and have solved the problem.  For now, the forwarding files will

have to be set so no two stations forward bulletins to a non-

compatible system.

How the new code works:

When someone connects to the BBS, the first thing it will send

is [BBS313] followed by a carriage return.  Another BBS running

3.12 or 3.13 planning to forward will also send [BBS31x] as a

command when it sees the first prompt> from the receiving BBS.

WA7MBL Sysop Documentation                                    17

(C) 1986 Jeff Jacobsen

MAIL, BULLETINS & REQUESTS

Once the systems know they are talking to each other, forwarding

proceeds as follows:

The receiving BBS will switch to a very short prompt  ">".

The sending side will send something like:

S   W1XXX  @  K3YYY  <  K7ZZZ      [a normal type message] or

SA  ALL    <  W1AW   $ARRLDX00123  [a bulletin]

The receiving side will decide if it wants the message.

In the first case, if it dunno how to reach K3YYY, it could

refuse the message (although at present it wont):

N - K3YYY unknown

In the second case, if it has already received the bulletin

from someone else, it will refuse it by sending:

N - Already have ARRLDX00123

If the receiving side refuses a message, the sending side will

wait for the next prompt ">" and go on to the next message.

In the case of a refused bulletin, the sender marks the mail

header to indicate that the other side already has it and wont

try to forward it again.

If the receiving side doesn't have the bulletin, or if it is a

normal message, it responds with:

OK - Snd # 1234

At which time the sender sends the subject line, followed

immediately by the text.  No additional prompts are sent by the

receiving end until the next ">" after the full message has been

received.  If it was a bulletin, the receiver marks the sender

as having the bulletin (if the sender is in the receiver's BULL

file).  Thus preventing the receiver from trying to forward it

back later.

This repeats until the sender has no more message to forward.

At this time the sender will send the command "F>" to request

reverse forwarding.  Now if the other BBS has mail for the

original sender, it will send the:

S W1YYY etc

and wait for an N or OK and mail will flow the opposite

direction.

WA7MBL Sysop Documentation                                    18

(C) 1986 Jeff Jacobsen

MAIL, BULLETINS & REQUESTS

When the other side has no more mail, it will send "*** done"

(or anything not starting with an "S".  The original sender will

then disconnect and go away.

When forwarding to a station not identified as a compatible BBS,

the sending side will send bulletins as any other mail without

the $BULLID field, and will do the usual:

S TO @ BBS < FROM

Header

[eat two lines]

Text

^Z

At present, the information following the OK or N from the

receiving BBS is not critical.  If other systems would adopt a

similar protocol, it would be possible to reject messages that

we don't know how to forward, or even to accept them but

indicate that more info may be needed.

OK - Route unknown, will dump it on W3IWI

5)  One final unimplemented detail.  I should add a "how old"

line in the config file and not try to move any bulletins older

than x days.

Entering Bulletins:

Bulletins are entered with the regular S command.  Any message

with a bulletin identifier will have the status set to "$".

S command lines are now parsed better and can be entered in a

more free field format.

SP  SYSOPS@MAFIA <W3IWI $W3IWI00123

or

SP  SYSOPS  @   MAFIA   <   W3IWI   $  W3IWI00123

both will be parsed the same, in fact:

SP SYSOPS@MAFIA<W3IWI$W3IWI00123

will also work (but I wouldn't encourage it!)

WA7MBL Sysop Documentation                                    19

(C) 1986 Jeff Jacobsen

MAIL, BULLETINS & REQUESTS

Some example bulletins as entered from the Local console, both

with append text files:

Local> s all@UT <PIGOUT $PIGOUT014

Enter Subject for Msg # 342:

N/L #14 (Jan 87)

Enter Text, ^Z to end:

/EX

Enter Append filename:

PIGOUT\NL014.TXT

Warning: Cant find file: FILES\PIGOUT\NL014.TXT

Local> s sysops @ HF $WA7MBL00343

Enter Subject for Msg # 343:

New L.HLP File

Enter Text, ^Z to end:

Here is the newest version of L.HLP:

/EX

Enter Append filename:

\bbs\help\l.hlp

Local> ll 2

Msg# TS  Size TO     @ BBS  From   Date    Subject

343  $   804 SYSOPS@HF     WA7MBL 03-Jan  New L.HLP File

BID: WA7MBL00343   AFILE: \BBS\HELP\L.HLP

342  $     0 ALL   @UT     PIGOUT 03-Jan  N/L #14 (Jan 87)

BID: PIGOUT014     AFILE: PIGOUT\NL014.TXT

You will note the last message needs a file copied to the

appropriate place, or the append filename edited by using the

E 342 command.

WA7MBL Sysop Documentation                                    20

(C) 1986 Jeff Jacobsen

MAIL, BULLETINS & REQUESTS

REQFIL & REQDIR - File and Directory Requests.

File Requests and Directory Requests are supported similar to

the manner used in the KA2BQE BBS code. To request a directory,

the requesting stations sends the following message:

S REQDIR @ WA7MBL < W1XXX

Subject:

TECH @ K9ZZZ

Text:

^Z  (any text will be ignored)

For directory requests, the subject may be left blank to request

a list of all files in the FILES directory, or a subdirectory

name may be entered for a list of files in FILES\subdirectory.

So that the BBS knows where to address the reply, an optional

@BBS may be added after the subdirectory name (if any).  If only

the @BBB is in the Subject field, it will do a directory of the

main FILES directory.

Some example subject fields:

@W1AW                [directory of FILES\*.*]

*.DOC                [directory of FILES\*.DOC]

ARRL/NTS*.* @ AA7A   [directory of FILES\ARRL\NTS*.*]

Immediately upon receipt, a reply message is generated.  In the

first case, the reply would be a message addressed to:

SP W1XXX @ K9ZZZ < WA7MBL

Subject:

Req Dir: TECH

Text:

TNC2.TIPS     1439    etc...

If nothing is found, the text will read:

REQFIL: No files found.

Message type is ignored on incoming requests.  All outgoing

messages are sent as type P.

The incoming message is deleted upon receipt as the reply

message is generated.  It is saved in OLDMAIL if you have the

flag set to save killed messages.

WA7MBL Sysop Documentation                                    21

(C) 1986 Jeff Jacobsen

REMOTE FILE & DIRECTORY REQUESTS

File requests are handled in the same manner, with the request

sent to:

S REQFIL @ WA7MBL < W7AA

Subject:

TECH\TNC2.TIPS @ K7PYK

In this case, the reply is addressed as before:

W7AA @ K7PYK < WA7MBL

If the file is not found, the subject will be:

No File: TECH\TNC2.TIPS

and there will be no text.

If the file is found, the subject will be:

Req File: TECH\TNC2.TIPS

and the Text will be the file.  (Actually there will be no text

in the form of MSGxxxxx.MAI, but the Append file name will be

set to the requested file.)  So if the file is removed before

the message is forwarded, sorry Charlie.

One other thing to watch for, which I am not going to fix at

this time.  A legal filename may contain an "@", but I do a

quick and dirty parse of the subject to grab any @BBS.  Thus, if

someone requests FIL@NAM.TXT, the program will try to send the

file via @ NAM.TX

The reply message generated from a REQDIR or REQFIL will be

logged into the log file.

Because the message is immediately created: if the request is

received from a station supporting reverse forwarding, and if

the reverse forwarding file is set so the reply will be

forwarded to the station that just posted the request, the reply

will be available to be reverse forwarded off the system.

Automatic transfer of binary files is not implemented (yet!).

WA7MBL Sysop Documentation                                    22

(C) 1986 Jeff Jacobsen

REMOTE FILE & DIRECTORY REQUESTS

SERVER - A general purpose request server.

The server works similar to the REQDIR and REQFIL commands,

however it is much more flexible and can be used for many things

including database lookup, gatewaying, etc.  We are currently

using it at WA7MXZ-2 to transfer mail from the BBS to DECNET,

BITNET and ARPANET.

1 - Upon receipt a message addressed to SERVER @ WA7MBL, the

BBS will append the message to a file specified in the

configuration file in the following format:

K7PYK;P;234;870515/1400    (From Call;Msg Type;Msg #;Date/Time)

Subject Line Here

Text ...

/EX

2 - A user program can be called from within the forwarding

file using the D command.  This program can read the server

input and based upon the subject or text can provide any needed

response by writing it to a standard text file.  The format is

almost the same as entering a normal message, except that an

"Append" file can be specified as part of the "S" line:

SP K7PYK @ WB7BNI < WA7MBL $ ARRL0001 + APPFIL.DOC

Subject

Text ...

/EX

This file can contain any number of messages.

3 - The forwarding file now implements a "T" command which

reads a text file in the above format and loads the messages

into the BBS.

The forwarding file to do the above may look somewhat like the

following:

D DBSERVE.COM           [run DOS command]

00-23                   [all hours]

-----

T DBOUT.TXT             [convert any text to mail]

00-23                   [all hours]

-----

Currently, the DOS programs are only able to run once per hour

from the forwarding file (unless started manually), so response

is not immediate upon receipt of the mail to SERVER.

WA7MBL Sysop Documentation                                    23

(C) 1986 Jeff Jacobsen

FILE LAYOUTS

The Configuration File - BBSxxx.CNF (xxx = version #)

The file BBSxxx.CNF is a text file that contains customized

parameters.  Edit it to have the proper information for your

site.

When you receive a new copy of the BBS code, you will want to

check to be sure lines have not been added or deleted to the

configuration file.  Strange things may happen if you have one

line too few or too many.

You may notice lines containing "$X" in the configuration file.

The form $X is a variable text field. The "$X" is replaced by

the current value for that text.  (NOTE: lower case IS

significant!)

New in v3.12 ($a,$o,$p,$r,$s,$u).  These match the upper case

equivalent.  The upper case version always takes 6 characters

space filled at the right.  The lower case is the length of the

callsign associated with it.

$A - @ BBS of the current message.

$B - Bell - insert an ASCII BEL (CTL-G).

$C - Next available message number.

$D - The current date (861111)

$d - Current date in new format (11-Nov)

$E - Version number.

$F - Forward minute.

$G - TO of the current message.

$I - Users name, from user file.

$J - Date from current msg header (861111)

$j - Mail date in new format (11-Nov)

$K - Time from current msg header.

$L - Number of the last message used.

$M - Message number from current msg header.

$N - Number of active messages.

$O - Call of system owner.

$P - FROM from current msg header.

$Q - list of unread mail calls

$R - Call in a "connect request:" from TNC.

$S - Call of the station connected via gateway. [reserved]

$T - The current time.

$U - Call of the current User.

$V - Owners name.

$W - Wait to send - adds a ^V before CR so packet not sent.

Use at end of line only.

$X - Date user last logged in.

$x - Last date user logged on in new format

$Y - Time user last logged in.

$Z - User "L-msg" number.

WA7MBL Sysop Documentation                                    24

(C) 1986 Jeff Jacobsen

FILE LAYOUTS

The Configuration File - Sample

BBS313.CNF

CACHE ARC BBS - Logan, UT

*** EOF   (end of message of the day - Shown to ALL users)

[The message of the day is sent to all users upon connect,

including BBS stations.  You may want to leave it off.]

Hello $I, new $Z - $L, active $N.

*** EOF   (end of regular user hello)

[The regular user hello is sent to normal users, those who

are not Sysop, BBS, or Expert Users.]

$N active messages - Last message is # $L.

Please use N command to enter your name.

Type ? <return> for help.

*** EOF   (end of New user hello)

[The newuser hello is sent to new users.  A new user is

defined as a normal user who has not entered his name.]

You are not in the user list for this frequency.

You may only leave a message, talk to $o, or log off.

*** EOF   (end of non-BBS Guest user hello)

[The guest user hello is sent to a non-BBS user connecting

on a BBS-only port if guest mode is allowed.]

BBS connects only on this frequency - Sorry!

*** EOF   (end of non-BBS hello/goodbye on BBS-only port)

[The non-BBS hello/goodbye is sent to a non-BBS user

connecting on a BBS-only port if guest mode is not allowed.

The user is then disconnected.]

$o BBS (B,D,H,I,J,K,L,N,R,S,T,U,V,W,X,?) >

*** EOF   (end of "long" menu for normal users)

[This menu is sent to normal users.]

$o BBS>

*** EOF   (end of short menu sent to BBS/Expert/Sysop stations)

[This menu is sent to BBS/Experts/Sysops.]

WA7MBL Sysop Documentation                                    25

(C) 1986 Jeff Jacobsen

FILE LAYOUTS

$o BBS (B,H,S,T) >

*** EOF   (end of menu for guest users)

[This is the menu for guest users.]

Next?

*** EOF   (remote sysop menu)

[This is the menu for remote sysops.  It lets the remote

sysop know that he is in remote sysop mode.]

WA7MBL        (Owner call sign)

Jeff          (Owner name)

[The owner callsign goes into the $O & $o variables and the

owner name into the $V variable.]

CALLS.BBS     (List of monitored callsigns)

FWD.BBS       (forwarding file)

RVSFWD.BBS    (reverse fwding file - may be same as fwding file)

SERVER.TXT    (file of Mail addressed to SERVER)

LOG.BBS       (file for logging)

JLIST.BBS     (Save file for J lists)

[These are files used by the BBS.  If you are running a

floppy based system, you may want to place them on another

drive.]

FILES\        (path/drive for upload/download files)

YFILES\       (path for YAPP files )

MAIL\         (path for mail )

OLDMAIL\      (path for saved mail )

HELP\         (path for single character help files )

[These are the paths where certain files are stored.  They

may also include a drive identifier.  The paths must exist

when starting the system.  The BBS does not create the

subdirectories for you.]

BBSMAIL.BTR   (mail header file)

BBSUSER.BTR   (user data file)

BBSBULL.BTR   (bbs data file)

YES           (open BTRIEVE files in Accelerated mode)

[If YES, the above 3 files are opened in accelerated mode.

This allows faster updates, inserts, and deletes.  However,

the information is held in a cache buffer with less

frequent disk writes.  Hence, you may lose data if the

system dies.  If you select yes, do backups often!!]

WA7MBL Sysop Documentation                                    26

(C) 1986 Jeff Jacobsen

FILE LAYOUTS

HELP\INFO.HLP      (info file - I command)

HELP\HELP.HLP      (help file - H command)

HELP\GHELP.HLP     (guest help file - H command)

HELP\LHELP.HLP     (local help file - H command)

HELP\YHELP.HLP     (yapp help file - YH command)

[These are help files used by the BBS.]

7    (Color attribute for console)

112  (Color attribute for status line)

15   (TNC A Color attribute)

7    (TNC B Color attribute)

7    (TNC C Color attribute)

7    (TNC D Color attribute)

7    (TNC E Color attribute)

7    (TNC F Color attribute)

[Color attributes may be changed if you are using a color

monitor so that data coming from the different TNCs appear

as different colors on the screen.]

2    (TNC A Port - 0 if none)

1    (TNC B Port - 0 if none)

3    (TNC C Port - 0 if none)

0    (TNC D Port - 0 if none)

0    (TNC E Port - 0 if none)

0    (TNC F Port - 0 if none)

[Specify the port used by each TNC in use.  The BBS will

support up to 6 ports if the appropriate I/O drivers have

been loaded.]

4800 (Baud rate on TNC A)

4800 (Baud rate on TNC B)

4800 (Baud rate on TNC C)

4800 (Baud rate on TNC D)

4800 (Baud rate on TNC E)

4800 (Baud rate on TNC F)

TNC A : 145.01 Mhz - Local LAN

TNC B : 7093 or 14107 Mhz - HF Gateway

TNC C : 146.02/72 Mhz - Repeater

TNC D : Not connected

TNC E : Not connected

TNC F : Not connected

[A description of the each TNC may appear here.  This will

appear during a J list.]

WA7MBL Sysop Documentation                                    27

(C) 1986 Jeff Jacobsen

FILE LAYOUTS

NO   (BBS only on TNC A)

YES  (BBS only on TNC B)

NO   (BBS only on TNC C)

NO   (BBS only on TNC D)

NO   (BBS only on TNC E)

NO   (BBS only on TNC F)

[YES will set the port for BBS access only.]

YES  (if BBS only on TNC A allow guests)

YES  (if BBS only on TNC B allow guests)

YES  (if BBS only on TNC C allow guests)

YES  (if BBS only on TNC D allow guests)

YES  (if BBS only on TNC E allow guests)

YES  (if BBS only on TNC F allow guests)

[If the port has been set for BBS access only, a YES here

will allow "guest" access by non-BBS stations.  Guest

access allows only limited usage.]

NO   (Hide guest mail)

[If YES, Mail left by guest will be "hidden" until released

by the Sysop with the H command.  If NO, the Mail will be

available for anyone to read (unless Private).]

2    (Max digi on TNC A)

0    (Max digi on TNC B)

0    (Max digi on TNC C)

0    (Max digi on TNC D)

0    (Max digi on TNC E)

0    (Max digi on TNC F)

[The maximum number of digipeaters a user may use to

connect is specified here.  A user digipeating through more

than the specified number will be disconnected.  A user

flagged as a BBS or remote sysop may connect, and this

limit will be ignored.]

nomode on

conok off

cmsg off

mon off

unproto mail

*** EOF    {end of TNC A init - sent to TNC A on startup}

[These commands will be sent to TNC A when the BBS is

started.]

WA7MBL Sysop Documentation                                    28

(C) 1986 Jeff Jacobsen

FILE LAYOUTS

nomode on

conok off

*** EOF    {end of TNC B init}

*** EOF    {end of TNC C init}

*** EOF    {end of TNC D init}

*** EOF    {end of TNC E init}

*** EOF    {end of TNC F init}

[A separate set of initialization commands is available for

each TNC.]

unproto cq

nomode off

*** EOF    {end of TNC A bye - sent to TNC A at shutdown}

[Commands sent to TNC A when the BBS is exited via the Q

command.]

unproto cq

nomode off

*** EOF    {end of TNC B bye}

*** EOF    {end of TNC C bye}

*** EOF    {end of TNC D bye}

*** EOF    {end of TNC E bye}

*** EOF    {end of TNC F bye}

[A separate set is also available for each TNC.]

WA7MBL Test BBS 3.13

LstMsg# $L Mail: $Q

*** EOF    {end of TNC A "beacon" text}

[This is the "beacon" text sent to TNC A.  It is not loaded

into the BTEXT, but is sent in converse mode by the BBS

program.  The text may be multiple line.  The $Q variable

is loaded with the callsigns of any unread messages (or the

string "None" if there are none).]

LstMsg# $L Mail: $Q

*** EOF    {end of TNC B beacon text}

*** EOF    {end of TNC C beacon text}

*** EOF    {end of TNC D beacon text}

*** EOF    {end of TNC E beacon text}

*** EOF    {end of TNC F beacon text}

[Different beacon text may be used for each TNC.]

WA7MBL Sysop Documentation                                    29

(C) 1986 Jeff Jacobsen

FILE LAYOUTS

mon on

conok on

*** EOF    {end of TNC ready for connect}

[These commands are sent to every TNC when the BBS is ready

and awaiting a connect.]

conok off

mon off

*** EOF    {end of TNC off during local sysop use or forwarding}

[These commands are sent to all TNCs before going into

local mode for sysop use or before forwarding.]

conok off

mon off

*** EOF    {end of TNC cmds to non-connected TNC ports when in

use}

[These are sent to the TNCs which are not connected when a

connect is received by a user.]

conok off

mon off

*** EOF    {end of TNC cmds to connected TNC port when in use}

[These are sent to the TNC which is connected when a

connect is received by a user.]

flow on

mon on

nomode off

*** EOF    {end of TNC commands when entering terminal mode}

[These are sent to the TNC after the T command is used to

enter terminal mode.]

mon off

flow off

nomode on

*** EOF    {end of TNC commands when leaving terminal mode}

[Sent to TNC after terminal mode.]

WA7MBL Sysop Documentation                                    30

(C) 1986 Jeff Jacobsen

FILE LAYOUTS

flow on

*** EOF    {end of TNC commands from user chat request}

[Sent to TNC before entering terminal mode after answering

or requesting a user chat.]

flow off

*** EOF    {end of TNC commands when leaving chat mode}

[Sent to TNC after chat.]

30   (Minutes between mail beacons)

[Beacons are sent when a user disconnects.  If there are no

more connect for this period of time, the BBS will send the

mail beacon.]

4    (Minutes timeout when waiting for user response)

[When in use, the BBS will wait this long for input before

determining that the user is no longer there and doing a

disconnect.  This time starts anytime the BBS is awaiting

input.  This value is also used for Console timeout.]

45   (Seconds timeout waiting for initial > during forwarding)

[When forwarding, if the other station has their BBS off

but their TNC allows a connect, you don't want to wait too

long for the menu line before timing out and going on to

the next forward.  This parameter allows you to set a

shorter time to wait for the initial prompt.]

20   (Seconds timeout while waiting for disconnect)

[After the BBS sends a 'D' command to disconnect, it will

wait this long for the disconnect to occur.  After this

time, it will send a second 'D' command to the TNC to force

an immediate disconnect.]

300  (Max # calls for CALLS.BBS)

[This controls the size of the CALLS.BBS file which is not

used by anything else in the program.  If you are not using

the CALLS.BBS file, you may set this number low.  Note: If

this number plus the numbers in the next 3 lines get too

big, you may run out of memory and receive the runtime

error "FF" - stack overflow.]

WA7MBL Sysop Documentation                                    31

(C) 1986 Jeff Jacobsen

FILE LAYOUTS

20   (Max # calls for "recent calls heard" (J) lists)

[This is how many calls show up on a J list.]

30   (Max # calls for monitored BBSs)

[This is the list that holds the last time heard for the

users in your BBS file.  It should be at least as big as

the number of users in your BBSBULL file. ]

200  (Max # for unread mail list)

[This is used to build the mail beacon and is used by the

forwarding file.  It is a list of all unread mail.

The best size will depend on the number of active unread

messages on your system.]

20   (Max # calls in unread mail $Q beacon variable)

[This sets the number of calls included in the $Q variable.

If you wish to beacon all mail, it may be as large as the

unread mail list above.  If you have a lot of active

messages and only want to beacon the most recent mail, you

may set the number down as low as you wish.]

YES  (Exclude connect with illegal callsign)

[If YES, an user with an illegal callsign (NOCALL, PK80,

etc) will be disconnected.]

NO   (YES to recognize Bell (^G), NO to ignore)

[If YES, your speaker will beep when a bell character is

received.  If NO, you will not hear bells.]

YES  (YES if Sysop available to chat, NO to ignore chat request)

[This is the starting value for sysop available.  There is

a command to change this once the BBS has started.]

YES  (Turn on logging)

YES  (Log file transfers)

YES  (Log message events)

NO   (Log local console commands)

YES  (Log system events - BBS startup, etc)

[The above commands control event logging into the LOG.BBS

file.  If the first line is NO, no logging will occur.

Some runtime errors are logged before the BBS is terminated

by the error.  It is recommended that logging be left ON.]

WA7MBL Sysop Documentation                                    32

(C) 1986 Jeff Jacobsen

FILE LAYOUTS

Enter JA - JF for TNC A to F.  Enter JK to show connected

stations.

[The above line is sent as a "help" line for the J command

if only a "J" is received.]

File exists - Type Y to erase:

Oh, Hello $I

$R just tried to connect.

Please stand by, $V would like to talk to you.

Hang on one minute, I will page $V.

$V did not answer, you might leave a message in the MailBox.

$V is not available, you might leave a message in the MailBox.

Enter Subject for Msg # $C:

Enter Text, ^Z (CTL-Z) or /EX to end:

[The above lines are various BBS messages and prompts:

Prompt if file exists. Erase and start new, or not.

Message to send user when get *** LINKED to or N command.

Sent to the user when a connect request occurs.

Sent when you break into a MailBox session to talk to the

user.

Response to the user when he wants to talk to you.

Message to send when you do not respond to the request to

talk.

Message to send when you are not available to chat.

Prompt for title of message.

Prompt for message text.]

You have unread mail - RM to read it - Please KILL after reading

You have unread mail

[There are 2 "You have mail" lines - The first longer one

is sent to normal users, the second shorter one is sent to

BBS/Expert users. I no longer send the list of waiting mail

in either case.]

$p/WA7MBL/$M/Logan,UT/$J$Kz

[This is the forwarding header that is prepended to any

mail that is forwarded.  There are two or three "standard"

headers.  Take your pick.]

Enter your first name (up to 12 characters)

13  (Minute of the hour to do forwarding)

[Each hour at this minute if the BBS is not in use, it will

begin forwarding.]

YES (Date format  YES = 01-Nov  NO = 861101)

WA7MBL Sysop Documentation                                    33

(C) 1986 Jeff Jacobsen

FILE LAYOUTS

[New more readable date format is available for J-lists, L

type mail list, and variable strings within the config file

(see $string info).]

NO  (Retain partial messages from non-BBS)

[If NO, a message that is interrupted by a Timeout or

Disconnect is discarded.  If YES, the message is kept. This

feature is included primarily for debugging, or for

stations in sparsely populated areas where paths are poor

and a partial message is better than none at all.

Warning: Setting this to YES may be hazardous to your

health.  If you start forwarding partial messages (or even

worse, start accepting partial messages from BBS stations

and then forward them) you may flood the system with

multiple, incomplete copies of the same message.]

NO  (Retain partial messages from BBS)

[See above discussion.  This line applies to messages from

stations flagged as BBSs and in most cases should be NO!]

YES (Show NTS traffic on L & LL lists)

[If YES, type S & T (NTS Service & Traffic) messages will

appear when a user does an "L" or "LL" command.  If NO,

they will not appear.  If you have folks who regularly

check the traffic, you may want to set it to NO and not

bother the normal users.  If you don't handle much traffic,

you should set it to YES, so the traffic will be noticed

and not sit on the system.]

YES (Drop @BBS upon receipt of mail @THISBBS)

[When a message is received by the BBS if the @BBS is the

local callsign, the @BBS is blanked.  This allows the

message to be forwarded according to the forwarding file.

Messages for local users will remain, as they are not

listed in the forward file anyway.  This allows less

intervention by the Sysop.  For example, if I were running

a regional gateway that was well known, any user in my area

could be addressed as  K7XXX @ WA7MBL.  As soon as the mail

arrives here, I drop the @ WA7MBL and check my forwarding

file and forward (if possible) to any minor BBSs or

personal mail-drop users.  The only disadvantage is that if

someone wants mail held here they will have to notify me to

modify my forwarding file so the mail is held.]

WA7MBL Sysop Documentation                                    34

(C) 1986 Jeff Jacobsen

FILE LAYOUTS

YES (Kill message after forward)

[If YES, messages will be killed (or moved to OLDMAIL if

save after kill is set).  If NO, messages will be marked

with a status of F after forwarding.  If YES, killing of

certain type messages may still be overridden by the

following lines.]

YES (Keep type 'A' message after forward)

YES (Keep type 'B' message after forward)

YES (Keep type 'F' message after forward)

[Keep type 'A' and 'B' are for W2JUP.  He enters the latest

ARRL bulletin as a message to send to a number of BBSs (ie

ALL @ W1XXX  ALL @ W3YYY).  Rather than enter the same

message a number of times, he addresses it to the first

BBS, then forwards.  Next he edits the message header to

change the status from 'F' to 'N' and the @BBS to the next

one then forwards again.  With the old code he either has

to set Kill after forward to NO or else keep copying and

editing the message from OLDMAIL.  This is mainly used with

non-MBL BBS stations that do not use bulletin flooding.]

YES (Save message after user/sysop kill)

YES (Save message after kill after forward)

[If YES, the above 2 lines allow killed mail (or mail that

is killed after forwarding) to be saved into a separate

subdirectory.  This will solve the problems for the foreign

users that HAVE to save a copy of forwarded mail.  If you

read the current FCC rules about unattended operation, you

may also find it wise to save mail.  Local Sysop may use

the KX command to kill mail without saving it.]

Send the file, ^Z (CTL-Z) or /EX to end.

*** Can't create file - Invalid file name or path.

*** Can't do it - Unauthorized.

*** Not found or None found.

*** There already is a file with that name.

*** Timeout.

*** Invalid Command.

*** Done.

[These lines are additional messages used by the BBS]

WA7MBL Sysop Documentation                                    35

(C) 1986 Jeff Jacobsen

FILE LAYOUTS

The Forwarding File

The file FWD.BBS contains information that drives the

automatic forwarding of messages.  If the file does not exist,

no forwarding is done.  The file contains four types of

forwarding command groups:

1 - DOS command (D group)

2 - TNC Parameter setting information (P groups)

3 - Text to Message command (T group)

4 - Forwarding information (F, G & H groups)

Forwarding is done at the minute selected in the forwarding

file.  Each of the above commands has an associated time

selection line.  This line allows you to choose the hour or

hours for each command to occur.

The time selection line may look like any one of the

following line:

00-23                                [forward all hours]

01,03,05,07,09,11,13,15,17,19,21,23  [forward odd hours]

02-05,11,13-17,22                    [selected hours]

Each group is separated from the next group by a series of

at least three "-"s.

DOS Command (D group)

A DOS command may be executed from the forwarding file by

including it in a D group.  The first line format is:

Column      Data

1         "D"

2         blank

3-end     DOS command

The second line is a time selection line.

The third and final line is the group separator.

A DOS command group may appear as follows:

D CHGFREQ /B 145.32

00-23

-----

WA7MBL Sysop Documentation                                    36

(C) 1986 Jeff Jacobsen

FILE LAYOUTS

TNC Parameter Command (P group)

TNC parameters may be changed from the forwarding file by

including TNC commands in a P group.  The first line format is:

Column      Data

1         "P"

2         TNC port

The second line is a time selection line.

The third and following lines contain TNC commands.

The final line is the group separator.

A TNC parameter command group may appear as follows:

PA

22-04,11-12

RETRY 10

MAXFRAME 2

DWAIT 8

-----

There should be a "P" group at the end of the FWD.BBS file

to reset TNC parameters back to default values for BBS

operation.

Text to Mail Command (T group)

This command is used to convert an ASCII text file into a

message or messages.  It is executed from the forwarding file by

specifying the file name in a T group.  The first line format

is:

Column      Data

1         "T"

2         blank

3-end     ASCII file name

The second line is a time selection line.

The third and final line is the group separator.

A sample group is shown below:

T SERVOUT.TXT

00-23

-----

WA7MBL Sysop Documentation                                    37

(C) 1986 Jeff Jacobsen

FILE LAYOUTS

Forwarding Commands (F, G, & H groups)

The F, G, and H groups contain lists of stations for whom

you forward mail.  They are grouped according to the callsign of

the BBS to which the mail will be forwarded.  These groups also

contain information on how to connect to the mailbox and may

have "script" commands for connecting via a NET/ROM or other

type network.

The first line of a Forwarding Group is:

Columns     Data

1         "F" , "G" or "H" depending on type of forwarding

2         TNC port

3         blank

4-end     Callsign of BBS forwarding to

A "G" does normal forwarding.  An "F" requests "reverse

forwarding" from the other BBS after normal forwarding.  An "H"

request reverse forwarding even if there are no messages to be

forwarded.  If the other BBS identifies itself as supporting

reverse forwarding, "G" will also request it.

The BBS callsign is used with the "X callsign" command when

requesting an immediate forward to a specific BBS, and may

include an SSID.  The callsign (less SSID) is also used in a

Reverse Forwarding file.

The second line is a time selection line.  In the case of

forwarding groups, there is a variation of the time selection

line available as follows:

00-23<4

A regular formatted time line followed by a "<" followed by

a number will select a maximum file size to forward during that

time period.  In the above case, files of less than 4000 bytes

will be forwarded every hour.  If the above option is used, the

next line must contain a normal time selection line for

forwarding any size file.  For example:

00-23<4        [less than 4K anytime]

23-03          [any size only during specified hours]

The next line contains an "@" followed by the connect path

to the BBS.  If you are connecting via NET/ROM, this would

contain the callsign of the NET/ROM node you initially connect

to:

@WB7TRX v WA7MBL-1

WA7MBL Sysop Documentation                                    38

(C) 1986 Jeff Jacobsen

FILE LAYOUTS

Following the connect command, there may be up to 10 lines

of script commands which control forwarding via NET/ROM.  The

four types are Send, Timeout, Yes, and No.

The send type begins with a "." and is followed by the text to

send to the connected node.

The timeout type begins with a "#" and is followed by the number

of seconds to wait for either a Yes or No response string before

giving up.

The yes type begins with a "+" and is followed by the text to

indicated the desired response to match for.

The no type begins with a "?" and is followed by the text that

indicates an undesired response.  If this response is received,

the BBS will disconnect.

A short example of a portion of a G group containing a NET/ROM

script follows:

GA W0RLI

00-23

@SFO         [connect to SFO NET/ROM]

.C W0RLI     [command to send to NET/ROM]

#20          [timeout if no match within 20 seconds]

+to          [good - part of "Connected to W0RLI"]

?fm          [bad - part of "Busy fm W0RLI"]

[Message list follows here]

Only 1 yes and 1 no response is permitted.  The timeout is used

in case any other bad response is received.  If no timeout value

is listed, the waiting for ">" timeout time from the

configuration file is used.  It is possible to include a series

of script lines up to a total of 10.  You may leave out the No

response (or both the No and Yes response).  Two "send" lines in

a row may be used to send multiple lines of text.

GA W0RLI     [a more complex example]

04-22<3K

23-03

@AA6TN-1     [uplink to AA6TN NET/ROM]

.C SFO       [tell AA6TN NET/ROM to connect to SFO NET/ROM]

#30          [timeout time]

+to          [wait for connect to SFO]

.C W0RLI     [tell SFO NET/ROM to connect to W0RLI]

+to          [good - part of "Connected to W0RLI"]

?fm          [bad - if RLI is busy]

W0RLI

-----

WA7MBL Sysop Documentation                                    39

(C) 1986 Jeff Jacobsen

FILE LAYOUTS

The connect script (or the "@" line with the connect info)

is followed by the message list.  The message list is a list of

stations to forward to.  The list may also contain "Wild cards"

and up to 20 exceptions not to forward to.  For example:

FA WB7TRX

00-23

@WB7TRX

!WA7MBL        [don't forward any MBL mail]

!WA7MXZ        [don't forward any MXZ mail]

*              [forward everything else]

----

Could be used to cause my BBS to keep all mail for WA7MBL and

WA7MXZ and forward everything else to WB7TRX.

!NTSUT

NTS*

would be used to keep all NTSUT and forward all other NTSxxx.

The exception list and wild card feature does not apply to

the list of bulletins to forward (those beginning with "$").

There is no limit to the number of groups or the number of

calls in each list.  Your BBS will do the connect and send the

message onward.  The message will then be marked with an F

status (forwarded) or be deleted, depending on the setting of

various options in the Configuration file.

WA7MBL Sysop Documentation                                    40

(C) 1986 Jeff Jacobsen

FILE LAYOUTS

REVERSE FORWARDING

"Reverse forwarding" has been added.  After you have forwarded

to selected stations, you will send "F>" in response to his

command prompt.  He will then send any mail that is to be

forwarded to you, and your machine will automatically capture

it.

Note - please only use the "H" version for the following reason:

If you are set up to change frequency, the H version will allow

you to ask another BBS if it has any mail for you even if you

have nothing for it.  The only time it makes sense to do so is

if you are moving from a frequency that you normally operate on

to a different frequency for the purpose of sending and

receiving mail from another BBS.  This allows someone running a

single port local BBS to tie into a network without a second

port.

If both BBSs can change frequency, you probably don't need the H

version.

The code in FWD.TNC may look something like this

D CHGFRQ 145.09     (chg freq - you write it!)

00-23

------

HA WA7XYZ           (take care of lone BBS on 145.09)

00-23

@WA7XYZ

WA7XYZ

------

D CHGFRQ 145.01     (chg back to normal freq)

etc...

Reverse forwarding has the option for a separate file, or it can

run off of your existing FWD.BBS file.  The main reason for a

separate file is if you have alternate routing.  (For instance,

if you first attempt to forward certain mail to K7PYK but later

in the forwarding file route the same to KR5S in the event you

didn't connect to K7PYK, you may not want the reverse file to

send the mail to KR5S just because he has traffic to you.

Reverse forwarding ignores parameter lines, DOS command lines,

and Text to Message lines.

It only looks at F, G & H  groups.  The callsign of the

connecting BBS must match the callsign (not including SSID) on

the F, G or H line.  Port information is ignored.  Lines

containing connect script information are ignored.  Forwarding

times and file sizes are NOT ignored.

WA7MBL Sysop Documentation                                    41

(C) 1986 Jeff Jacobsen

FILE LAYOUTS

The Log File - LOG.BBS

Each line in the log file contains an event code, the date and

time, followed by further information about the event.

'S' - System event.

S8612181023I *** BBS Initialize    (Program started)

S----------A *** BBS Online        (Awaiting connects - A cmd)

S----------L *** LOCAL mode        (Sysop went to local mode)

S----------Q *** BBS Quit          (Program terminated)

'C' - User connected to system.

C----------A W1BBS   (connect on TNC A)

C----------B WA7MXZ  (connect on TNC B)

etc

C----------F K9DOG   (connect on TNC F)

C----------L W2XYZ   (a *** LINKED to from a gateway)

'X' - Exit.

Logoff after normal connect

X----------B       Normal Bye from menu

X----------D       User Disconnected

X----------F       Forced Off by sysop

X----------H       Requested Help file too many times

X----------T       Timeout

X----------?       Too many Illegal commands

Immediate logoff

X----------E       Excluded user

X----------G       User on BBS only port - Guest mode OFF

X----------I       Illegal callsign

X----------P       Too many digiPeaters

'F' - File event.  Command line shown as user entered it.

'Y' - Yapp File event.  Command line shown.

WA7MBL Sysop Documentation                                    42

(C) 1986 Jeff Jacobsen

FILE LAYOUTS

The Log File - LOG.BBS continued

'M' - Message event.  Message number shown.

Normal message events

'K' - Message has been killed.

'L' - Message headers listed.

'R' - Message has been read by user.

'S' - Message has been sent by user (received by us).

Addressee shown.

Forwarding message events  [subject to change]

'F' - Message has been forwarded to another BBS.  Connect

path shown.

'$' - Forwarded a bulletin.

'<' - Message received in response to a reverse forward

request sent to another BBS.  Connect path shown.

'>' - Message sent to connected BBS in response to a

reverse forward request from them.

'N' - Message delivery attempt rejected by receiving BBS.

'*' - Message created automatically upon receipt of a

REQFIL or REQDIR message.

'E' - Run time error.  Error message shown.

'T' - Text to Message event

(Messages from the SERVER)

WA7MBL Sysop Documentation                                    43

(C) 1986 Jeff Jacobsen

FILE LAYOUTS

Files used by the BBS.

BBS.COM       - The BBS code

BBS.000       - The BBS code (overlay file)

BBS.001

BBSxxx.CNF    - Configuration data, message of the day,

help command text, error message text, etc.

FWD.BBS       - The routing tables for forwarding messages.

RVSFWD.BBS    - Optional reverse forwarding file.

SERVER.TXT    - File of received messages addressed to SERVER.

LOG.BBS       - The station log.

JLIST.BBS     - The saved "J" lists.

CALLS.BBS     - List of all calls seen in monitor mode.

BBSMAIL.BTR   - The mail headers.

BBSUSER.BTR   - The user database.

BBSBULL.BTR   - The BBS database.

Mail messages are stored as individual files using the Mail path

specified in BBSxxx.CNF as MSG00001.MAI, etc.

BBSMAIL.PRE, BBSUSER.PRE, or BBSBULL.PRE may appear during use.

These are temporary files created by BTRIEVE and may be ignored.

WA7MBL Sysop Documentation                                    44

(C) 1986 Jeff Jacobsen