Rocky Mountain Packet Radio Association

The WA7MBL Packet Bulletin Board System

USER GUIDE

This guide compliments of the Rocky Mountain Packet Radio

Association - May 1987.  Prepared by Steve Linn, N4CAK with input

and review by Bob Gobrick, WA6ERB

Along with all the commands needed to operate your Packet TNC,

most PBBS's (or mailbox's) have a number of additional user

commands.  This guide provides a quick overview of the WA7MBL

Version 3.13 PBBS system, and is intended to assist you in its

efficient use.

The PBBS software was written by Jeff Jacobsen, WA7MBL, of Logan,

Utah (a RMPRA member).  Much of the format is styled after the

PBBS software work pioneered by Hank Oredson, W0RLI.

--- What does the PBBS Do? ---

The PBBS provides the following resources and functions:

> Personal mail identified by call can be sent and received

> General files, programs, documentation, maps, etc.

> Frequency activity monitored (stations heard)

> Automatic forwarding of bulletins to other PBBS's

> Automatic forwarding of mail via HF PBBS Gateway station

How do you log on?

You begin a PBBS session by simply connecting with the station

running the PBBS.  Sometimes, however, the response coming back

will indicate the PBBS is busy.  When the PBBS is available

again, it will transmit its beacon.  You may now attempt to

connect again.  The PBBS automatically records your call from the

connect message and knows who you are from then on.  You then get

a greeting message and a prompt line.

Commands are one or two letters.  Some commands are followed by a

message number, call or file name.  For these commands, you must

leave a space between the command and the message number, call or

file name. During your connection to a PBBS, you send a series of

commands to perform the functions you desire and at the end you

send a B (bye command) to disconnect from the PBBS.  I'll explain

what the commands are as we go on.  At the end of this guide you

will find a consolidated list of commands.  If you feel a need to

rush in before reading all of this, PLEASE send your commands

only once.  If you don't see any response, wait, do not send the

command again.  I'll explain later.

OK.  The first time on you'll probably see "Please use N command

to enter your name" followed by "Type H (return) for help".  So

type  N Steve <cr> (assume for the rest of this guide that you

hit a return after each command) and the system comes back with

Hello Steve or something similar.  Now the PBBS and the SysOp

(System Operator - that generous person who has contributed his

equipment and time to make this PBBS available) know who you are.

Before we go too far, you need to know how data is stored on the

PBBS. There are two main formats for data which you can access.

These are not interchangeable and you must make sure that you are

using the correct commands for the type of data.

Mail Messages:

Most of your interaction will probably be with the mail system

and mail messages.  Mail messages are to and from specific users

or to generic users like ALL.

Data Files:

This is the other format of storage and can contain any kind of

data. These files generally contain larger amounts of data such

as newsletters, programs, maps, etc.  Data files are always named

with 1 to 8 characters, a period and 0 to 3 more characters.

Examples are USER.DOC, COLORADO.MAP, etc.  Using the same suffix

(i.e. .MAP) allows grouping of files of similar type.

Now that you are rolling, let's see what other commands are

available and what you can do.

--- HELP-TYPE COMMANDS ---

These commands give the user information about the system and how

to use it.

Command: H or ?

HELP - Gives general HELP with all system commands.  It isn't

context sensitive - it always gives the same answer regardless of

where you are in the system.  This shouldn't be necessary if you

have this guide.

Command: ?[Letter]

Detailed HELP with individual system commands.  For example, type

"?U" for HELP with uploading.  The descriptions in this guide

are similar to what you would get with this command.

Command: I

INFORMATION - about this particular PBBS - equipment, stations

served, links, SysOp, etc.

--- MESSAGE-TYPE COMMANDS ---

These commands are used to read and send personal messages to

other users.  All messages are referred to in every command by

number, and you find out what the number of each message is by

using the LIST command.  You will not see any of the message

text.

The READ command is for looking at the message text.  The

highest current message number and the number of outstanding

messages are shown in the prompt line when you log in as well as

in the beacon text, so you should have some idea of what the

active messages are.  The first time you log into a PBBS, you may

wish to list all messages.  PLEASE don't list all messages each

time.  It takes a long time and ties up the frequency needlessly.

There are several variations of the list command. Each is

explained below, but first let's look at a sample message list

and explain what all the information means.  Here's a sample

message list:

Msg# TS  Size TO     @ BBS  From   Date   Subject

5059  Y   102 N4CAK         WA6ERB 05-May Tnx for User Manual

5054 BN 15659 ALL    MAP    KC0RL  05-May Colorado.Map 5/1/87

5053 BN   723 ALLCO         KA0MQA 05-May Swapfests

5052 TN  1172 NTSCO         W1HAB  05-May Traffic for N0IA

5051  N   126 K0VLD  K0VLD  W0HJX  04-May Dual port BBS

5050 PY   243 WA6ERB        KE6LT  04-May 450 Link

Msg# - This is the message number.  You will use this number in

other commands such as read or kill. You designate which message

you wish to read by specify its number in the read command.

T (Type) - The message type designation.  This allows

categorization of messages.  B indicates a bulletin message, T an

NTS Traffic message and P a private message (private messages can

only be listed or read by the sender, recipient and SYSOP).

S (Status) - This column contains Y or N depending on if the

recipient has read the message (it is not changed if someone

OTHER than the intended recipient has read it).  If N, the

message has not been read and the recipient call still appears in

the beacon text.  The column also can have an F or $ to signify a

message or bulletin that has been forwarded as part of the

automatic PBBS forwarding system.

Size - This column tells you how big the message is in bytes.  On

longer messages, you might want to capture the message to disk as

it comes into your computer so you can read it later.  You can

abort a listing by typing the letter A<cr> if it is too long.

TO - This is the intended recipient of the message.  It is

specified by the user sending the message.  It's generally a

callsign or ALL. A name like ALLCO, or something like that, is

used to specify mail to be forwarded to all users at one or more

other PBBS's.  We will discuss message forwarding later.

@ BBS - This column indicates another PBBS that this message is

to be forwarded to.

From - This column is set automatically to the call of the

station sending the message.

Date - This is the date that the message was posted at this PBBS.

It may not be the same date that the sender sent it if it was

forwarded to the PBBS from another PBBS.

Subject - A description of what the message is about.  It may

also contain "Forwarded from ..." for messages from another PBBS.

It is set initially by the originating station and can be changed

by the SYSOP.

--- LIST COMMANDS ---

Command: L

LIST new messages - List any new messages that have been posted

to the PBBS since the last time you used the L command.

Command: LM

LIST MINE - List only messages to you.

Command: LN

LIST NEW - List only unread messages to you.

Command:  LL x

LIST LAST - List the most recent number of messages specified by

x. LL 10 will therefore list the last 10 messages.

Command: L x

LIST From # - List only messages above number x.  L 325 list

messages above #325.

Command: L x y

LIST From # To # - List a group of messages.  L 300 325 list

messages from #300 to #325.

Command: La

LIST TYPE - List messages of type 'a'.  LB lists bulletins, LP

lists private messages (although you will only see those sent by

you or to you), LT lists NTS traffic messages (mainly used by NTS

traffic handlers), etc.  You can also do a La x or La x y.

Command: L> callsign

LIST TO - List all messages sent to the specified call sign. L>

N0CCZ will therefore list all messages currently in the PBBS that

were posted there for N0CCZ.  Entering L> your-call is the same

as entering LM.

Command: L< callsign

LIST FROM - List all messages sent by the specified call sign.

L< W0HJX will therefore list all messages currently in the PBBS

posted by W0HJX.

--- READ COMMANDS ---

Command: R x

READ - Read message #x (not "files").  You may have up to 6

messages numbers per line.  R 325 327 340 would list out those

three messages.

Command: RM

READ MINE - Read all messages addressed to you.

Command: RN

READ NEW - Read all new messages addressed to you.

In version 3.13, messages read with the R command have any standard

forwarding headers stripped off.  This routing information is normally

not needed by the user.  To read the entire message text, you may use

the V (VERBOSE) versions of the above R commands.

Command: V x

VERBOSE READ - Read message #x.  You may also have up to 6

messages numbers per line.  V 325 327 340 would list out those

three messages.

--- KILL COMMANDS ---

Command: K x

KILL - Kill a specific message.  Only the sender, recipient or

SYSOP can kill a message.  You may have up to 6 messages numbers

per line.

Command: KM

KILL MINE - Kill all messages sent to you that you have read.

This will not kill messages sent to you that you have not read.

Please kill your messages that no longer need to be on the PBBS.

--- SEND COMMANDS ---

Command: S callsign

SEND - Sends an open message to 'callsign'.  Others can read this

message.  You can not send S alone - you need a callsign.

Command: SP callsign

SEND PRIVATE - Send a personal message to 'callsign'.  Others

cannot read this message, although the 'callsign' will appear on

the beacon. Preferred to cut down on long L listings by others.

Command: S callsign @ callsign

SEND VIA - Send a message to a station at another PBBS by

automatic mail forwarding.  SP N4CAK @ WA6ERB would send a

private message to N4CAK at WA6ERB's PBBS.  Forwarding via HF is

covered later. Typically, once a message is forwarded, the

originating PBBS will automatically delete it.

Command: SB ALL

SEND BULLETIN - Send a general bulletin addressed to ALL.

Command: ST NTSxx

SEND TRAFFIC - Send NTS traffic with xx as the state

abbreviation.  ST NTSNY would send NTS traffic for NY.  Your

subject column should contain additional NTS information about

recipient - callsign, city, state, etc.

After you've told the PBBS who the message is to, the PBBS will

then ask you to supply a title for the message.  This entry will

show up in the message list in the Subject column.  Please be

brief.

You will then be asked to enter your message.  When finished, you

terminate text entry by sending a control-Z.  (Some user computers

are unable to send a control-Z.  In this case, you may place the

command /EX on a separate last line).

If you are going to be entering a rather long message, it's a

good (excellent) idea to compose the message off line with a word

processor or even a simple text editor.  Then connect to the

PBBS, execute the send command, enter the title and then send the

precomposed message text to the PBBS.  You could even include the

send command and message title in the precomposed message as the

first and second lines of the message.  Add /EX as the last line

of the message and you can completely automate your message

entry.

--- FILE-TYPE COMMANDS ---

The following commands deal with data files, which generally

contain text material (AMSAT Bulletins, RMPRA NEWS, area user

maps, etc.) or program files.

Command: W

WHAT files - List of all the data files available on the PBBS.

Recall that data file names are of the format xxxxxxxx.yyy.  You

may use wildcards, such as W *.DOC will list all files with the

extension of DOC (Documentation).

In the file list, you may also see file names followed by the

designation <DIR>.  This indicates a file subdirectory, which

will contain a number of additional files.  Usually files that

are related will be placed in a subdirectory.  All AMSAT

Bulletins might be placed in a subdirectory named AMSAT.  To list

the contents of a subdirectory enter the W command followed by

the subdirectory name.  W AMSAT will list all files in the AMSAT

subdirectory.  Many subdirectories will have a "README" file

which may give you information about the files in that

subdirectory.

Command: WN

WHAT NEW - List of new files since you last logged on.

Command: D filename

DOWNLOAD - Transmits a file from the PBBS to you.  The D is

followed by a filename you want to download, such as D USER.DOC.

You generally want to have the capture buffer on your computer

open to receive the incoming file.

If the file you are downloading is in a subdirectory, you precede

the file name with the subdirectory and a slash, such as D

MAP/COLORADO.MAP if COLORADO.MAP was in the MAP subdirectory.

Command: D filename x

Partial Download - If 'x' is 10, you would see first 10 lines.

If 'x' is -20, you would skip the first 20 lines.  If 'x' is 5 15

you would see lines 5 to 15.  This is valuable if you want to

'preview' a file before you download it.

Command: U filename

UPLOAD - Send a file from you to the PBBS (opposite of the D

Command). The file naming convention is the same for uploading as

it is for downloading, including the subdirectory name if any.

Terminate the upload by sending a control-Z.  As an example, to

upload the Denver ham exam schedule, type U EXAMSCH/DENVER.SCH.

One danger in uploading to the PBBS is that you could use up all

of the remaining space on the system.  This can crash the system

since the mail, user and log files need room to expand.  ALWAYS

check to see how much space is left on the disk with the W

command before you upload something.  Don't push the storage.

You may be looking for a command to kill a file, similar to the K

command which removes a message.  Sorry, there's nothing like

that. Only the SYSOP can do that from the console - leave a

message to him making the request.

--- OTHER COMMANDS ---

Command: A

ABORT -Abort a listing, either mail or files.  A fair amount of

the message or file may be buffered in the PBBS's TNC, so the

abort is not instant, but it will work.  Always try a partial

download (ie. D MAP/WYOMING.MAP 10) first to see if the file is

what you want. Otherwise hit the A <cr>.

Command: B

BYE - Log off the PBBS (gracefully) and disconnects your call.

You will be logged off automatically if there is no response to a

menu in 4 minutes due to inactivity or path timeout.

Command: J

Calls Heard - List of calls heard by up to 6 TNC's, the channel

or service assigned to each TNC, and the stations that have been

connected to this PBBS.

Command: JK

Calls of users that have connected to the PBBS.

Command: Ja

Calls heard - on TNC 'a'.  Type A, B, C, D, E or F for up to 6

TNCs.

Command: N name

NAME - Enter or change your name in the database.  You can use up

to 12 characters.  Phony names will be canceled.

Command: T

TALK - Pages the SYSOP.  If the SYSOP is available to chat,

you'll get a response within ONE MINUTE.  Otherwise, the PBBS

will advise you that SYSOP hasn't answered.  You'll be returned

to the command prompt and can continue normally.  You may leave a

message with the SYSOP if the SYSOP is not available.

Command: V

VERSION - Shows specific version of this PBBS, the date of the

software release, number of active messages and the next message

number.

Command: X

EXPERT - You can change your status to expert.  This will give you

a shorter menu prompt, no Welcome message when you log on, and no

Goodbye message when you log off.

Command: Y

YAPP - Binary transfer programs.  Currently, the YAPP program is

available ONLY for IBM-PCs and compatible MS-DOS computers.  Type

Y or ?Y for the HELP listing on YAPP - it's almost two pages

long.

--- FORWARDING ---

Automatic forwarding of messages to other PBBS's adds a new

dimension to packet message handling.  It allows mail to be

passed between PBBS's automatically, according to a pre-defined

list.  The forwarding can occur in off-hours to minimize traffic

on the frequency or can occur on a different frequency.

Each mailbox has the capability to maintain a list of all

stations and their "home" PBBS.  When not connected to another

user, the PBBS checks the mail file each hour for stations with

mail on the forwarding list.  If there is mail to any user on the

forwarding list and it is the proper time to forward to that

station, the PBBS goes off line, connects with the other PBBS and

sends the mail.  If the connect is not successful or the other

PBBS is busy, the mail is not forwarded and waits until the next

forwarding time (generally an hour later).

Each PBBS in the link is assigned a forwarding time - a certain

number of minutes after the hour.  These are coordinated to

prevent 2 stations from trying to forward to each other at the

same time.  When not inhibited, forwarding occurs at the same

time each hour for a given station.  Start and stop times can

also be specified for each forwarding time to each destination.

This allows inhibiting forwarding during busy peak times.

You don't have to do anything special to use forwarding.  The

SYSOP is responsible for getting the forwarding files set up.

All you do is send the message.  Mail sent to a user served by

another PBBS will automatically be forwarded if that user is in

the forward file or you assign an @ BBS.  Typically, all

mailboxes in a given area will have entries in their forwarding

files for all other PBBS's in the area, perhaps even for the

entire country. So if you know that N4CAK reads his mail on the

WA6ERB PBBS, all you need to do when you send your message to

N4CAK is enter S N4CAK @ WA6ERB.  Your PBBS will then forward it

on to WA6ERB where N4CAK will see it.

However, HF forwarding is a little different, at least for now.

If you would like to send non-NTS HF traffic (by a 20 meter HF

Gateway) from the Colorado area as an example, you need to do the

following:

-1. Address mail to the packet station that will receive it in

the TO column (packet equipped station only for now).

-2. In the forward column (@), put either W1HAB (Boulder), K0HOA

(Colorado Springs), KC0QJ (Walsenberg) or KN5O (New Mexico).

These are the packet HF Gateway Bulletin Boards for the Rocky

Mountains and are the only stations that will forward HF traffic

at this time.

-3. In the Subject column put the following info: HF Traffic to

City, ST - W1xxx BBS.  This tells the Gateway station that it is

HF traffic to a certain city and state and if you know the local

BBS in the recipients area.

-4. Finally, when you sign your message, tell the other station

your "home" BBS and the Gateway that he should send to.

If you have a HF BBS question, leave a message for the HF Gateway

BBS Sysop by way of your local PBBS.

NTS traffic forwarding is similar and your NTS packet coordinator

can assist you in the proper use.

If you travel and operate packet at other PBBS's, you might want

to add an SSID number to your call, such as K0GUZ-1.  Then when

the PBBS's share forwarding files, only one (your HOME) PBBS will

have your plain call sign (such as K0GUZ), and mail sent to you

will be forwarded to only one PBBS and not numerous ones.

Not every PBBS is set up to forward to every packet user.  The

only way a SYSOP will know what your home PBBS is if you tell

him.  It might be a good idea to send a message to your home PBBS

SYSOP and ask him to inform the other PBBS's of your "home"

status.  That way, if someone posts a message to you on a PBBS

across town and doesn't specify your home PBBS when sending the

message, the PBBS itself will have a record in its forwarding

file of your call sign and what your home PBBS is.  That PBBS

will then automatically forward the message on to you at your

home PBBS.

There are certain "ALL" destinations that are used to send mail

to all users at a remote PBBS.  For example, ALLNCO will

distribute a message to all northern Colorado PBBS's, as ALLSCO

will do for southern Colorado.  What "ALL" designations are

available and where the messages so addressed will go is

determined solely by the SYSOPs of the various PBBS's.  You might

leave a message for the SYSOP of your home board asking for a

copy of his forward file. This will tell you everything you need

to know about what goes where.

For Bulletins needing wide distribution, a "flood" system has

been designed.  You need only Send the Bulletin to ALL (SB ALL),

then send a second message to your SYSOP asking that message

number 'x' be distributed to whatever area you want.  The SYSOP

will then set up a flood for that message for you.  This is

preferable (and faster) to you sending multiple bulletins @

different PBBS's.

--- Good Operating Ideas from a Sysop's Viewpoint ---

The following ideas come from many hours watching the SYSOP's

screen and seeing difficulties that users have had.

Do not send commands more than once.  New users to the PBBS will

occasionally send a command, wait for a response, and seeing

nothing will become impatient and send the command again.  This

is unnecessary because in packet radio,  either the packet

will get through to the other station or you'll "retry out".  If

you're still connected, the packet will get through.  If you send

the command twice, the PBBS will respond twice.

Repeatedly executing the information or help functions is

unnecessary. The H or ? (HELP) command prints a summary list of

commands.  The I (Information) command gives information on the

particular PBBS installation.  They aren't context sensitive and

their responses won't vary depending on what you're doing in the

PBBS.  The information is always the same.  Therefore, there's no

need to print it more than once, but remember to print it once.

If possible, stack multiple commands using the PASS character

(usually control-V).  With at TAPR TNC or clone, each time you

hit return, you send a packet.  You have to end a command to the

PBBS with a return so you have to send it.  You can save air

time, though, by "stacking" commands in the same packet by

preceding the return with the PASS character.  This will put all

the commands in the same packet.  Of course, don't use the PASS

character before the last return or you'll never send the packet

at all!

Don't list all messages.  The message file gets pretty long

pretty fast and listing the whole file takes a long time.  Along

these lines, please be courteous during peak usage hours - limit

your access time so others may pick up their mail.

Finally , please recognize two things.  First, it takes a good

deal of time to properly run a PBBS , so be patient with

occasional problems which may occur.  Disks have to be backed up,

files moved around, old messages deleted, and new versions of the

program have to be loaded.  ENJOY!

Rocky Mountain Packet Radio Association

Command Summary for MBL313 PBBS

A - ABORT a listing

B - BYE, logs off PBBS and disconnects your call

D - DOWNLOAD a file FROM the PBBS

H - HELP information file

I - INFORMATION about this PBBS

J - List of calls heard by this PBBS

JK - Connected

JA - heard on TNC-A, JB - heard on TNC-B, etc.

K - KILL a message

K x - a specific message

KM - Mine that I have read

L - LISTs messages in the mailbox

L - New messages since last using the L command

LM - Only messages to you (List Mine)

LN - Only unread messages to you (List New)

LT - List NTS Traffic (NTS Traffic Handlers)

LL 10 - List last 10 messages

L< callsign - Messages from 'callsign'

L> callsign - Messages to 'callsign'

L@ callsign - Messages sent care of 'callsign'

L x - Messages above a given number 'x'

L x y - Group of messages from 'x'' to 'y'

N - NAME - N (your first name)

R - READ messages (not "files") without forwarding headers

R x - Message number 'x' - up to 6 message numbers

RM - Messages addressed to you

RN - New messages addressed to you

S - SEND messages (not "files")

S callsign - Send message to 'callsign'

SP callsign - Send private message to 'callsign'

S callsign @ pbbs callsign - Send message to a station

at another PBBS

ST NTSxx - Send NTS traffic for 'xx' state (NTS handlers)

SB ALL - Send a general Bulletin addressed to ALL

T - TALK to the SysOp - pages the PBBS System Operator

U - UPLOAD a file TO the PBBS

V - PBBS software VERSION, number of active messages, next

message number

V - VERBOSE READ messages including forwarding headers

V x - Message number 'x' - up to 6 message numbers

VM - Messages addressed to you

VN - New messages addressed to you

W - WHAT's available (File Section Directory)

X - EXPERT Status - EXTENDED prompt line

Y - YAPP Binary File Transfer - Y alone same as ?Y

Y or ?Y or YH - YAPP help file

YW - Binary file directory (What)

YN - Binary new files (What New)

YU - YAPP Upload

YD - YAPP Download