COMBIOS was originally furnished by AA4RE, and modified extensively

by WA7MXZ and WA7MBL.

COMBIOS, COM1BIOS, COM2BIOS and QUADBIOS are routines that were

written to replace the standard Int 14 routines included in the

IBM ROM BIOS for use with the WA7MBL BBS.

COMBIOS  supports the standard COM1 & COM2 ports.

COM1BIOS supports only COM1.

COM2BIOS supports only COM2.

QUADBIOS supports the 5 ports on a Quadram Quadport AT board.

When using the Quadram board, you may only have one standard serial

port in use, thus QUADBIOS may be loaded in conjunction with either

COM1BIOS or COM2BIOS for a total of 6 ports.

You may also load both COM1BIOS and COM2BIOS in place of COMBIOS, but

there is no reason to do so (unless you want to use up a little additional

memory.)

Due to the shortage of interrupt lines in the IBM PC there seems to be no

standard for COM3.  However, it is possible to modify COMBIOS to add an

additional port, or COM3BIOS can be written.  If you write additional

drivers for specific machines, I would appreciate a copy of any that are

written.

The routines are called in the same manner as the standard Int 14 calls

are made.  However, there are more possibles calls as determined by the

value passed in AH.

The port number should be passed in DX.  (0-COM1 1-COM2, etc)

Valid values for DX:

COMBIOS  (0,1)

COM1BIOS (0)

COM2BIOS (1)

QUADBIOS (2,3,4,5,6)

AH = 0     Initialize

---------------------

This initializes the communications port.  AL contains baud rate, parity,

stop bit, and word length as in the standard BIOS call.  On return, AH

contains the Line Status Register information (except bit 0 - Data ready

tells whether there is information in the extended buffer rather than the

SIO chip receive buffer).  AL contains the Modem Status Register.

AH = 1     Send Character

-------------------------

AL contains the character to send.  Return values are the same as above.

AH = 2     Receive Character

----------------------------

On return, AL contains the character received.  AH contains the contents

of the LSR at the time the last character was received from the TNC (not

necessarily the character this routine returns).  Again, AH bit 0 indicates

if there is additional characters in the buffer.  This routine does NOT

timeout while waiting for a character.

AH = 3     Return Status

------------------------

Return values are the same as returned by the Initialize routine.

AH = 4     Inquiry

------------------

AH returns 0AAH.  AL returns 055H.  This call is used to see if the COMBIOS

driver has been loaded for a particular port.

AH = 5     Drop RTS

-------------------

This drops DTR (pin 20) and RTS (pin 4).  Used to tell the TNC not to send

any more information.  TNC-1 and PK-232 use pin 4 for handshaking.  TNC-2

uses pin 20 (dont ask me why!)  No return value.

AH = 6     Raise RTS

--------------------

Raise DTR and RTS.  No return value.

AH = 7     Send Break

---------------------

Generates a "Break".  Useful for getting TNC-2 or PK-232 into Command mode

from Converse or Transparent mode.  (However, if the TNC is sending characters

at the time, they will be garbled.)  No return value.

AH = 8     Non-destructive Read

-------------------------------

Returns the next character in the buffer in AL without removing it from the

buffer.  AH value undefined.

AH = 9     Reserved

-------------------

If you wish to add extra functions, please either leave a few functions

open, or try to coordinate any added functions so we can have a compatible

set of I/O modules.  Thanks.

Jeff Jacobsen, WA7MBL

1400 E 900 N

Logan, UT  84321