Msg # 6287  Stat:N  To: W2JUP   From: N1DL    Date: 04-Jan/1036

Subject: rough notes on Quadboard

N1DL  /N1DL/1992/Huntington Bay/NY/8701041502 (!516)

First rough info that should help get started:

I strongly suggest to first remove any other serial or parallel communications

board and try to get the system working with the QUADPORT board first before

experimenting with a mixed arrangement.  (Note: if I had done this it would

have saved me "muchos horas".)

I found the most confusing thing the way the ports are assigned and the

QUADRAM supplied manual is very poor on this.  The 9 pin socket on the

motherboard is actually port 1   ... but in the MBL BBS software is addressed

as 3 in the configuration file.  (another timesink if you experiment without

documentation) As there are no markings on the daughterboard sockets (four)

you have to look from behind and will find markings J2, J3, J4 and J5 if you

look carefully.  Suggest you remark the sockets on the plugside accordingly.

eg.  J2 = B    J3 = C    J4 = D      J5 = E

Don't forget the motherboard socket is  A  port.

Unless you are very sure that all ur cables and TNCs and config files and

hardware is working properly I suggest you start with making one channel

work first with the Quadboard.

Contrary to the Quadram supplied manual you DO NOT NEED the QPMODE.COM

program and no initial software load with the supplied diskette is required.

Here are the PIN configurations in case you are starting with a standard

25 pin RS232C cable.

25Pin        9Pin

-----        -----

8            1

3            2

2            3

20            4

7            5

6            6

4            7             suggest you write this next to the info

5            8             given by Quadram on page 27 of the QUADPORT

22            9             Operations manual.

The correct jumper settings on the Quadboard when it is used alone (as the

only COM board in the PC) are:

J1   2-3

J2   1-2

J3   2-3

J4   3-4

J5   5-6

J6   2-3

This will give you 5 serial channels (in Quad mode) and 1 printer parallel

channel.

When you are all ready to run the software .... make sure your AUTOEXEC.BAT

file includes

QUADBIOS

BTRIEVE /C

BBS

your CONFIG.SYS file should have

FILES = 20

Buffers = 30        or higher numbers for both .. but these work.

Your config file should look similar to the example given in exhibit A.  NOTE

the Port assignments and make sure the Terminal Baud rate on the back of ur

TNC2's (or equivalent) is the same as the Port Baud rate you specified in your

CONFIGURATION file for the BBS program.

IF the PC upon power on goes thru its initialization routines, then the

autoexecute segment with QUADBIOS and Btrieve and then loads the BBS program,

be patient ... the 3.11 Revision takes about a minute and a half to load.

If after this time a blue bar appears (or grey bar in Black/White systems) and

nothing further happens ... chances are very good your TNC's arent turned on,

or your cables from the Quadboard to the TNCs are not plugged in or wrongly

made up, or your port assignments in the configuration file of the BBS program

are not matching the plug + TNC arrangement.  Thats why I suggested you start

with one channel first and then one by one add more allowing you to check TNCs

and cables and start with less unknowns.

THE BBS program needs to see an operating TNC on each channel that is

specified in the configuration file. Otherwise it will just hang and wait.

More to come as I experience and write it ... 73 karl N1DL